# 3GPP TSG-RAN WG4 Meeting # 107                               R4-2308418

# Incheon, KR, May 22 – May 26, 2023

# Title: 	Discussion on ATG scenarios

# Source: 	ZTE Corporation

# Agenda item:	8.14.5.1

# Document for:	Approval

# 1 Introduction

In RAN#96e meeting[1], the revised work item on Air-to-ground network for NR was approved as one of Rel-18 RAN4 package.

During the last RAN4 meeting, some conclusions has been reached for ATG scenarios and deployment which are listed below[2].

Issue 1-1: Scenario Agreement: - Focus on ATG deployment scenario which aligned with co-existence session, other scenarios (e.g., the scenario with large Doppler jumping in an ATG cell) can be considered in the future enhancement when needed. Issue 1-2: UE assumption Tentative agreements: - Take ATG UE pre-compensation on UL frequency shift and UL timing shift for ATG demodulation as baseline. - Take ATG UE pre-compensation on DL frequency shift for ATG demodulation as baseline. Issue 1-3-1: Channel model Agreement: - At least single path AWGN channel with doppler Issue 1-3-2: Doppler shift assumption: Agreement from GTW: - Align with RF session agreement on frequency error requirements with below values as starting point - For FDD channel model, set Doppler as [220Hz] - For TDD channel model, set Doppler as [400/500Hz]. FFS: - For FDD channel model, - Option 1: set Doppler as 220Hz - Option 2: Set Doppler as 200Hz for UL, 220Hz for DL - For TDD channel model: - Option 1: set Doppler as 400Hz. - Option 2: set Doppler as 500Hz

This contribution shares some views about Doppler shift assumption.

# 2	Discussion

With regard to scenario for ATG, the Doppler shift assumption for demodulation requirement is still FFS. The agreement reached in RAN4 #106 meeting clearly states that 0.1ppm Doppler shift is assumed as copied below[3].

Issue 2-1-3: Channel model Agreement: - Candidate channel models for ATG deployment: - Option 1: Single path AWGN channel - Option 2: NTN-TDL-C (FFS for the delay spread and K factor) - Doppler shift assumption: 0.1 ppm

Table 2-1 lists the bands for ATG with new band were introduced for ATG[4].

Table 2-1 NR operating bands related to ATG in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n1 | 1920 MHz – 1980 MHz | 2110 MHz – 2170 MHz | FDD |
| n3 | 1710 MHz – 1785 MHz | 1805 MHz – 1880 MHz | FDD |
| n34 | 2010 MHz – 2025 MHz | 2010 MHz – 2025 MHz | TDD |
| n39 | 1880 MHz – 1920 MHz | 1880 MHz – 1920 MHz | TDD |
| n41 | 2496 MHz – 2690 MHz | 2496 MHz – 2690 MHz | TDD |
| n78 | 3300 MHz – 3800 MHz | 3300 MHz – 3800 MHz | TDD |
| n79 | 4400 MHz – 5000 MHz | 4400 MHz – 5000 MHz | TDD |

With regard to the Doppler shift assumption for FDD channel model, option 2 (200Hz for UL, 220Hz for DL) is more consistent with the agreement.

With regard to the Doppler shift assumption for TDD channel model, we can find that different band demands different Doppler shift if 0.1 ppm is assumed. As can be seen the frequency range for TDD band is from 1880MHz to 5000MHz, if one Doppler shift is assumed, option 2 (set Doppler as 400Hz) is not suitable. As is agreed in Meeting#106bis UE demodulation session, 1+1 DMRS is used for demodulation requirement. So 500Hz for TDD channel model is more suitable if only one Doppler shift assumption is considered. Based on demodulation experience, there is no significant performance difference between a fixed Doppler shift of 200Hz and 220Hz.

Observation 1. Option 2 (set Doppler as 400Hz) is not suitable as multiple band with different frequency range need to be considered for TDD channel model.

Proposal 1. To consider the following Doppler shift assumption for FDD channel model and TDD channel model.

- For FDD channel model,

  - Option 1: set Doppler as 220Hz

- For TDD channel model:

  - Option 2: set Doppler as 500Hz

# 3 Conclusion

In this contribution, we give our viewpoint of Doppler shift assumption for ATG scenario.

The conclusions are:

Observation 1. Option 2 (set Doppler as 400Hz) is not suitable as multiple band with different frequency range need to be considered for TDD channel model.

Proposal 1. To consider the following Doppler shift assumption for FDD channel model and TDD channel model.

- For FDD channel model,

  - Option 1: set Doppler as 220Hz

- For TDD channel model:

  - Option 2: set Doppler as 500Hz

# 4	Reference

[1]  RP-221369, Revised WID on Air-to-ground network for NR, CMCC

[2]  R4-2305899, WF for ATG demodulation requirements, CMCC

[3]  R4-2302996, WF for ATG demodulation requirements, CMCC

[4]  TS 38.876-030