3GPP TSG-RAN WG4 Meeting #107	R4-2308420

# Incheon, KR, May 22-26, 2023

Title: 	On Redcap RRM maintenance

Source: 	vivo

Agenda item:	5.2.5.2

Document for:	Discussion

Introduction

The remaining issues for Rel-17 Redcap WI was discussed at RAN4 106 meeting and the WF is [1]. This contribution provides our further considerations on remaining issues for Redcap.

Discussion

- Mobility procedures with NCD-SSBs

Handover to a BWP with unmeasured SSB of a known cell

- No conclusion on whether to add following condition on Tx power of the SSB of the target and measured SSBs:

  - Option 1: Add condition on assumption on the TX power of the SSB of the target SSB and measured SSB as follows (underlined highlighted text is the addition to previous agreement).

    - For the case when the UE performs measurement on a SSB and handover to a BWP with un-measured SSB, the following apply:

      - The requirements apply for the scenario where the measured SSB and the SSB in the target BWP for HO belong to the same target cell, i.e., they share the same physCellID (PCI)

      - UE may assume that the TX power of the SSB of the target cell’s active BWP is same as that of the measured SSBs that belong to the same PCI

    - Note: the above agreement is for RedCap, where the measured SSB and the target SSB for HO of the same target cell are:

      - NCD-SSB and NCD-SSB, respectively

      - CD-SSB and NCD-SSB, respectively

      - NCD-SSB and CD-SSB, respectively

    - One additional sample is needed for known inter-frequency handover.

To our understanding the scenario is two neighbour cells have the same PCI and the handover command triggers a UE handover to a NCD-SSB has not been measured before and there is another NCD-SSB has been measured by that UE however it is not the target for the handover. In this case the UE will use the measured data to prepare handover since that UE cannot differentiate different SSB with same PCI. The issue is this scenario has already been classified to the case “handover to known cell” however the Tx power could still be different for different cells (with the same PCI), hence the measurement results will differ and if a UE uses results of another cell other than the target cell for handover command, handover failure could occurs. From our understanding this scenario maybe not a typical case for handover however to guarantee performance, it is ok to add further constraints on the UE’s assumption on the TX power.

Proposal 1:  Handover to a BWP with unmeasured SSB of a known cell, ok to add the conditions that “UE may assume that the TX power of the SSB of the target cell’s active BWP is same as that of the measured SSBs that belong to the same PCI”.

Revision of the intra/inter-frequency definition in handover

- FFS on whether to add following clarification to handover requirement:

  - Handover for a RedCap UE is defined as intra-frequency handover if the center frequency and subcarrier spacing (SCS) of the reference SSB of the serving cell is same as the center frequency and SCS of the reference SSB of the target cell, where:

    - Option 1a: The reference SSB of the serving cell is the SSB configured in the BWP-specific servingCellMO, if configured, else the SSB configured in the servingCellMO

    - Option 1b: The reference SSB of the serving cell is the SSB configured in the active BWP of the serving cell

    - The reference SSB of the target cell is the SSB configured in the firstActiveBWP of the target cell

For this issue, our understanding is similar definition to that of intra-frequency measurement can be used hence we are ok to option 1a.

Proposal 2: Option 1a could be used as the further clarification on handover requirement.

Conclusions

In this paper we provided our views on the question in the LS [1] and have the following proposal:

Proposal 1:  Handover to a BWP with unmeasured SSB of a known cell, ok to add the conditions that “UE may assume that the TX power of the SSB of the target cell’s active BWP is same as that of the measured SSBs that belong to the same PCI”.

Proposal 2: Option 1a could be used as the further clarification on handover requirement.

Reference

[1] R4-2303264, WF on R17 RedCap RRM maintenance, RAN4