3GPP TSG-RAN WG4 Meeting #107                                                               R4-2308435

Incheon, Korea Republic, May 22nd – 26th, 2023

Source:	Samsung

Title:	Discussion on RRM requirements for intra-band CA scenario for FR2 HST

Agenda item:			8.13.4.2

Document for:	Discussion

# 1. Introduction

In RAN#95e meeting, the Rel-18 RAN4-led WI on enhanced NR support for high speed train scenario in FR2 has been approved as [1]. This Rel-18 can be regarded as the continuous enhancement over the Rel-17 feature of NR support of FR2 HST, in which Rel-17 WI RAN4 has focused on train roof-mounted high-power devices for NR SA single carrier scenario in FR2, by studying the FR2 HST deployment scenario and specifying the channel modelling, RF, RRM and demodulation requirements for FR2 HST.

In the past RAN4#106, the WF on intra-band carrier aggregation (CA) scenario for FR2 HST enhancement was agreed [3], with the following agreement achieved:

<Agreement from RAN4#106, in R4-2303173> Agreements - RAN4 to specify intra-band CA RRM requirements in HST FR2 scenarios only for open space deployment

Furthermore, the approved WF [4] in RAN4#106Bis-e contains further agreements on supporting CA with multi-RX chains, in which RAN4 agreed that “Do not define RRM requirements for CA for PC6 UEs capable of multi-panel (multi-Rx chain) operation with multi-panel operation enabled in Rel-18”, and further study the detailed set of requirements. In this contribution, we would like to provided our further analysis of Intra-band CA scenario for FR2 HST enhancement.

2. Discussion

NR intra-frequency measurement

As discussed in last meeting, for the FR2 intra-band CA requirement required for Rel-18 FR2 HST enhancement WI, the intra-frequency NR measurement is not intended for the SCC in the FR2 intra-band CA, due to the requirement applicability provided in the clause 9.2.3.2. Specifically, with the below wording given in 9.2.3.2, there is no RRM requirement impact or enhancement expected for Rel-18 FR2 HST UE.

#### 9.2.3.2 Requirements for FR2 For one single intra-frequency layer in a band, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least: - 6 identified cells, and - 24 SSBs with different SSB index and/or PCI, where this single intra-frequency layer shall be: - PCC when UE is configured with SA NR operation mode with PCC in the band; or - PSCC when UE is configured with EN-DC with PSCC in the band; or - PSCC when UE is configured with NR-DC with PSCC in the band; or - One of the SCCs on which UE is configured to report SSB based measurements when neither PCC nor PSCC is in the same band, so that the selected SCC shall be an SCC where the UE is configured with SS-RSRP measurement reporting if such SCC exists, otherwise the selected SCC is determined by UE implementation. The UE shall also be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least 2 SSBs on serving cell for each of the other intra-frequency layer(s) in the same band.

On the other hand, in the last meeting, the following agreement is achieved to confirm that the measurement period for intra-frequency measurement can be reused at least for non-tunnel deployment scenario. Considering the agreement in RAN4#106, i.e., “RAN4 to specify intra-band CA RRM requirements in HST FR2 scenarios only for open space deployment”, we see no more discussion is needed even for the measurement period requirement for intra-frequency measurement even for tunnel deployment scenario.

Considering the above observations, we can reach the following proposal to close the discussion on intra-frequency measurement for Rel-18 FR2 HST UE supporting intra-band CA

Proposal 1: The following common understanding needs further confirmation from RAN4:

- No RRM requirement impact or enhancement is needed to enable NR intra-frequency measurement for Rel-18 FR2 HST UE supporting intra-band CA.

NR inter-freq. PSS/SSS, time index detection and measurement

The following agreement is achieved in last meeting on NR inter-frequency measurement:

(1) PSS/SSS detection, Time index detection for intra-frequency and inter-frequency measurements Agreements and way forwards: - The enhanced FR2 HST requirements for intra-frequency detection in Rel-17 FR2 HST WI are also applied for SCells inter-frequency cell detection if introduced. - FFS RAN4 need to specify the requirements for inter-frequency PSS/SSS detection and - FFS RAN4 need to specify the requirements for time index detection.

However, in last meeting, some companies proposed that because only intra-band CA scenario considered in Rel-18 FR2 HST work item, the MRTD for FR2 intra-band non-contiguous NR CA is only 260ns, which is small enough compared with the symbol length, the specified requirement is copied as below.

Table 7.6.4-1: Maximum receive timing difference requirement for intra-band non-contiguous NR carrier aggregation

| Frequency Range | Maximum receive timing difference (µs) |
| --- | --- |
| FR1 | 31 |
| FR2-1 | 0.26 |
| Note 1: In the case of different SCS on different CCs, if the receive time difference exceeds the cyclic prefix length of that SCS, demodulation performance degradation is expected for the first symbol of the slot. |  |

From technical feasibility perspective, the FR2 intra-band CA for HST scenario can have the frame boundary alignment (including half frame, subframe and slot boundary alignment) across cells on the target carrier and reference carrier within a tolerance not worse than min(2 SSB symbols of target carrier, 1 PDSCH symbol of the reference cell) and the SFNs of all cells on the target carrier and reference carrier are the same. Therefore, deriveSSB-IndexFromCellInter-r17 can be enabled for FR2 HST UE.

Observation 1: For FR2 HST intra-band CA scenario, deriveSSB-IndexFromCellInter-r17 shall be assumed to be enabled, which reduce the necessity of time index detection.

Accordingly, the following proposal is provided:

Proposal 2: No RRM requirement impact or enhancement is needed for NR inter-frequency time index detection for Rel-18 FR2 HST UE supporting intra-band CA.

Accordingly, by following the agreement from last meeting, the requirement for intra-frequency PSS/SSS detection and intra-frequency measurement for Rel-17 FR2 HST UE (which is enhanced over Rel-15 requirement) shall be applied for inter-frequency PSS/SSS detection and inter-frequency measurement. Specifically, in Rel-17, the RX beam sweeping factor is assumed to be 2 or 6 (reduced from 8 for FR2 PC3) when highSpeedMeasFlagFR2-r17 is configured as “set1” or “set2” respectively, which can also be followed for Rel-18 inter-frequency requirement.

Proposal 3: For Rel-18 PC6 UE supporting Rel-18 FR2 HST intra-band CA with [highSpeedMeasFlagFR2-r17] configured, RX sweeping factor is assumed as follows to derive inter-frequency PSS/SSS detection and inter-frequency measurement requirement, for with and without gap:

- RX beam sweeping factor is 2 if [highSpeedMeasFlagFR2-r17] = set1;

- RX beam sweeping factor is 6 if [highSpeedMeasFlagFR2-r17] = set2.

Inter-frequency PSS/SSS detection and measurement without gap

The below is the detailed proposal for PSS/SSS detection without gap.

Proposal 4: For Rel-18 PC6 UE supporting Rel-18 FR2 HST intra-band CA with [highSpeedMeasFlagFR2-r17] configured, for inter-frequency measurement without gap

- If SMTC <=40ms, the time period TPSS/SSS_sync_inter used in NR inter-frequency PSS/SSS detection without gap in active BWP is given in the below table

Table 9.3.9.1-x: Time period for PSS/SSS detection when [highSpeedMeasFlagFR2-r17] is configured (Frequency range FR2) when SMTC period <= 40ms

| DRX cycle | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | max(600ms, ceil(M1Note 2 x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFinter |
| DRX cycle≤ 80ms | max(600ms, ceil(M1Note 2 x Kp x Klayer1_measurement) x max(SMTC period, DRX cycle))Note 1 x CSSFinter |
| 80ms< DRX cycle≤ 320ms | max(600ms, ceil(1.5 x Mpss/sss_sync_inter x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFinter |
| DRX cycle>320ms | ceil(Mpss/sss_sync_inter x Kp x Klayer1_measurement) x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: Void NOTE 3: For UE supporting power class 6, M1 = 6 if highSpeedMeasFlagFR2-r17 = set1 or M1 = 18 if highSpeedMeasFlagFR2-r17 = set2 |  |

- Otherwise, the existing requirement for PC3 applies.

- Note: Mpss/sss_sync_inter for FR2 PC6 UE shall follow the definition for PC3 UE.

Similar as the above discussion for NR inter-frequency PSS/SSS detection without gap, the similar change can be applied for NR inter-frequency measurement without gap.

Proposal 5: Similar as NR inter-frequency PSS/SSS detection without gap, the similar change can be applied for NR inter-frequency measurement without gap.

Inter-frequency PSS/SSS detection and measurement with gaps

On the other hand, for inter-frequency PSS/SSS detection and measurement with measurement gaps, for a UE supporting FR2-1 power class 3, Mpss/sss_sync_inter = Mmeas_period_inter = 40 samples, which is relaxed from 24 samples for intra-frequency case. Therefore, the detailed proposal is given as below:

Proposal 6: For Rel-18 PC6 UE supporting Rel-18 FR2 HST intra-band CA with [highSpeedMeasFlagFR2-r17] configured, for inter-frequency measurement with gaps

- If SMTC <=40ms, the time period TPSS/SSS_sync_inter used in NR inter-frequency PSS/SSS detection with gaps is given in the below table

Table 9.3.4.1-x: Time period for PSS/SSS detection when [highSpeedMeasFlagFR2-r17] is configured (Frequency range FR2) when SMTC period <= 40ms

| Condition NOTE1 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600ms, Ceil(Kgap M2Note 3) Max(MGRP , SMTC period)) CSSFinter |
| DRX cycle≤ 80ms | Max(600ms, Ceil(Kgap M2Note 3) Max(MGRP , SMTC period, DRX cycle)) CSSFinter |
| 80ms< DRX cycle≤ 320ms | Max(600ms, Ceil(1.5 * Kgap Mpss/sss_sync_inter) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | Ceil(Kgap Mpss/sss_sync_inter) DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: For a UE supporting concurrent gaps, the MRGP above is the MRGP of the measurement gap associated with the target frequency layer to be measured if concurrent measurement gaps are configured. NOTE 3: For UE supporting power class 6, M2 = 10 if highSpeedMeasFlagFR2-r17 = set1 or M2 = 30 if highSpeedMeasFlagFR2-r17 = set2 |  |

- Otherwise, the existing requirement for PC3 applies.

- Note: Mpss/sss_sync_inter for FR2 PC6 UE shall follow the definition for PC3 UE.

Proposal 7: Similar as NR inter-frequency PSS/SSS detection with gap, the similar change can be applied for NR inter-frequency measurement with gap.

SSB-based BFD and RLM on SCell

As discussed in last meeting, the following agreement is achieved for BFR on SCell [4]:

(2) Beam failure recovery on SCell Agreements: - The enhanced FR2 HST requirements for BFD in Rel-17 FR2 HST WI are also applied for SCells.

During Rel-17 FR2 HST work item discussion, the scaling factor for SSB-based BFD and SSB-based RLM has been enhanced for FR2 power class 6 UE configured with highSpeedMeasFlagFR2-r17.

However, for SSB-based BFD requirement, normal intra-band CA case, the relevant requirement is not applicable if UE is not required to perform BFD on more than 1 serving cell per band:

### 8.5.2 Requirements for SSB based beam failure detection #### 8.5.2.1 Introduction The requirements in this clause apply for each SSB resource in the set configured for a serving cell, provided that the SSB configured for beam failure detection is actually transmitted within the UE active DL BWP during the entire evaluation period specified in clause 8.5.2.2. The requirements in this clause could not be applicable if UE is required to perform beam failure detection on more than 1 serving cell per band.

Therefore, we observed no RRM impact is required for FR2 power class 6 UE supporting intra-band CA.

Proposal 8: No RRM requirement impact or enhancement is needed for BFD for FR2 power class 6 UE supporting intra-band CA.

For the Rel-17 enhancement for FR2 HST UE RLM performance, the relevant changes introduced in Rel-17 can be used for any serving cell, therefore we observed no RRM impact is required for FR2 power class 6 UE supporting intra-band CA.

Proposal 9: No RRM requirement impact or enhancement is needed for RLM on SCell for FR2 power class 6 UE supporting intra-band CA.

L1-RSRP and L1-SINR measurement on SCell

As discussed in last meeting, the following agreement is achieved for L1-RSRP measurement on SCell [4]:

(3) L1-RSRP measurement on SCell Agreements: - The enhanced FR2 HST requirements for L1-RSRP measurement in Rel-17 FR2 HST WI are also applied for SCells.

In the Rel-17 FR2 HST work item, the enhancement on SSB-based L1-RSRP measurement as below is introduced for FR2 power class 6 UE when highSpeedMeasFlagFR2-r17 is configured,

Table 9.5.4.1-3: Measurement period TL1-RSRP_Measurement_Period_SSB configured with highSpeedMeasFlagFR2-r17 for FR2 Configuration; TL1-RSRP_Measurement_Period_SSB (ms). non-DRX; max(TReport, ceil(M*P*N1Note2)*TSSB). DRX cycle ≤ 80ms; max(TReport, ceil(M*P*N1Note2*M2)*max(TDRX,TSSB)). 80ms< DRX ≤ 320ms; max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)). DRX cycle > 320ms; ceil(1.5*M*P*N)*TDRX. Note1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: N1 = 2 when highSpeedMeasFlagFR2-r17 = set1; N1 = 6 when highSpeedMeasFlagFR2-r17 = [set2]. Note 3: M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1.

Based on the agreement achieved in the last meeting, the Rel-17 introduced requirement for FR2 PC6 UE can be applied to L1-RSRP measurement on SCell directly, and no additional RRM impact observed.

Observation 2: Rel-17 introduced requirement for FR2 PC6 UE can be applied to L1-RSRP measurement on SCell directly, and no additional RRM impact observed.

Proposal 10: No RRM requirement impact or enhancement is needed for L1-RSRP measurement on SCell.

Similarly, the same conclusion can be made for L1-SINR requirement, for which RAN1 also introduced the requirements for FR2 PC6 UE in Rel-17.

Proposal 11: No RRM requirement impact or enhancement is needed for L1-SINR measurement on SCell.

TCI state switching on SCell

As discussed in last meeting, the following agreement is achieved for TCI state switching on SCell [4]:

(4) TCI state switch in SCell Agreements: - The enhanced FR2 HST requirements for SSB based L1-RSRP reporting in Rel-17 FR2 HST WI, used for TCI state switch in PCells, are also considered for SCells.

For TCI state switching delay clause, the requirement is applicable for a UE configured with one or more TCI state configurations on serving cell in MR-DC or standalone NR, and no specific clause for PCell or SCell. For the requirement to TCI switching to an unknown TCI state, the relevant L1-RSRP measurement requirement is referred to and therefore no explicit requirement impact is expected for the clause of active TCI state switching delay.

Proposal 12: No RRM requirement impact or enhancement is needed for TCI state switching on SCell.

SCell activation delay requirement

As discussed in last meeting, the following WF is achieved for SCell activation delay requirement [4]:

(6) SCell activation delay Way forward: - Check SCell activation delay and applicable conditions in HST scenario.

Considering the existing SCell Activation Delay Requirement for Deactivated SCell specified in clause 8.3.2, the applicable condition for FR2 intra-band CA is provided as below:

Upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length , where: THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3] Tactivation_time is the SCell activation delay in millisecond. <unrelated contents omit> If the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band, then Tactivation_time is TFirstSSB+ 5ms provided: - The UE is provided with SMTC for the target SCell, and - The SSBs in the serving cell(s) and the SSBs in the SCell fulfil the condition defined in clause 3.6.3, - The parameter ssb-PositionsInBurst is same for the serving cell(s) and the SCell. - SSB is in the same half-frame on the SCell and the contiguous FR2 active serving cell If the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band, if the UE supporting scellWithoutSSB is not provided with any SMTC for the target SCell, Tactivation_time is 3 ms, provided - the RS (s) of SCell being activated is (are) QCL-TypeD with RS (s) of one active serving cell on that FR2 band. <unrelated contents omit>

Obviously, the above applicable condition for FR2 intra-band CA is clear and not relevant to any factors related to FR2 HST scenario, therefore we observed that there is no RRM requirement impact or enhancement needed for SCell activation delay.

Proposal 13: No RRM requirement impact or enhancement is needed for SCell activation delay requirement.

Inter-frequency Measurement in IDLE mode

As discussed in last meeting, the following WF is achieved for inter-frequency measurement requirements in IDLE mode [4]:

Agreements: - Inter-frequency measurement requirements in Idle mode: - RAN4 introduce requirement in Rel-18 FR2 HST WI. - Rel-17 FR2 HST principles is reused to define inter-frequency measurement requirements.

A recap from TS 38.133, the specified measurements of intra-frequency NR cells for RRC_IDLE state mobility is copied as follow:

4.2.2.4 Measurements of inter-frequency NR cells

The existing enhanced measurement is applicable to PCell, as single CC is supported Rel-17 FR2 HST. Since PCell need to perform NR measurement to add the PSCell/SCell before CA, it is also necessary to investigate the RRM impacts on measurements of inter-frequency NR cells in IDLE state incurred by introducing CA operation. Accordingly, by following the agreement from last meeting, the requirement for Rel-17 FR2 HST shall be considered as the principle for inter-frequency cell detection, measurement and evaluation requirements. Specifically, in Rel-17, the RX beam sweeping factor is assumed to be 2 or 6 when highSpeedMeasFlagFR2-r17 is configured as “set1” or “set2” respectively, which can also be followed for Rel-18 inter-frequency requirement.

Proposal 14: For Rel-18 PC6 UE supporting Rel-18 FR2 HST intra-band CA with [highSpeedMeasFlagFR2-r17] configured, RX sweeping factor is assumed as follows to derive	inter-frequency measurement in IDLE mode:

- RX beam sweeping factor is 2 if [highSpeedMeasFlagFR2-r17] = set1;

- RX beam sweeping factor is 6 if [highSpeedMeasFlagFR2-r17] = set2.

The below is the detailed proposal for inter-frequency measurement in IDLE mode.

Proposal 15: For Rel-18 PC6 UE supporting Rel-18 FR2 HST intra-band CA with [highSpeedMeasFlagFR2-r17] configured, for inter-frequency measurement in IDLE mode

- The measurement requirement of inter-frequency NR cells is given in the below table

Table 4.2.2.3-x: Tdetect,NR_Inter_HST, Tmeasure, NR_Inter_HST and Tevaluate, NR_Inter_HST for FR2 configured with highSpeedMeasFlagFR2-r17 note2

| DRX cycle length [s] | Scaling Factor (N1) | Tdetect,NR_Inter_HST [s] (number of DRX cycles) | Tmeasure, NR_Inter_HST [s] (number of DRX cycles) | Tevaluate, NR_Inter_HST [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- |
| 0.32 | N1Note3 | 2.56 x N1 x M2 (8 x N1 x M2) Note1 | 0.32 x N1 x M3 (1 x N1 x M3) Note1 | 0.96 x N1 x M4 (3 x N1 x M4) Note1 |
| 0.64 | 5 | 17.92 x N1 (28 x N1) | 1.28 x N1 (2 x N1) | 5.12 x N1 (8 x N1) |
| 1.28 | 4 | 32 x N1 (25 x N1) | 1.28 x N1 (1 x N1) | 6.4 x N1 (5 x N1) |
| 2.56 | 3 | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
| Note 1: When SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 1.5, M3 = M4 = 2 Note 2: The support of HST Idle mode inter-frequency measurement enhancement is optional without capability signalling. Apply for UE declarating supports idle mode inter-frequency measurement enhancement for HST, otherwise Table 4.2.2.4-1 shall be used. Note 3: N1 = 2 when highSpeedMeasFlagFR2-r17= set1; N2 = 6 when highSpeedMeasFlagFR2-r17= set2. |  |  |  |  |

Network signaling for Rel-18 FR2 HST CA Scenario

From last meeting, it is agreed that

Issue 2-3-1: HST signaling for CA enhancement Agreements: - RAN4 to postpone the agreement on network signalling until the CA requirements are defined.

Although technically we proposed that PCell signalling, i.e., highSpeedMeasFlagFR2-r17 (with “set1” and “set2” as two candidate values), can be reused for SCell measurement and other operations for FR2 HST scenario. In other words, no need to introduce another IE in Rel-18. However, due to the above agreement, we can just focus on detailed CA requirement firstly in this meeting, and by just assume one flag signaling (no matter new or reused on) is indicated by NW to FR2 PC6 UE, which shall apply the corresponding requirement (to be introduced in Rel-18) on SCell.

Observation 3: Before deciding a new or the reused flag signaling for FR2 HST CA, RAN4 can assume there is one flag signaling indicated by NW, and accordingly the FR2 PC6 UE shall apply the corresponding new SCell operation requirements (to be introduced in Rel-18).

3. Conclusion

In this contribution, we provided our viewpoints on the RRM aspects for this work item, accordingly the following observations and proposals are obtained:

Observation 1: For FR2 HST intra-band CA scenario, deriveSSB-IndexFromCellInter-r17 shall be assumed to be enabled, which reduce the necessity of time index detection.

Observation 2: Rel-17 introduced requirement for FR2 PC6 UE can be applied to L1-RSRP measurement on SCell directly, and no additional RRM impact observed.

Observation 3: Before deciding a new or the reused flag signaling for FR2 HST CA, RAN4 can assume there is one flag signaling indicated by NW, and accordingly the FR2 PC6 UE shall apply the corresponding new SCell operation requirements (to be introduced in Rel-18).

Proposal 1: The following common understanding needs further confirmation from RAN4:

- No RRM requirement impact or enhancement is needed to enable NR intra-frequency measurement for Rel-18 FR2 HST UE supporting intra-band CA.

Proposal 2: No RRM requirement impact or enhancement is needed for NR inter-frequency time index detection for Rel-18 FR2 HST UE supporting intra-band CA.

Proposal 3: For Rel-18 PC6 UE supporting Rel-18 FR2 HST intra-band CA with [highSpeedMeasFlagFR2-r17] configured, RX sweeping factor is assumed as follows to derive inter-frequency PSS/SSS detection and inter-frequency measurement requirement, for with and without gap:

- RX beam sweeping factor is 2 if [highSpeedMeasFlagFR2-r17] = set1;

- RX beam sweeping factor is 6 if [highSpeedMeasFlagFR2-r17] = set2.

Proposal 4: For Rel-18 PC6 UE supporting Rel-18 FR2 HST intra-band CA with [highSpeedMeasFlagFR2-r17] configured, for inter-frequency measurement without gap

- If SMTC <=40ms, the time period TPSS/SSS_sync_inter used in NR inter-frequency PSS/SSS detection without gap in active BWP is given in the below table

Table 9.3.9.1-x: Time period for PSS/SSS detection when [highSpeedMeasFlagFR2-r17] is configured (Frequency range FR2) when SMTC period <= 40ms

| DRX cycle | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | max(600ms, ceil(M1Note 2 x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFinter |
| DRX cycle≤ 80ms | max(600ms, ceil(M1Note 2 x Kp x Klayer1_measurement) x max(SMTC period, DRX cycle))Note 1 x CSSFinter |
| 80ms< DRX cycle≤ 320ms | max(600ms, ceil(1.5 x Mpss/sss_sync_inter x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFinter |
| DRX cycle>320ms | ceil(Mpss/sss_sync_inter x Kp x Klayer1_measurement) x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: Void NOTE 3: For UE supporting power class 6, M1 = 6 if highSpeedMeasFlagFR2-r17 = set1 or M1 = 18 if highSpeedMeasFlagFR2-r17 = set2 |  |

- Otherwise, the existing requirement for PC3 applies.

- Note: Mpss/sss_sync_inter for FR2 PC6 UE shall follow the definition for PC3 UE.

Proposal 5: Similar as NR inter-frequency PSS/SSS detection without gap, the similar change can be applied for NR inter-frequency measurement without gap.

Proposal 6: For Rel-18 PC6 UE supporting Rel-18 FR2 HST intra-band CA with [highSpeedMeasFlagFR2-r17] configured, for inter-frequency measurement with gaps

- If SMTC <=40ms, the time period TPSS/SSS_sync_inter used in NR inter-frequency PSS/SSS detection with gaps is given in the below table

Table 9.3.4.1-x: Time period for PSS/SSS detection when [highSpeedMeasFlagFR2-r17] is configured (Frequency range FR2) when SMTC period <= 40ms

| Condition NOTE1 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600ms, Ceil(Kgap M2Note 3) Max(MGRP , SMTC period)) CSSFinter |
| DRX cycle≤ 80ms | Max(600ms, Ceil(Kgap M2Note 3) Max(MGRP , SMTC period, DRX cycle)) CSSFinter |
| 80ms< DRX cycle≤ 320ms | Max(600ms, Ceil(1.5 * Kgap Mpss/sss_sync_inter) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | Ceil(Kgap Mpss/sss_sync_inter) DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: For a UE supporting concurrent gaps, the MRGP above is the MRGP of the measurement gap associated with the target frequency layer to be measured if concurrent measurement gaps are configured. NOTE 3: For UE supporting power class 6, M2 = 10 if highSpeedMeasFlagFR2-r17 = set1 or M2 = 30 if highSpeedMeasFlagFR2-r17 = set2 |  |

- Otherwise, the existing requirement for PC3 applies.

- Note: Mpss/sss_sync_inter for FR2 PC6 UE shall follow the definition for PC3 UE.

Proposal 7: Similar as NR inter-frequency PSS/SSS detection with gap, the similar change can be applied for NR inter-frequency measurement with gap.

Proposal 8: No RRM requirement impact or enhancement is needed for BFD for FR2 power class 6 UE supporting intra-band CA.

Proposal 9: No RRM requirement impact or enhancement is needed for RLM on SCell for FR2 power class 6 UE supporting intra-band CA.

Proposal 10: No RRM requirement impact or enhancement is needed for L1-RSRP measurement on SCell.

Proposal 11: No RRM requirement impact or enhancement is needed for L1-SINR measurement on SCell.

Proposal 12: No RRM requirement impact or enhancement is needed for TCI state switching on SCell.

Proposal 13: No RRM requirement impact or enhancement is needed for SCell activation delay requirement.

Proposal 14: For Rel-18 PC6 UE supporting Rel-18 FR2 HST intra-band CA with [highSpeedMeasFlagFR2-r17] configured, RX sweeping factor is assumed as follows to derive	inter-frequency measurement in IDLE mode:

- RX beam sweeping factor is 2 if [highSpeedMeasFlagFR2-r17] = set1;

- RX beam sweeping factor is 6 if [highSpeedMeasFlagFR2-r17] = set2.

Proposal 15: For Rel-18 PC6 UE supporting Rel-18 FR2 HST intra-band CA with [highSpeedMeasFlagFR2-r17] configured, for inter-frequency measurement in IDLE mode

- The measurement requirement of inter-frequency NR cells is given in the below table

Table 4.2.2.3-x: Tdetect,NR_Inter_HST, Tmeasure, NR_Inter_HST and Tevaluate, NR_Inter_HST for FR2 configured with highSpeedMeasFlagFR2-r17 note2

| DRX cycle length [s] | Scaling Factor (N1) | Tdetect,NR_Inter_HST [s] (number of DRX cycles) | Tmeasure, NR_Inter_HST [s] (number of DRX cycles) | Tevaluate, NR_Inter_HST [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- |
| 0.32 | N1Note3 | 2.56 x N1 x M2 (8 x N1 x M2) Note1 | 0.32 x N1 x M3 (1 x N1 x M3) Note1 | 0.96 x N1 x M4 (3 x N1 x M4) Note1 |
| 0.64 | 5 | 17.92 x N1 (28 x N1) | 1.28 x N1 (2 x N1) | 5.12 x N1 (8 x N1) |
| 1.28 | 4 | 32 x N1 (25 x N1) | 1.28 x N1 (1 x N1) | 6.4 x N1 (5 x N1) |
| 2.56 | 3 | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
| Note 1: When SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 1.5, M3 = M4 = 2 Note 2: The support of HST Idle mode inter-frequency measurement enhancement is optional without capability signalling. Apply for UE declarating supports idle mode inter-frequency measurement enhancement for HST, otherwise Table 4.2.2.4-1 shall be used. Note 3: N1 = 2 when highSpeedMeasFlagFR2-r17= set1; N2 = 6 when highSpeedMeasFlagFR2-r17= set2. |  |  |  |  |

4. Reference

- RP-220985, “New WID on enhanced NR support for high speed train scenario in frequency range 2 (FR2)”, Samsung.

- RP-222272, “New WID on enhanced NR support for high speed train scenario in frequency range 2 (FR2)”, Samsung.

- R4-2303173, “WF on NR FR2 HST Enhancements RRM requirements”, Samsung.

- R4-2306341, “WF on intra-band CA for FR2 HST PC6 UE”, Ericsson.