3GPP TSG-RAN WG4 Meeting #107	R4-2308439

Incheon, Korea, May 22- May 26, 2023

Agenda Item:	8.26.2.3

Source: 	Ericsson

Title:	Discussions on NW-A’s requirement in MUSIM gaps

Document for:	Discussion

# Introduction

In last meeting, RAN4 had an initial discussion on NW-A’s requirements for MUSIM gap as follow [1].

Issue 3-1-1: Principle on layer 1 and layer 3 measurement requirements after gap collision handling - Proposals - P1: Frameworks of LBT failure in NR-U design can be used as starting point when discussing NW A L3/L1 requirement impact due to MUSIM gaps (Apple) - P2: Reuse the same principle of Rel-17 concurrent gaps WI to define network A L1/L3 measurement requirements when MUSIM gaps are configured, i.e., introduce a scaling factor like Kx = Ntotal /Navailable for network A requirements when MUSIM gaps are configured. (CMCC xiaomi vivo ZTE oppo Huawei Ericsson Qualcomm MTK Nokia) - P3: RAN4 to postpone the detail NW-A’s requirement discussion until RAN4 achieves the consensus on MUSIM gaps’ priority. (Ericsson Nokia) Issue 3-1-3: On the time window W for aperiodic gap - Proposals - P1: Not take aperiodic gap into account when determining the time window W, and clarify that the related measurement period will be longer. (vivo xiaomi oppo Huawei Nokia Qualcomm) - P2: max(SMTC period, MGRP_max)+[M], where MGRP_max is the largest periodicity among all the periodic gaps and [M] is a time margin for the one-shot aperiodic MUSIM gap. (MTK) Agreements: P1 - Not take aperiodic gap into account when determining the time window W, and clarify that the related measurement period will be longer

In this contribution, we will continue to discuss the general issues on NW-A’s requirements for MUSIM gaps.

# General

The main impacts to NW-A’s requirement due to MUSIM gaps are as follow:

- Intra-frequency measurement

- Inter-frequency measurement

- L1 measurement

In Rel-17, RAN4 went through the measurement requirements when Con-MGs are introduced. A unified method to count the available occasions in time window W is defined. We suggest to follow the same framework to update the requirement with further consideration of MUSIM gaps. However, there are still some uncertain aspects since no clear UE behaviour has defined when MUSIM gap collides with NW-A’s MG and collision within MUSIM gaps. RAN4 should recheck the detail of requirement after the collision issues solved.

Proposal 1: RAN4 to reuse the principle in Rel-17 concurrent gaps WI as the baseline to define network A L1/L3 measurement requirements when MUSIM gaps are configured.

Proposal 2: RAN4 to postpone the detail NW-A’s requirement discussion until RAN4 achieves the consensus on MUSIM gaps’ priority.

Furthermore, considering the MUSIM gaps may have a long MGRP, such as 640ms, we think the collision issue mentioned by other companies is also valid. For example, as the figure below, based on current Rel-17 Conllision rule, the measurement window W of duration max(SMTC period,  MGRP_max) = 640ms. The effective measurement occasions within W is larger than the minimum requirement measurement samples (5). If based on current logic, the measurement delay requirement will have unnecessary relax.

Figure 1. The collision between SMTC and MUSIM gap with longer MGRP

Observation 1: The L1/L3 measurement requirement will have unnecessary relaxed due to long MGRP of MUSIM gaps.

Proposal 3: RAN4 to further consider the L1/L3 measurement requirement when the configured MUSIM gaps have longer MGRP.

- Summary

In this contribution, we have discussed the MUSIM gaps requirements. Based on the discussions, we have made following proposals and observations:

Proposal 1: RAN4 to reuse the principle in Rel-17 concurrent gaps WI as the baseline to define network A L1/L3 measurement requirements when MUSIM gaps are configured.

Proposal 2: RAN4 to postpone the detail NW-A’s requirement discussion until RAN4 achieves the consensus on MUSIM gaps’ priority.

Proposal 3: RAN4 to further consider the L1/L3 measurement requirement when the configured MUSIM gaps have longer MGRP.

- References

- R4-2306357, “WF on RRM requirements for Rel-17 MUSIM gaps”, Vivo