3GPP TSG-RAN WG4 Meeting # 107	R4-2308463

# Incheon, KR, May 22 – May 26, 2023

Agenda Item:	8.10.3.1

Source: 	OPPO

Title: 	On measurement without gaps for UEs reporting NeedForGapsInfoNR

Document for:	Approval

1	Introduction

Last RAN4 meeting discussed measurements without gap for UEs reporting NeedForGapsInfoNR and the related conclusions were captured in [1]. We will provide our considerations on the remaining issues.

2	Discussion

- Issue 1-1-1: Framework of the interruption requirements < Agreement/Way forward >: - Define interruption length and ratio - FFS on possible restrictions for interruptions - Option 1: The UE is only allowed to cause interruptions on PCell or activated Scell(s) immediately before and after an SMTC. The UE is not expected to cause interruption on each SMTC occasion. - Option 2: The UE is only allowed to cause interruptions on Pcell or activated Scell(s) in the certain time window before and after an SMTC. - Other options are not precluded.

It was agreed to define interruption length and ratio for case 2. However, whether any possible restriction for interruptions will be defined needs further discussion. We believe such the restrictions are beneficial for network scheduling, for example network will avoid important data transmission during the possible interruption slots, but still can try something less important. Between the two candidate options, we prefer option 1. The interruptions are quite similar to the VIL for NCSG and should only be allowed around the SMTC occasion.

Proposal 1: Introduce restrictions for interruptions, and interruptions are only allowed on PCell or activated Scell(s) immediately before and after an SMTC.

- Issue 1-1-2: Requirements on the interruption length, if allowed < Way forward >: - FFS on: - Option 1: As a starting point, the interruption length can be same as VIL defined for NCSG,e.g. - When UE reporting “[no-gap,TBD]” in [NeedForGapInfoNR, TBD] the interruption length can be VIL=1ms in FR1 and VIL=0.75ms in FR2. - When UE reporting “[others,TBD]” in [NeedForGapInfoNR, TBD] no interruption allowed - Option 2: As a starting point, when UE reporting “no-gap [TBD]” in [NeedForGapInfoNR, TBD], the interruption length can be specified based on the same RTT assumption as for NCSG (0.5ms in FR1 and 0.25ms in FR2) interruption occasion.

As for the interruption length, the VIL for NCSG should be used as a starting point, i.e. 1ms for FR1 and 0.75ms for FR2. Compared with option 2, additional time margin on top of RTT assumption is considered, which is also necessary in this case. The UE implementation in case of no-gap with interruption is the same as NCSG, RTT is required to turn on/off RF chain or retuning the current RF chain and additional time margin is needed to prepare simultaneous measurement and data transmission/reception.

Proposal 2: Support option 1, the interruption length can be same as VIL defined for NCSG, e.g.

- When UE reporting “[no-gap, TBD]” in [NeedForGapInfoNR, TBD], the interruption length can be VIL=1ms in FR1 and VIL=0.75ms in FR2.

- When UE reporting “[other, TBD]” in [NeedForGapInfoNR, TBD], no interruption allowed

- Issue 1-1-5: Requirements on the interruption ratio, if allowed < Way forward/Agreement >: - Interruption ratio is defined as follows: - 80ms ≤ Tcycle < 160ms: up to [2.50%] probability of interruption - 160ms ≤ Tcycle < 320ms: up to [1.25%] probability of interruption - 320ms ≤ Tcycle: up to [0.625%] probability of interruption - FFS if the interruption rate can be captured in equation format - Do not define requirement for the case Tcycle < 80ms - FFS if interruption ratio applies to a single frequency layer or all frequency layers - Tcycle definition is FFS - Option 1: Tcycle = SMTC x CSSF x Kp - Other options are not precluded

As shown above, the framework to define interruption ratio with respective to different Tcycle is introduced with some remaining issues. The first issue is whether to capture interruption ratio in equation format. We are open with both options and this can be discussed directly during the CR drafting phase. Then we would like to discuss the definition of Tcycle. Without specific periodicity for NCSG or MG, it is straightforward to derive measurement cycle based on SMTC period. Besides, the measurement cycle should be scaled by factors Kp and Klayer1_measurement to consider the overlapping between SMTC occasions and gaps or L1 measurement resources.

Proposal 3: When defining Tcycle, consider SMTC period and Kp, and in addition consider Klayer1_measurement for FR2.

As for CSSF, we think it should be precluded and the agreed interruption ratio should only apply to single frequency layer. For example, if one frequency layer with 160ms SMTC period is configured, the interruption ratio should be 1.25% assuming up to 2ms interruption around each SMTC occasion. When two frequency layers with 160ms SMTC period are configured as illustrated in the figure above, the measurement cycle is scaled to 320ms if CSSF=2 is considered. However, UE is required perform measurements twice during 320ms, either for the same layer or different layers and the total interruption is 4ms. Then, the interruption ratio within 320ms measurement cycle is 1.25% and 0.625% cannot be satisfied.

Proposal 4: CSSF should not be considered in Tcycle and the agreed interruption ratio should only apply to single frequency layer.

Based on the aforementioned example, the same interruption ratio for single frequency layer should be maintained in case of multiple frequency layers. One special case should be noticed is when the measurement cycles for multiple frequency layers are different, for example one is 80ms and the other is 160ms. In our view, the worst interruption ratio with the shortest measurement cycle should apply. But we are also open to other solutions.

Proposal 5: In case of multiple frequency layers with different measurement cycle, the interruption ratio with the shortest measurement cycle should apply.

During the last meeting, a generic formula is proposed to define measurement period, max([low_bound], ceil( [meas_samples] x [scaling_factor 1]x [scaling_factor 2] x [scaling_factor 3])) x [meas_cycle] ) x [CSSF], where:

- [scaling_factor 1] is M2 depending on DRX

- [scaling_factor 2] is Kp for SMTC overlapping with measurement gap

- [scaling_factor 3] is Klayer1_measurement for L1 measurements RS non-overlapping with measurement gap and should only be considered for FR2

- [meas_samples] is the number of measurement samples without AGC, and its value should be discussed case by case

- [meas_cycle]:

  - Option 1: replace “measCycleSCell” wih Tcycle (depending on issue 1-1-5)

  - Option 2: Depending on Tcycle definition in issue 1-1-1

If measurement cycle Tcycle has already taken the scaling factors such as Kp and Klayer1_measurement into account, then using Tcycle as [meas_cycle] will unnecessarily extend the total delay. We prefer to use SMTC period in case of non-DRX, and use max(SMTC period, DRX cycle) in case of DRX.

Proposal 6: When defining delay for measurement without gap with interruption, the [meas_cycle] should be SMTC period in case of non-DRX and max(SMTC period, DRX cycle) in case of DRX.

For intra-frequency measurements, 5 samples are assumed for PSS/SSS detection no matter measured without gap, with gap or with NCSG as shown in Annex. However, for inter-frequency measurement, 5 samples are assumed for PSS/SSS detection without gap, and 8 samples are assumed for PSS/SSS detection with gap or NCSG. In our view, the measurement without gap but allow interruption should be aligned with NCSG, that is 8 samples are assumed for inter-frequency PSS/SSS detection. The extra 3 samples are for AGC and there is no need to introduce additional samples for AGC.

Proposal 7a: When defining delay for measurement without gap with interruption, the sample number within NCSG in Rel-17 (i.e. 8-sample including AGC) should be used.

One the other hand, for inter-frequency case 1, i.e. without gap and without interruption, the target SSB is generally contained within the active BWP and 5 samples without AGC are sufficient.

Proposal 7b: When defining delay for inter-frequency measurement without gap without interruption, the sample number for measurement without gap in Rel-16 (i.e. 5-sample) should be used.

3	Conclusion

This contribution gave our general views on RRM requirements without gaps and the following proposals:

Proposal 1: Introduce restrictions for interruptions, and interruptions are only allowed on PCell or activated Scell(s) immediately before and after an SMTC.

Proposal 2: Support option 1, the interruption length can be same as VIL defined for NCSG, e.g.

- When UE reporting “[no-gap, TBD]” in [NeedForGapInfoNR, TBD], the interruption length can be VIL=1ms in FR1 and VIL=0.75ms in FR2.

- When UE reporting “[other, TBD]” in [NeedForGapInfoNR, TBD], no interruption allowed

Proposal 3: When defining Tcycle, consider SMTC period and Kp, and in addition consider Klayer1_measurement for FR2.

Proposal 4: CSSF should not be considered in Tcycle and the agreed interruption ratio should only apply to single frequency layer.

Proposal 5: In case of multiple frequency layers with different measurement cycle, the interruption ratio with the shortest measurement cycle should apply.

Proposal 6: When defining delay for measurement without gap with interruption, the [meas_cycle] should be SMTC period in case of non-DRX and max(SMTC period, DRX cycle) in case of DRX.

Proposal 7a: When defining delay for measurement without gap with interruption, the sample number within NCSG in Rel-17 (i.e. 8-sample including AGC) should be used.

Proposal 7b: When defining delay for inter-frequency measurement without gap without interruption, the sample number for measurement without gap in Rel-16 (i.e. 5-sample) should be used.

4	Reference

- R4-2306331, WF on NR_MG_enh2 Part 2, Intel Corporation.

5	Annex

For intra-frequency measurement:

Table 9.2.5.1-1: Time period for PSS/SSS detection without gap, (Frequency range FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max( 600ms, ceil( 5 x Kp) x SMTC period )Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max( 600ms, ceil(M2 Note 2x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(5 x Kp) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1. NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured and UE supports measurementEnhancementCA-r17, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1. |  |

Table 9.2.6.2-1: Time period for PSS/SSS detection with gap (FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600ms, 5 x Kgap x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(600ms, ceil(M2Note 1x 5 x Kgap) x max(MGRP, SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | Ceil( 5 x Kgap ) x max(MGRP, DRX cycle) x CSSFintra |
| NOTE 1: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1. NOTE 2: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 3: For a UE supporting concurrent measurement gaps, if multiple concurrent gaps are configured, the MGRP is the periodicity of the MG pattern associated to the intra-frequency layer. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured, the requirements apply to UE on measurements of secondary component carrier with active SCell. |  |

Table 9.2.7.1-1: Time period for PSS/SSS detection with NCSG (FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600ms, 5 x max(VIRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(600ms, ceil(M2Note 1x 5) x max(VIRP, SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | 5 x max(VIRP, DRX cycle) x CSSFintra |
| NOTE 1: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1. NOTE 2: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or [intraRAT-MeasurementEnhancement-r16] on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. |  |

For inter-frequency measurement:

Table 9.3.4-1: Time period for PSS/SSS detection with gap (Frequency range FR1)

| Condition NOTE1,2 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600ms, Ceil(8 * Kgap) Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(600ms, Ceil(8*1.5 * Kgap) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | Ceil(8 * Kgap) DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: For a UE supporting concurrent gaps, the MRGP above is the MRGP of the measurement gap associated with the target frequency layer to be measured if concurrent measurement gaps are configured. |  |

Table 9.3.9.1-1: Time period for PSS/SSS detection without gap, (FR1)

| DRX cycle | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | max( 600ms, ceil( 5 x Kp) x SMTC period )Note 1 x CSSFinter |
| DRX cycle≤ 320ms | max( 600ms, ceil(M2x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFinter |
| DRX cycle>320ms | ceil(5 x Kp) x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: Void NOTE 3: When highSpeedMeasInterFreq-r17 is not configured, M2 = 1.5; When highSpeedMeasInterFreq-r17 is configured, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1 |  |

Table 9.3.10.1-1: Time period for PSS/SSS detection with NCSG (FR1)

| Condition NOTE1,2 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600ms, 8 Max(VIRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(600ms, Ceil(8*1.5) Max(VIRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | 8 DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. |  |