3GPP TSG-RAN WG4 Meeting # 107		R4-2308478

# Incheon, KR, May 22 – May 26, 2023

Agenda Item:	8.8.3.4

Source: 	OPPO

Title: 	On measurement restrictions for FR2 multi-RX DL reception

Document for:	Approval

1	Introduction

A WF on L1 enhancement for FR2 multi-RX DL reception was approved in RAN4#106bis meeting [1].

In this contribution, we further discuss scheduling/measurement restrictions requirements for FR2 multi-Rx reception [2].

2	Discussion

### Issue 3-1-1: Conditions/cases that scheduling restriction for L1 measurements can be relaxed for multi-Rx <Way forward >: Company is encouraged to bring analysis on high-level principle in the next meeting. ### Issue 3-1-3: Conditions/cases that scheduling restriction for L3 measurements can be relaxed for multi-Rx <Way forward >: - Proposals - Option 1: - For scheduling restriction for L3 measurement, if UE supports multi-Rx chain and the multi-Rx chain is enabled: - if the spatial separation is large enough between the RS for L3 measurement and the CSI-RS for TCI of PDCCH or PDSCH, no scheduling restriction shall be applied on this L3 measurement occasions. - UE may need to indicate scheduling restriction information to network. Details of indication and signaling can be FFS, and needs more conclusions from RF session. - Option 2: - Reuse existing scheduling restriction for L3 measurement based on multi-Rx chain scenario in this release. - Option 3: - For L3 measurement, once the UE and NW achieve alignment on the Rx beam sweeping pattern between multiple panels, simultaneous L3 measurement and data reception is feasible given that the RS used for L3 measurement and data are transmitted from different TRPs. Therefore, the scheduling restriction on L3 measurement can be partially relaxed. - Option 4: - Scheduling restriction requirements for L3 measurements without gaps cannot be enhanced for multi-Rx UE. - Option 5: - For R18 multi-Rx DL receptions, it is suggested not to consider simultaneous data reception and L3 measurement, and the existing scheduling restrictions requirements due to L3 measurements still need to be applied. - Option 6a: - For multiRx UEs, during L3 measurements, scheduling restrictions can be relaxed during the SMTC by temporarily reducing to only 1 indicated TCI state (instead of 2), i.e. reduction of 4-layer to 2-layer. - Option 6b: - RAN4 to define requirements where 1 TCI state is expected to be available for data while another is unavailable for measurements during an SMTC occasion. - Option 6c: - RAN4 to define requirements where TCI state with lower RSRP or QCI is unavailable in more SMTC occasions than TCI state with higher RSRP or QCI. - Option 6d: - RAN4 to define a rule based on SFN to determine which TCI state is experiencing scheduling restrictions. - Option 6e: - Determine scheduling restrictions based on the following rule: - If mod(ISMTC, L+M) < M, Scheduling restrictions apply for TCI #2 and TCI#1 have no scheduling restrictions, otherwise scheduling restrictions apply for TCI #1 and TCI#2 have no scheduling restrictions - Where ISMTC = SFN*10ms/TSMTC it he SMTC index - L and M are the number of SMTC occasions used for TCI#1 and TCI#2, which are termined as - L=6 and M=2 is RSRP_1-RSP_2 > X dB - L=2 and M=6 is RSRP_2-RSRP_1 > X dB - L=M=4 otherwise

The measurement restrictions for RLM/BFD/CBD and L1-RSRP measurement could be revisited for R18 UE capable of simultaneous DL reception from different directions with different QCL TypeD RSs on one of component carriers. The followings are some examples:

- Case 1: CSI-RS+SSB overlapped in the same OFDM symbol

e.g., UE could be able to receive CSI-RS for RLM in the PRBs that overlap with an SSB in the same OFDM symbol, or UE is expected to measure both CSI-RS for RLM and SSB in the same OFDM symbol.

- Case 2: SSB+ CSI-RS with same or mixed numerology

- e.g., If UE is not capable of IBM on one FR2 band pair, UE could be possible to perform the related SSB based measurements without any measurement restrictions when the network configures same or mixed numerology between SSB for L1-RSRP measurement and CSI-RS for RLM, BFD, CBD, L1-RSRP or L1-SINR measurement.

- Case 3: CSI-RS+ CSI-RS overlapped in the same OFDM symbol

e.g., When the CSI-RS for L1-RSRP measurement on one CC is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement on the same CC or different CCs in the same band, UE is required to measure one of but not both CSI-RS for L1-RSRP measurement and the other CSI-RS if these two CSI-RS-es are not QCL-ed w.r.t. QCL-TypeD, or the QCL information is not known to UE in the existing requirement. However, for R18 UE capable of simultaneous DL reception from different directions with different QCL TypeD CSI-RSs, such measurement restriction could be removed.

Observation 1：For UE supporting simultaneous DL reception from different directions with different QCL TypeD RSs on one of component carriers, measurement restrictions for RLM/BFD/CBD and L1-RSRP measurement could be revisited at least for the following cases:

- Case 1: CSI-RS+SSB overlapped in the same OFDM symbol

- Case 2: SSB+ CSI-RS with same or mixed numerology

- Case 3: CSI-RS+ CSI-RS overlapped in the same OFDM symbol

Based on the above observations, we could further consider to remove some measurement restrictions of L1 measurement for UE capable of simultaneous multi-Rx reception.

Proposal 1: Some measurement restrictions of L1 measurement can be removed for UE capable of simultaneous multi-Rx reception.

Similarly, if simultaneous receptions of L1 measured RS and data scheduling are allowed, the scheduling restrictions could be also discussed later based on the conclusion of UE capability and scenarios of simultaneous Rx.

Proposal 2: Scheduling restriction requirements can be discussed later based on the conclusion of UE capability and scenarios of simultaneous reception.

3	Conclusion

In this contribution, we propose our analysis and views on measurement/scheduling restriction for L1 enhancements of FR2 multi-RX DL reception.

Observation 1：For UE supporting simultaneous DL reception from different directions with different QCL TypeD RSs on one of component carriers, measurement restrictions for RLM/BFD/CBD and L1-RSRP measurement could be revisited at least for the following cases:

- Case 1: CSI-RS+SSB overlapped in the same OFDM symbol

- Case 2: SSB+ CSI-RS with same or mixed numerology

- Case 3: CSI-RS+ CSI-RS overlapped in the same OFDM symbol

Proposal 1: Some measurement restrictions of L1 measurement can be removed for UE capable of simultaneous multi-Rx reception.

Proposal 2: Scheduling restriction requirements can be discussed later based on the conclusion of UE capability and scenarios of 'simultaneous reception.

4	Reference

[1] R4-2306318, WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 1), vivo

[2] RP-223527, Revised WID: Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception, Qualcomm Incorporated