3GPP TSG-RAN WG4 Meeting # 107		R4-2308481

# Incheon, KR, May 22 – May 26, 2023

Agenda Item:	8.9.2.2

Source: 	OPPO

Title: 	Discussion on L1 enhancement for FR2 Scell activation delay reduction

Document for:	Approval

1	Introduction

A WF on FR2 SCell activation enhancement was approved in RAN4#106bis meeting [1]. In this contribution, we further discuss the L1 related requirements for FR2 SCell activation delay reduction.

2	Discussion

Enhancement for L1-RSRP

Issue 2-1-2: Whether and how to skip L1-RSRP measurement of FR2 unknown SCell activation (for the case when no valid L3 measurement result is reported after SCell activation command)? FFS: - Option 1 (Apple, Nokia, CMCC, LGE, CTC, NTT DCM, OPPO, Xiaomi, ZTE, OPPO, Ericsson): - if L3 and L1 measurement are using same RS or QCLed type D RSs, skip L1-RSRP measurement and use measurement result from L3 stage for L1-RSRP reporting. - Option 1a (Apple, LGE, CTC): - if L3 measurement is performed without L3 part enhancement - Option 2 (Intel): - if L1-RSRP report can be re-used for L3 measurement results, two sets of requirement and applicability needs to be defined. - Option 3 (QC): - Defining union delay requirements for cell detection and L1 measurement instead of discussing whether skipping L1 measurement. E.g ) X*TSSB where X = total beam sweeping factor for both cell detection and L1 measurement, FFS : X value. - Option 4 (MTK): - L1-RSRP measurement of FR2 unknown SCell activation can be skipped if NW can trigger the UE to report the latest RSRP measurements from L3 part given that no enhancement is applied in L3 part. - Option 5 (vivo, HW): - If zero is one of the possible entry of UE capability for X2 om issue 1-2-1, the discussion of issue 2-1-2 of R4-2303228 can be merged into 1-2-1 and 2-1-1 of R4-2303228. - Option 5a (HW): - Not to discuss the explicit UE behavior on “skipping L1-RSRP” or whether UE shall derive L1-RSRP based on L3 measurement, which can already be reflected by capability indication of X2 if X2 = 0 is introduced. - Option 6 (ZTE): - When the following conditions are satisfied, L1-RSRP measurement can be ignored: - UE can identify Tx beam during L3 part – L3 part includes all same RS or QCLed Type D RS used by L1 part. - UE can identify Rx beam during L3 part –UE has swept all Rx beam needed by L1 part during L3 part - Option 7 (Intel): - if side condition of SNR>=-2dB can be satisfied, rough beam based L3 measurement can be performed during L3 sync steps, which can satisfy the measurement accuracy requirement. - The L3 measurement result obtained during L3 sync steps will be sent to NW by L1 report. L1 report can be triggered with Scell activation command or separate command. - If new L1 reporting method for L3 measurement result can be defined and there is available L3 measurement result, UE can skip both L1-RSRP measurement and L1-RSRP reporting in Scell activation procedure

As discussed in last meeting, UE is assumed to be able to derive some rough beam information during L3 measurement, which can be reused for L1-RSRP measurement. If L1-RSRP measurement of FR2 unknown SCell activation is not skipped, RAN4 can consider Rx beam sweeping factor reduction for L1-RSRP. RAN4 also agreed to discuss the conditions for skipping L1-RSRP measurement when activating an FR2 unknown SCell.

Observation 1: Assuming measurement results from L3 measurement can be used for L1-RSRP reporting, the Rx beam sweeping factor for L1-RSRP measurement can be reduced, or even L1-RSRP measurement can be skipped.

Once cell detection has been completed, UE may have some knowledges about Rx beams of to-be-activated SCell. If SSB is used to perform L1-RSRP measurement for UE, only candidate target beams other than all beams need to be measured, e.g., utilizing the best beam selected during cell detection. If the SSB has been measured during L3 synchronization or measurement, UE may skip L1-RSRP/beam measurement, and directly report SSB-RSRP to network based on L3 measurement.

If L3 measurement and L1 measurement are using same RS or QCLed type D RSs, UE sweeps all Rx beam needed by L1 part during L3 part. Thus, UE can skip L1-RSRP measurement by using measurement result from L3 stage for L1-RSRP reporting.

Proposal 1: UE can skip L1-RSRP measurement by using measurement results from L3 stage for L1-RSRP reporting, if L3 measurement and L1 measurement are using same RS or QCLed type D RSs

3	Conclusion

Observation 1: Assuming measurement results from L3 measurement can be used for L1-RSRP reporting, the Rx beam sweeping factor for L1-RSRP measurement can be reduced, or even L1-RSRP measurement can be skipped.

Proposal 1: UE can skip L1-RSRP measurement by using measurement results from L3 stage for L1-RSRP reporting, if L3 measurement and L1 measurement are using same RS or QCLed type D RSs

4	Reference

[1] R4-2306315, WF on R18 RRM enhancement- FR2 SCell activation delay reduction, Apple