3GPP TSG-RAN WG4 Meeting # 107		R4-2308482

# Incheon, KR, May 22 – May 26, 2023

Agenda Item:	8.9.2.3

Source: 	OPPO

Title: 	Discussion on other enhancements for FR2 Scell activation delay reduction

Document for:	Approval

1	Introduction

A WF on Rel-18 eFeRRM- FR2 SCell activation delay reduction was approved in RAN4#106bis meeting [1].

In this contribution, we further discuss other potential enhancements for FR2 SCell activation delay reduction.

2	Discussion

Issue 3-1-1: Scheduling enhancement for FR2 unknown SCell activation FFS: - Option 1 (Apple, QC, OPPO, HW, Ericsson, ZTE): the early scheduling is network implementation and it’s not precluded in existing SCell activation requirement. No enhancement is needed. - Option 1a(ZTE): - For FR2, the initial scheduling can be decided by the NW after receiving the beam info and signal quality info after L1-RSRP report. For FR1, once the signal quality info derived by NW, the initial scheduling is possible. - Option 2 (Nokia): - The potential to enable earlier data transmission within the activation period shall be studied. - The UE shall be scheduled by the network immediately after L3 or L1-RSRP reporting. - Starting from receiving the TCI activation command, the UE shall report CSI, instead of OoR, if the UE has available uplink resources to report CQI for the SCell.

In our understanding, if network wants to schedule this UE before valid CQI reporting, it’s possible and allowed, but whether this UE will response or react correctly to this scheduling is up to UE.

For example, when the UE has DL timing and beam/TCI information, UE can be scheduled and such early scheduling is not precluded. As long as UE is ready on timing and beam, UE will receive such scheduled data to increase the throughput and reduce latency. Thus, it is up to network implementation and no enhancement is needed in the spec.

Proposal 1: No scheduling enhancement is needed for FR2 unknown SCell activation, which is up to network implementation.

3	Conclusion

Proposal 1: No scheduling enhancement is needed for FR2 unknown SCell activation, which is up to network implementation.

4	Reference

[1] R4-2306315, WF on R18 RRM enhancement- FR2 SCell activation delay reduction, Apple