3GPP TSG-RAN WG4 Meeting # 107		R4-2308487

# Incheon, KR, May 22 – May 26, 2023

Agenda Item:	8.25.2.3

Source: 	OPPO

Title: 	On L1/L2 inter-cell mobility delay requirements

Document for:	Approval

1	Introduction

A WF on Rel-18 Mobility enhancement was approved in RAN4#106bis meeting [1].

In this contribution, we discuss the remaining issues on L1/L2 inter-cell mobility delay requirements.

2	Discussion

General and Principles

Issue 3-1-1: Clarification on “without SCell change” < Agreement>: - Keep the existing agreement. Further check whether clarification is needed while capturing it in CR Issue 3-1-2: Whether to define cell switch delay requirements for SCell addition (without activation) at SpCell change < Agreement>: - No need to define requirements for SCell during SCell addition (without activation) at SpCell change Issue 3-1-3: Whether to define requirements for Direct SCell activation at SpCell change (target SCell is not current PCell) < Agreement>: - No more discussion until RAN2 concludes to support direct SCell activation at SpCell change Issue 3-1-4: Whether to define requirements for Direct SCell activation at SpCell change (target SCell is current PCell) < Agreement>: - Specify cell switch requirements for the following scenarios: - Target Pcell/SCell is current SCell/PCell. Issue 3-1-5: SpCell cell switch delay requirements for SpCell change with SCell change < Way forward >: FFS the following options - Option 1: For the scenario “PCell change with SCell change”, PCell switch delay is not extended by SCell changes, i.e., UE is supposed to perform SCell change after cell switch of PCell is finished. - Option 2: FFS Issue 3-1-6: LTM delay requirements < Way forward >: FFS the following options - Option 1: Not define the LTM delay requirement which starts from UE receives RRC configuration on candidate cell(s). - Option 2: FFS

Based on RAN2’s guidance, we think PSCell change without MN involvement could have higher priority in this release. So, considering the workload, we support to not define cell switch delay requirements for the case “PCell change with PSCell change”. We are open to define cell switch delay requirements for SpCell change with SCell change in this release since they are in the scope clearly.

We slightly prefer to consider the case as higher priority that target Pcell/SCell is current SCell/PCell. For issue 3-1-5, most companies expects PCell switch delay is not extended by SCell changes, i.e., UE is supposed to perform SCell change after cell switch of PCell is finished. We think role change between PCell and SCell within MCG or PScell and Scell within SCG could be more possible and reduce the delay in LTM.

Proposal 1: For SpCell change with SCell change, role change within the same CG are supported to reduce the delay in LTM.

Timeline of cell switch delay

Issue 3-2-1: Ending point of RACH-less cell switch delay for PCell/PSCell < Agreement>: - Wait for more progress in RAN2 Issue 3-2-2: Procedure of cell switch < Way forward >: FFS the following options - Option 1: Further discuss whether UE can perform T/F fine tracking (TΔ) if needed at first and then L1/L2/L3 processing (Tprocessing,2) to reduce the interruption time during cell switch. - Option 2: Under the condition that target cell is known, UE can perform T/F fine tracking (TΔ) if needed at first and then L1/L2/L3 processing (Tprocessing,2) to reduce the interruption time during cell switch. - Option 3: if T/F fine tracking (TΔ) is needed after receiving cell switch command, UE is not required to perform it before L1/L2/L3 processing (Tprocessing,2). - Option 4: UE cannot perform T/F tracking before necessary L1 reconfiguration, which get the UE's L1 ready to receive DL of the target cell. If needed, some other L1 configuration, L2/L3 reconfiguration and L2 reset can be performed during cell switch, i.e., in some cases it can be performed after the T/F tracking.

As agreed in last RAN4 meeting, for RACH-based case, the timeline for L1/L2-based inter-cell mobility is defined as the time UE receives the cell switch command to UE starts transmission of the new uplink PRACH channel to the target cell.

RAN2 assumes that both RACH-based (CFRA, CBRA) and RACH-less procedures for L1 L2 mobility switch may be supported. For RACH-less, the UE is not expected to acquire TA during the cell switch, but before cell switch command is received in L1/L2 based mobility. The feasibility of RACH-less may depend on RAN1, and RAN1 is working on RACH-based or RACH-less solutions of acquire TA of candidate cells.

From RAN4’s perspective, both RACH-based and RACH-less based timeline for L1/L2 mobility delay need to be defined if RACH-less and RACH-based case are supported.

- For RACH-less case (if supported), the timeline for L1/L2-based inter-cell mobility is defined as the time UE receives the cell switch command to UE performs the first DL/UL reception/transmission on the indicated beam of the target cell.

- For RACH-based case (if supported), the timeline for L1/L2-based inter-cell mobility is defined as the time UE receives the cell switch command to UE starts transmission of the new uplink PRACH channel to the target cell.

The baseline of RACH-based Cell switch delay requirements for PCell/PSCell is command processing time and interruption: DL1/L2_mobility = Tcmd + Tinterrupt.

- Tcmd is L1/L2 command processing delay, e.g. MAC/DCI decoding time;

- Tinterrupt is the data interruption time during the mobility procedure including all the other components in L1/L2 inter-cell mobility delay except L1/L2 command processing delay (Tcmd). L1/L2 inter-cell mobility delay (DL1/L2_mobility) is defined as

Proposal 2: DL1/L2_mobility = Tcmd + Tinterrupt, where Tinterruption includes all the other components in L1/L2 inter-cell mobility delay except L1/L2 command processing delay (Tcmd).

The interruption should at least consider the following components and the possible reduction: UE processing time, Cell search time, Fine timing tracking time and Uncertainty delay.

- RAN2 agreed that a L1/L2 inter-cell mobility candidate (target) configuration is received within an RRC message before the L1/L2 dynamic switch is triggered. RAN2 assumes L1/2 mobility trigger information is conveyed in a MAC CE, FFS if the MAC CE or a DCI is used for the actual triggering. Thus, the MAC/DCI decoding delay should be one component instead of RRC processing delay in the timeline.

- For UE processing, the following is assumed to be performed after receiving the cell switch command: MAC/RLC reset (when configured), RF retuning (e.g. needed for inter-frequency), baseband retuning. No security update support in Rel-18 with L1/L2 based mobility.

- For cell search, the target/candidate cell is assumed to be known or current SCell. In this case, Tsearch = 0. Otherwise, for inter-frequency neighbor cell which has not detected before, Tsearch could be long duration which is not an expected scenario for L1/L2 based faster mobility.

- Similar to L3 mobility, TΔ for fine time tracking and acquiring full timing information and Tmargin for SSB or CSI-RS post-processing are also needed for L1/L2 based mobility.

- TIU is also needed due to interruption uncertainty in acquiring the first available PRACH occasion in the new cell or the first DL/UL reception/transmission on the indicated beam.

- We prefer to consider known TCI state only and extra TCI state switching delay is not needed. If TCI state is unknown, UE is supposed to perform L1-RSRP measurement during cell switch, which leads to much longer delay and interruption, compared to legacy L3 HO. There is no benefit to trigger cell switch for unknown case.

In summary, we propose to consider the following components for L1/L2 based mobility delay:

Proposal 3: L1/L2 based inter-cell mobility delay should consider at least the following components

- Tcmd : L1/L2 command processing delay, e.g. MAC/DCI decoding time

- Tprocessing : UE processing time including MAC/RLC reset (when configured), RF retuning and baseband retuning

- Cell search time: if the target/candidate cell is assumed to be known or current SCell, Tsearch = 0

- TΔ : fine timing tracking time

- Tmargin: SSB or CSI-RS post-processing

- TIU: Interruption uncertainty in acquiring the first available PRACH occasion in the new cell or the first DL/UL reception/transmission on the indicated beam

3	Conclusion

In summary, we propose to consider the followings:

Proposal 1: For SpCell change with SCell change, role change within the same CG are supported to reduce the delay in LTM.

Proposal 3: DL1/L2_mobility = Tcmd + Tinterrupt, where Tinterruption includes all the other components in L1/L2 inter-cell mobility delay except L1/L2 command processing delay (Tcmd).

Proposal 3: L1/L2 based inter-cell mobility delay should consider at least the following components

- Tcmd : L1/L2 command processing delay, e.g. MAC/DCI decoding time

- Tprocessing : UE processing time including MAC/RLC reset (when configured), RF retuning and baseband retuning

- Cell search time: if the target/candidate cell is assumed to be known or current SCell, Tsearch = 0

- TΔ : fine timing tracking time

- Tmargin: SSB or CSI-RS post-processing

- TIU: Interruption uncertainty in acquiring the first available PRACH occasion in the new cell or the first DL/UL reception/transmission on the indicated beam

4	Reference

[1] R4-2306395, WF on L1/L2 inter-cell mobility, MediaTek Inc.