3GPP TSG-RAN WG4 Meeting # 107	R4-2308505

Incheon, KR, May 22 – May 26, 2023

Agenda Item: 8.8.3.3

Source: MediaTek Inc.

Title: Discussion on RLM/BFD/CBD for multi-Rx UEs

Document for: Discussion

# Introduction

According to WF [1] and discussion summary [2], RAN4 had some discussions but most issues were not conclusion yet. In this meeting, this WI is divided into six agenda items to be discussed: (1) general issues, (2) L1 measurement, (3) RLM/BFD/CBD, (4) scheduling/measurement restriction, (5) dual TCI state switching and (6) receive timing difference. The discussion in this paper focus on the “RLM/BFD/CBD”.

# Discussion

In this section, the following sections are discussed sequentially

- Cell specific RLM/BFD/CBD for multi-RX

- TRP specific BFD/CBD for multi-RX

Before each section, we introduce our view about N factor and P factor.

For both N and P factor for RLM/BFD/CBD, we tend to believe they can reuse the proposals we discussed in our paper [3].

### 2.1 Cell specific RLM and BFD/CBD requirements enhancement for multi-Rx

The corresponding discussion [2] is provided below.

#### Issue 2-1-1: Cell specific RLM and BFD/CBD requirements enhancement for multi-Rx - Proposals - Option 1(Qualcomm): - RAN4 to not consider simultaneously formed multiple UE Rx beam based RLM and BFD/CBD on multiple resources from two TRPs. - Option 2(Apple): - Per-TRP RLM requirement is not considered. - Option 3(vivo): - Beam sweeping factor reduction is considered for SSB based and CSI-RS based RLM and BFD/CBD - RAN4 is to further study if L3 measurement results are always available to be used for RLM and BFD/CBD measurements. - Beam sweeping factor reduction is only feasible for the cases, if any, that L3 measurement results are NOT available to be used for RLM and BFD/CBD measurement. - Option 4(MediaTek): - Reducing the value of N in L1 measurement delay should be up to UE’s capability. - In L1 measurement delay requirement, P factor should not be enhanced due to collision between L1 and L3 measurement. - Option 5(NTT DOCOMO): - For R18 multi-Rx DL receptions, beam sweeping factor reduction is not considered for RLM and BFD/CBD measurements. - Option 6(Ericsson): - RLM CSI-RS: Beam sweeping factor N=1 in FR2 RLM requirements for CSI-RS, so no beam sweeping factor reduction can be applied for RLM to enhance multi-rx operation. - RLM CSI-RS: For multi-rx operation, the RLM evaluation periods (in-sync and out-of-sync) are reduced by a new scaling parameter L=1/2, provided [TBD conditions for multi-rx operation] are met, otherwise L=1. - BFD CSI-RS: Beam sweeping factor N=1 in FR2 BFD requirements for CSI-RS, so no beam sweeping factor reduction can be applied for BFD to enhance multi-rx operation. For CBD, N=8 in the current specification. Hence, the beam sweeping factor reduction approach is not efficient as a beam management multi-rx enhancement. - BFD/CBD CSI-RS: For multi-rx operation, the BFD/CBD evaluation periods (in-sync and out-of-sync) are reduced by a new scaling parameter L=1/2, provided [TBD conditions for multi-rx operation] are met, otherwise L=1. - Recommended WF - Needs further discussion.

The purpose of cell specific RLM and BFD/CBD is to monitor whether single cell or beam is becoming bad, so that UE have change to find another cell or beam. UE perform L1 SSB or CSI-RS measurement for these procedures. In multi-RX, UE may reduce beam sweeping (N) due to two active antenna modules in some case (E.g., CSI-RS with repetition), but this really depends on UE capability. So, the following proposals are provided.

Proposal 1: Reducing the value of N in L1 measurement delay (L1-RSRP/ BFD/CBD/RLM) should be up to UE’s capability.

Proposal 2: In L1 measurement (L1-RSRP/ BFD/CBD/RLM) delay requirement, P factor should not be enhanced due to collision between L1 and L3 measurement.

In addition. some company want to enhance evaluation period for RLM/BFD/CBD measurement. The corresponding discussion [2] is provided below.

#### Issue 2-1-4: Evaluation period requirements - Proposals - Option 1: - RAN4 need to investigate the applicability of the evaluation period scaling factor P in multi-Rx WI. - Option 2: - For multi-rx operation, the RLM evaluation periods (in-sync and out-of-sync) are enhanced with a new scaling parameter L. - For multi-rx operation, the BFD evaluation period is enhanced with a new scaling parameter L. - For multi-rx operation, the CBD evaluation period is enhanced with a new scaling parameter L

According to our analysis/proposal of N factor and P factor, we cannot agree option 1 and 2.

### 2.2 TRP specific BFD/CBD requirements enhancement for multi-Rx

To specify TRP specific BFD/CBD enhancement for multi-RX on below discussion [2].

#### Issue 2-2-1: TRP specific RLM and BFD/CBD requirements enhancement for multi-Rx - Proposals - Option 1: - Consider PTRP=1 for the TRP specific BFD/CBD requirements for multi-Rx operation. - Option 2: - Reuse R17 TRP specific BFD/CBD requirement for multi-RX operation.

In Rel-17 RRM requirement, we define TRP specific BFD and CBD measurement requirement for single panel. Below is an example for TRP specific BFD.

The values of PTRP defined in table 8.18.2.2-2 is 2, if SSB/CSI-RS resources in the two sets q0,0(TRP1) and q0,1(TRP2) are overlapped. Otherwise it is 1.

The value of PTRP is related to if an RS in one set is collied with another RS in another set. In terms of multi-RX chain, RAN4 only support intra-cell mTRP, we could only consider (1) SSB/CSI-RS or (2) CSI-RS/CSI-RS colliding cases. For SSB/CSI-RS case, UE should perform beam sweeping (N=8) for SSB, but should not perform beam sweeping (N=1) for CSI-RS. So PTRP has to be 2 to fulfil both expected UE behaviors. For CSI-RS/CSI-RS case, there is no need to perform beam sweeping (N=1) for TRP specific BFD, as the baseline defined in current spec. So PTRP = 1. In addition, For CSI-RS/CSI-RS case on TRP specific CBD, UE should perform beam sweeping (N=8), longer delay is expected and already defined in measurement restriction. From above analysis, we can conclude that PTRP is not always equal to 1 for multi-RX, so we cannot agree on option 1. In other words, no further changes are needed on PTRP for TRP specific BFD/CBD enhancement on multi-RX.

Observation 1: No need to introduce TRP specific BFD/CBD enhancement for PTRP on multi-RX.

From above observation 1 and option 2, RAN4 does not need to specify enhancement for TRP specific BFD/CBD delay requirement on multi-RX. So, the following proposal is suggested.

Proposal 3: Reuse R17 TRP specific BFD/CBD delay requirement on multi-RX operation.

In addition, TRP specific CBD procedure on below discussion [2].

#### Issue 2-2-2: Whether to enhance TRP specific CBD procedure Refer to R4-2300271 for details. - Proposals - Option 1: - Yes - Option 2: - No - Recommended WF - Needs further discussion.

Before discussing TRP specific CBD procedure, we should understand if this procedure could select the candidate beam for beam pair to support multi-RX operation. If only one beam from two panel is getting bad, and UE starts the TRP specific CBD. It may contradict to multi-RX operation due to the candidate beam may not form a good beam pair with existed beam. So, we think TRP specific CBD procedure should be checked further about how ensure resuming multi-RX operation.

From the following reference TS 38.133, TS38.321, TS38.331, we could know the details of TRP specific BFD/CBD procedure. In our understanding, if UE encounters beam failure on either one set of two BFD-RSs set from two TRPs, it will trigger BFR for this BFD-RS set of this TRP, i.e., indicate the list of candidate beams for beam failure recovery of this BFD-RS set. E.g., If beam failure detection on BFD-RS set #1, then UE will trigger BFR to indicate the list of candidate beams for beam failure recovery of BFD-RS set #1. So, it will never happen a case that beam failure is triggered on RSs set #1 from TRP#1, but candidate beam is indicated on RSs set #2 from TRP#2. At least, we can ensure that TRP specific BFD/CBD procedure can resume the failure beam from each RSs set with different QCL type D from two TRPs.

In addition, UE only indicates the candidate beam(s) above the rsrp-ThresholdBFR in BFR MAC CE. So, we can ensure the candidate beam with certain quality larger than threshold. With this understanding we believe that the quality of the reported candidate beam can be well controlled by network. Therefore, no new enhancement is needed for multi-Rx UEs.

Proposal 4: Rel-17 TRP specific BFD/CBD procedure can resume the failure beam from each RSs set with different QCL type D from two TRPs. No new enhancement is needed for multi-Rx UEs

Ref. TS 38.133 8.18.7 Requirements for TRP specific Beam Failure Recovery 8.18.7.1 Introduction For the UE provided with a configuration of PUCCH transmission with a link recovery request (LRR) as described in clause 9.2.4 in TS 38.213 [3], if beam recovery procedure is triggered for any of the TRPs, the UE shall transmit SR for UL resouce, followed by MAC CE providing beam index for the corresponding TRP whose radio link quality is worse than Qout,LR. The index qnew, provided in the TRP specific BFR MAC CE is provided by higher layer for a periodic CSI-RS configuration or for a SSB, as described in clause 5.17 of TS38.321 [7]. For the UE not provided with a configuration of PUCCH transmission with a link recovery request (LRR) as described in clause 9.2.4 in TS 38.213 [3], if beam recovery procedure is triggered for any of TRPs, the UE shall transmit preamble for UL-SCH resource application, followed by MAC CE providing one index for the corresponding TRP whose radio link quality is worse than Qout,LR, and the index qnew for a periodic CSI-RS configuration or for a SSB provided by higher layer, as described in clause 5.17 of TS38.321 [7], if any, for a corresponding TRP. 8.18.7.2 Requirement Provided that UE is configured by schedulingRequestID-BFR-r17 and/or schedulingRequestID-BFR2-r17for BFR set 1 (i.e., TRP1) and BFR set 2 (i.e., TRP2) respectively, a configuration for LRR in a PUCCH transmission, after BFR is triggered on any of the TRPs as described in clause 5.17 of TS38.321 [7], UE shall be capable of transmit PUCCH with a LRR within a period of T on any of the TRPs where BFR running.Where: - T = T1 x Ceil((T2+D) /T1) in which T1, T2 and D are defined as - T1 is equal to the periodicity of PUCCH configured with schedulingRequestID-BFR-r17 for BFR set 1 schedulingRequestID-BFR2-r17 for BFR set 2.. - T2 = TEvaluate_CBD is the evaluation period specified in clause 8.18.5 or 8.18.6 for SSB or CSI-RS based candidate beam detection, that is TEvaluate_CBD_SSB or TEvaluate_CBD_CSI-RS, depending on the applicable reference signal configured for candidate beam detection. - D = [2ms] is the UE Processing time.. Ref. TS 38.213 9.2.4 UE procedure for reporting SR … A UE can be provided by schedulingRequestID-BFR a first configuration for LRR and, if the UE provides twoLRRcapability, the UE can be provided by schedulingRequestID-BFR2 a second configuration for LRR in a PUCCH transmission using either PUCCH format 0 or PUCCH format 1. schedulingRequestID-BFR-r17 Indicates the scheduling request configuration (SchedulingRequestConfig) that the UE shall use upon detecting a beam failure on the detection resources configured in schedulingRequestID-BFR of a serving cell while beam failure is not detected on resources configured in failureDetectionSet2 of the same serving cell. schedulingRequestID-BFR2-r17 Indicates the scheduling request configuration (SchedulingRequestConfig) that the UE shall use upon detecting a beam failure on the detection resources configured in schedulingRequestID-BFR2 of a serving cell while beam failure is not detected on resources configured in failureDetectionSet1 of the same serving cell. BeamFailureDetection-r17 ::= SEQUENCE { failureDetectionSet1-r17 BeamFailureDetectionSet-r17 OPTIONAL, -- Need R failureDetectionSet2-r17 BeamFailureDetectionSet-r17 OPTIONAL, -- Need R additionalPCI-r17 AdditionalPCIIndex-r17 OPTIONAL -- Need R } failureDetectionSet1, failureDetectionSet2 Configures parameters for beamfailure detection towards beam failure detection resources configured in the set. If additional PCIs are configured using additionalPCIToAddModList for the serving cell, each RS in one set can be associated only with one PCI. Network always configures the failureDetectionSet1 and failureDetectionSet2 together. When a failureDetectionSetN is present, after the reconfiguration, the UE shall consider all the reference signals for this failure detection set as activated if at most maxBFD-RS-resourcesPerSetPerBWP-r17 reference signals are configured for each failure detection set, otherwise the UE shall considered all the reference signals in this failure detection set as deactivated.. Ref. TS 38.321 5.17 Beam Failure Detection and Recovery procedure The MAC entity may be configured by RRC per Serving Cell with a beam failure recovery procedure which is used for indicating to the serving gNB of a new SSB or CSI-RS when beam failure is detected on the serving SSB(s)/CSI-RS(s). Beam failure is detected by counting beam failure instance indication from the lower layers to the MAC entity. If beamFailureRecoveryConfig is reconfigured by upper layers during an ongoing Random Access procedure for beam failure recovery for SpCell, the MAC entity shall stop the ongoing Random Access procedure and initiate a Random Access procedure using the new configuration. When the SCG is deactivated, the UE performs beam failure detection on the PSCell if bfd-and-RLM is set to true. RRC configures the following parameters in the beamFailureRecoveryConfig, beamFailureRecoverySpCellConfig, beamFailureRecoverySCellConfig and the radioLinkMonitoringConfig for the Beam Failure Detection and Recovery procedure: … candidateBeamRSList: list of candidate beams for SpCell beam failure recovery; - candidateBeamRS-List-r16: list of candidate beams for SCell beam failure recovery or beam failure recovery of BFD-RS set one of Serving Cell; - candidateBeamRS-List2-r17: list of candidate beams for beam failure recovery of BFD-RS set two of Serving Cell. … BeamFailureRecoveryRSConfig-r16 ::= SEQUENCE { rsrp-ThresholdBFR-r16 RSRP-Range OPTIONAL, -- Need M candidateBeamRS-List-r16 SEQUENCE (SIZE(1..maxNrofCandidateBeams-r16)) OF CandidateBeamRS-r16 OPTIONAL, -- Need M ..., [[ candidateBeamRS-List2-r17 SEQUENCE (SIZE(1..maxNrofCandidateBeams-r16)) OF CandidateBeamRS-r16 OPTIONAL -- Need R ]] } … 1> if the Serving Cell is configured with two BFD-RS sets: 2> if beam failure instance indication for a BFD-RS set has been received from lower layers: 3> start or restart the beamFailureDetectionTimer of the BFD-RS set; 3> increment BFI_COUNTER of the BFD-RS set by 1; 3> if BFI_COUNTER of the BFD-RS set >= beamFailureInstanceMaxCount: 4> trigger a BFR for this BFD-RS set of the Serving Cell; 2> if BFR is triggered for both BFD-RS sets of the SpCell and the Beam Failure Recovery procedure is not successfully completed for any of the BFD-RS sets: 3> initiate a Random Access procedure (see clause 5.1) on the SpCell; 2> if the Serving Cell is SpCell and the Random Access procedure initiated for beam failure recovery of both BFD-RS sets of SpCell is successfully completed (see clause 5.1): 3> set BFI_COUNTER of each BFD-RS set of SpCell to 0. 3> consider the Beam Failure Recovery procedure successfully completed. … All BFRs triggered for an SCell shall be cancelled when a MAC PDU is transmitted and this PDU includes a MAC CE for BFR which contains beam failure information of that SCell. All BFRs triggered for a BFD-RS set of a Serving Cell shall be cancelled when a MAC PDU is transmitted and this PDU includes an Enhanced BFR MAC CE or Truncated Enhanced BFR MAC CE which contains beam failure recovery information of that BFD-RS set of the Serving Cell. 6.1.3.23 BFR MAC Ces The fields in the BFR MAC CEs are defined as follows: .… - AC: This field indicates the presence of the Candidate RS ID field in this octet. If at least one of the SSBs with SS-RSRP above rsrp-ThresholdBFR amongst the SSBs in candidateBeamRSSCellList or the CSI-RSs with CSIRSRP above rsrp-ThresholdBFR amongst the CSI-RSs in candidateBeamRSSCellList is available, the AC field is set to 1; otherwise, it is set to 0. If the AC field set to 1, the Candidate RS ID field is present. If the AC field set to 0, R bits are present instead; - Candidate RS ID: This field is set to the index of an SSB with SS-RSRP above rsrp-ThresholdBFR amongst the SSBs in candidateBeamRSSCellList or to the index of a CSI-RS with CSI-RSRP above rsrp-ThresholdBFR amongst the CSI-RSs in candidateBeamRSSCellList. Index of an SSB or CSI-RS is the index of an entry in candidateBeamRSSCellList corresponding to the SSB or CSI-RS. Index 0 corresponds to the first entry in the candidateBeamRSSCellList, index 1 corresponds to the second entry in the list and so on. The length of this field is 6 bits. .….

# Summary

In this paper, the discussion of R18 multi-Rx chain WI is provided. We have the following proposals:

Proposal 1: Reducing the value of N in L1 measurement delay (L1-RSRP/ BFD/CBD/RLM) should be up to UE’s capability.

Proposal 2: In L1 measurement (L1-RSRP/ BFD/CBD/RLM) delay requirement, P factor should not be enhanced due to collision between L1 and L3 measurement.

Observation 1: No need to introduce TRP specific BFD/CBD enhancement for PTRP on multi-RX.

Proposal 3: Reuse R17 TRP specific BFD/CBD delay requirement on multi-RX operation.

Proposal 4: Rel-17 TRP specific BFD/CBD procedure can resume the failure beam from each RSs set with different QCL type D from two TRPs. No new enhancement is needed for multi-Rx UEs

# Reference

- R4-2306318, WF on NR FR2 multi-Rx chain DL reception – Part 1, Vivo, RAN4 #106-bis-e

- R4-2306392, Topic summary for [106bis-e][202] FR2_multiRx_part1, Vivo, RAN4 #106-bis-e

- R4-XXXXXXX, Discussion on L1 measurements for multi-Rx UEs, MediaTek inc.