3GPP TSG-RAN WG4 Meeting #107	R4-2308533

Incheon, South Korea, 22nd May – 26th May 2023

Agenda Item:	8.14.1.2

Source:	Ericsson

Title:	Co-existence simulation results for unsynchronized scenarios

Document for:	Discussion

# 1	Introduction

During RAN4#106bis-e, some initial agreements for co-existence simulations in unsynchronized scenarios were agreed and captured in the WF [1]. The agreements are as follows:

- For the ATG BS ACLR/ACS values: “using following assumptions for non-synchronized scenarios to derive isolated distance as starting point: 45dB ACLR and 46dB ACS for both TN and ATG BS.”

- For the location, location relationship between ATG UE and TN for non-synchronized scenario to conclude isolation distance (FFS and to conclude in RAN4#107):

  - ATG UE and TN cluster are on the same side of ATG BS, i.e., ATG BS actually point at TN cluster to consider the worst case.

  - Minimum distance between ATG BS and ATG UE is 20/50 km.

  - There is no additional restriction between ATG BS and ATG CPE. Companies are welcome to provide their results for worst case with detailed deployment illustration.

  - Following show one example for simulation

    - if isolation distance between TN cluster and ATG BS is larger than 20/50km, ATG UE is always on top of TN cluster.

    - if isolation distance between TN cluster and ATG BS is less than 20/50km, ATG UE is dropped 20/50km distant from ATG BS.

  - The isolation distance is the distance from the ATG BS to the nearest TN BS. The ATG BS sector and nearest TN BS sector should be pointing towards one another in azimuth. The diagram below is illustrative and does not capture this.

We believe that the example proposed in the WF is fine when the TN BS is the victim, i.e., scenario 5. However, when the ATG BS is the victim, i.e., scenarios 7 and 14, the example is not a good option because the relative distance between the ATG BS and the ATG UE is different for different isolation distances when the isolation distance is fixed at 20/50km than the minimum distance. This may complicate comparison between different isolation distances.

In the present contribution, we first propose a new location relationship between ATG UE and TN cluster and, second, we share some initial results based on the proposed location relationship.

# 2	Discussion

## 2.1	Location relationship between ATG UE and TN cluster

In RAN4#106-bis WF [1], some guidelines for the location relationship between ATG UE and TN cluster were captured. The example depicted in the WF is divided in two cases:

- if isolation distance between TN cluster and ATG BS is larger than 20/50km, ATG UE is always on top of TN cluster.

- if isolation distance between TN cluster and ATG BS is less than 20/50km, ATG UE is dropped 20/50km distant from ATG BS.

In our view, the proposed relationship is fine when the TN BS is the victim. However, when the ATG BS is the victim, the proposed example does not always maintain the relative distance relationship between the ATG BS and the ATG UE. Since this relationship is broken, it is neither correct nor fair to compare the throughput loss of certain cases with the baseline scenario performance.

When the ATG BS is the victim, it is neither correct nor fair to follow the example depicted in RAN4#106bis-e WF for comparing the throughput loss of certain isolation distances with the baseline scenario performance.

In order to solve this issue, we propose the following location relationship between ATG UE and TN cluster: regardless of isolation distance between TN cluster and ATG BS, ATG UE is dropped randomly at a minimum distance of 20 km in the non-subarray case and of 50 km in the subarray case and at a maximum distance of 100 km from the ATG BS. The isolation distance is the distance from the ATG BS to the nearest TN BS. The ATG BS sector and nearest TN BS sector should be pointing towards one another in azimuth. This is depicted in the example drawing below:

There are at least two advantages of the proposed deployment:

- First, in this way, we always maintain the relative distance between ATG BS and ATG UE when the ATG BS is the victim, making possible to compare the throughput degradation with a single baseline.

- Second, the reason to introduce random ATG UE positions in the range 20/50 km to 100 km is to obtain different ATG BS antenna pointings and thus with sidelobes going in a richer set of directions.

Regardless of isolation distance between TN cluster and ATG BS, ATG UE is dropped at a minimum distance to the ATG BS of 20 km in the non-subarray case and of 50 km in the subarray case and a maximum distance of 100 km. The isolation distance is the distance from the ATG BS to the nearest TN BS. The ATG BS sector and nearest TN BS sector should be pointing towards one another in azimuth.

Note that, when the TN BS is the victim, the proposed update to the aircraft position it is not likely to make a large difference compared with the example in the WF (depicted at the beginning of this section). For simplicity of the simulation, we propose this to be the location relationship between ATG UE and TN cluster for both TN BS victim and ATG BS victim.

## 2.2	Initial results for unsynchronized scenarios

In the following, we study the impact of the isolation distance between TN and ATG network. The simulation settings used for calculating the results can be found in [2]. Besides, for the results shown in this section, we have assumed 8-column non-subarray TN and ATG BSs antennas, ATG UEs uniformly distributed in altitude (between 3 and 10 km altitudes) and 16x1 array antennas at the ATG UEs.

In scenario 5 (below), the isolation distance needed to maintain the throughput degradation of the TN (most impacted cell) at 5% is around 63 km.

In scenario 7 (below), the isolation distance needed to maintain the throughput degradation of the ATG network at 5% is around 120 km.

In scenario 14 (below), the isolation distance needed to maintain the throughput degradation of the ATG network at 5% is around 232 km.

The main reason for the large difference in isolation distance between the cases where the ATG BS is the victim, i.e., scenarios 7 and 14, and the case where the TN BS is the victim, is the number of BSs in the aggressor network; the TN consists of 57 cells while the ATG network consists of a single cell.

Based on these results, it is clear that for the cases where the ATG BS is the victim, i.e., scenarios 7 and 14, the isolation distance that needs to be maintained between TN and ATG BSs is unreasonable large, and the possibilities of future deployments are extremely slim. For the case where the TN BS is the victim, i.e., scenario 5, the isolation distance needed is around 50 km. An isolation distance of 50 km may seem small if compared with the other two cases, however, it is still quite large and may be impossible to deploy ATG networks if this isolation distance needed to be fulfilled.

With these results, the most reasonable solution might be to impose TN and ATG synchronization such that no isolation distance is needed.

For the cases where the ATG BS is the victim, i.e., scenarios 7 and 14, the isolation distance that needs to be maintained between TN and ATG BSs is unreasonable large, and the possibilities of future deployments are extremely slim.

For the case where the TN BS is the victim, i.e., scenario 5, the isolation distance needed is around 50 km. An isolation distance of 50 km may seem small if compared with the other two cases, however, it is still quite large and may pose difficulties for ATG network deployment if this isolation distance needed to be fulfilled.

TN and ATG synchronization is necessary to avoid unfeasible isolation distances.

TN and ATG synchronization implies that the UL and DL slots need to be the same between TN BSs and ATG BSs.

# Conclusion

In the previous sections we made the following observations:

Observation 1	When the ATG BS is the victim, it is neither correct nor fair to follow the example depicted in RAN4#106bis-e WF for comparing the throughput loss of certain isolation distances with the baseline scenario performance.

Observation 2	For the cases where the ATG BS is the victim, i.e., scenarios 7 and 14, the isolation distance that needs to be maintained between TN and ATG BSs is unreasonable large, and the possibilities of future deployments are extremely slim.

Observation 3	For the case where the TN BS is the victim, i.e., scenario 5, the isolation distance needed is around 50 km. An isolation distance of 50 km may seem small if compared with the other two cases, however, it is still quite large and may pose difficulties for ATG network deployment if this isolation distance needed to be fulfilled.

Observation 4	TN and ATG synchronization implies that the UL and DL slots need to be the same between TN BSs and ATG BSs.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Regardless of isolation distance between TN cluster and ATG BS, ATG UE is dropped at a minimum distance to the ATG BS of 20 km in the non-subarray case and of 50 km in the subarray case and a maximum distance of 100 km. The isolation distance is the distance from the ATG BS to the nearest TN BS. The ATG BS sector and nearest TN BS sector should be pointing towards one another in azimuth.

Proposal 2	TN and ATG synchronization is necessary to avoid unfeasible isolation distances.

# References

R4-2306607, WF on ATG co-existence evaluation, CMCC, RAN4#106bis-e.

R4-2304282, TR 38.876 ATG v0.3.0, CMCC, RAN4#106bis-e.