3GPP TSG-RAN4 Meeting #107	R4-2308546

# Incheon, Korea, 22-26 May 2023

Source:			Ericsson

Title:			Aerial UEs - LTE

Agenda item:		9.9.2

Document for:		Approval

# Introduction

The Work Item on UAV ([1]) was revised in last RAN#99 meeting to address the ETSI TFES request and request RAN4 to consider the additional OOBE limits for aerial UEs which were regulated by ECC Decision(22)07.

Based on the agreed way forward [4] from last RAN4#106-bis meeting, this contribution discusses the remaining open issues.

Note: this contribution has also been submitted ([5]) under AI 8.36.2 as it covers UAV topics for both LTE and NR: we thought it was not relevant to split this contribution in 2 to keep consistent any decision taken based on below proposals.

# Discussion

## Definition

We would propose the following definition of an aerial UE in TS 38.101-1 and TS 36.101, based on ECC Decision(20)07:

Proposal1: Adopt the following definition:

Aerial UE:  UE supporting UAS (Unmanned Aircraft Systems) features and services and requiring an aerial subscription.

## Power class

In the agreed way forward, aerial UE power class was left for further discussion.

ECC Decision(20)07 doesn’t clearly stipulate any aerial UE power class, nor UE maximum output power. Nevertheless, this Decision refers to ECC Report 309 and ECC Report 348 which capture the analysis made to come to that decision.

Looking further at ECC Report 309, only 23 dBm UE was considered in the coexistence studies in the 1920-1980 MHz frequency range (tables 71, 80 and 103) , in the 3.-3.8 GHz frequency range (tables 89 and 93) and in the 700, 800 and 2600 MHz bands (tables 9 and 28).

Also, when comparing LTE and NR technologies, ECC Report 348 only consider 23 dBm UE (table 3).

Based on those observations, we consider aerial UE could reuse UE power class 3.

Proposal2: Specify Aerial UE power class based on UE PC3 only.

## Regional requirements

The agreed WF identified the following options to specify regional requirement for aerial UEs:

With the NS based approach, one alternative is to specify the regional requirements and associated A-MPR reduction by introducing new NS values specific to aerial UE, and the non-aerial UE should ignore those NSs.

Moreover, the aerial UE would have to support this new UAV NS in addition  to those already specified for the considered band if there is any requirement not specific to UAV-specific in the band. The new UAV-specific NS should be the first in the list of the NSs sent (to make sure the UAV UE will apply this one). Then, legacy UEs would apply the second in the advertised NS list, which could be NS_01. However, this might cause some issue with some legacy UE implementations which are expecting to find the NS to be used in the first position in the list notwithstanding the RRC specification.

Another alternative would be to reuse an existing regional NS other than NS_01 for the bands of interest and re-specify the NS by adding the UAV regional requirements. But this alternative could only work if the existing NS are also (and only) applicable to CEPT countries.

With NR, the first alternative might work, even if there are some uncertainties to clarify.

But with LTE, it won’t be possible to add any new NS to an existing band. The behaviour of a legacy UE receiving an unknown NS is not specified, the legacy UEs might not attach anymore to the network.

Looking at the 2nd alternative (re-specify an existing NS for UAV UE), only band 38 has a specified NS, bands 3 and 7 don’t have any NS in TS 36.101. This alternative is then not possible for LTE either.

To solve this, we would propose then to specify 3 new UAV LTE bands which would correspond to band 3, 7 and 38. The additional CEPT requirements for UAV could still be specified via NS value indicating the CEPT requirement so that those band numbers could also be used in other regions.

To be consistent and avoid any uncertainties, we would propose to use the same approach for NR then, specifying 3 new UAV NR bands (as it was done for bands n90 and n41) and specifying the additional CEPT UAV requirements via NSs.

Proposal3: Specify 3 LTE bands and 3 NR bands to introduce aerial UE support and CEPT regional requirements for UAV operation in bands 3/n3, 7/n7 and 38/n38.

# Conclusion

In this contribution we discussed the impacts of ECC Decision(22)07 and the additional OOBE limits for aerial UEs. We made the following proposals:

Proposal1: Adopt the following definition:

Aerial UE:  UE supporting UAS (Unmanned Aircraft Systems) features and services and requiring an aerial subscription.

Proposal2: Specify Aerial UE power class based on UE PC3 only.

Proposal3: Specify 3 LTE bands and 3 NR bands to introduce aerial UE support and CEPT regional requirements for UAV operation in bands n3, n7 and n38.

# References

- RP-230782, Revised WID: NR Support for UAV (Uncrewed Aerial Vehicles), Nokia, Nokia Shanghai Bell

- ECC Decision(22)07, Harmonised technical conditions for the usage of aerial UE for communications based on LTE and 5G NR in the bands 703-733 MHz, 832-862 MHz, 880-915 MHz, 1710- 1785 MHz, 1920-1980 MHz, 2500-2570 MHz and 2570- 2620 MHz harmonised for MFCN, 18 November 2022

- R4-2304578, Draft CR to TS 38.101-1 - Introduction of additional OOBE for Aerial UEs , Ericsson

- R4-2306641, WF on NR Support for UAV, Nokia

- R4-2308545, Aerial UEs – NR, Ericsson