3GPP TSG-RAN WG4 Meeting # 107	R4-2308577

Incheon, KR, May 22 – May 26, 2023

Source: 	Huawei, HiSilicon

Title: 	Discussion on Rel-18 NTN coexistence study assumption

Agenda Item:	8.27.2

Document for:	Approval

Introduction

In RAN4#106bis meeting, a WF [1] for above 10GHz NTN-TN co-existence study was approved with a lot of agreements and some open issues for further discussion. Except for that, we still have some TBD/FFS/open issues in the simulation assumptions [2]. In this paper, we’d like to further discuss these parts and open issues.

Discussion on the relationship between NTN network and TN network

In the previous meetings, RAN4 hasn’t discussed the relationship between NTN network and TN network. Before discussing, it’s better to align the order of Rel-18 scenarios. It’s recommended to use the approved assumptions for the scenarios to discuss this issue. The approved scenarios in Rel-18 are listed as below.

| No. | Combination | Aggressor | Victim | Frequency band | Scope of Coexistence Simulation |
| --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | NTN UL | TN UL | 27 GHz | ACLR NTN UE to be varied/defined ACS TN gNB fixed |
| 2 | TN with NTN | TN UL | NTN UL | 27 GHz | ACLR TN UE fixed ACS NTN SAN to be varied/defined |
| 3 | TN with NTN | NTN UL | TN DL | 27 GHz | ACLR NTN UE to be varied/defined ACS TN UE fixed |
| 4 | TN with NTN | TN DL | NTN UL | 27 GHz | ACLR TN gNB fixed ACS NTN SAN to be varied/defined |
| 5 | TN with NTN | TN DL | NTN DL | 17 GHz | ACLR TN gNB fixed ACS NTN UE to be varied/defined |
| 6 | TN with NTN | NTN DL | TN DL | 17 GHz | ACLR NTN SAN to be varied/defined ACS TN UE fixed |
| 7 | TN with NTN | NTN DL | TN UL | 17 GHz | ACLR NTN SAN to be varied/defined ACS TN gNB fixed |
| 8 | TN with NTN | TN UL | NTN DL | 17 GHz | ACLR TN UE fixed ACS NTN UE to be varied/defined |
| NOTE 1: For coexistence between Ka-Band DL and adjacent TN bands, there are no 3GPP defined/specified TN bands. |  |  |  |  |  |

Proposal 1: After aligning the order of Rel-18 scenarios, it’s recommended to discuss the following relationship between TN and NTN for Ka band coexistence study.

| No. | Combination | Aggressor | Victim | Which NTN cell/UE to observe? | Which TN/UE to observe? | Which TN cells in a TN to observe? |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | NTN UL | TN UL | NTN cell: Nadir point. NTN UE: Option 1: NTN UEs dropped at the edge of TN clusters (NTN UE outside of the TN cluster) Option 2: NTN UEs dropped in TN clusters (NTN UE inside TN cluster) Option 3: NTN UEs dropped in or at the edge of TN clusters with a possibility (Random number). FFS on possibility. | TN randomly placed in this NTN beam | Option 1: All active TN clusters which has the NTN UE(s) at its edge. Option 2: Only the active TN clusters which contain the NTN UE(s). Option 3: All active TN clusters no matter whether the NTN UE(s) is (are) included in the TN cluster or at the edge of TN clusters. |
| 2 | TN with NTN | TN UL | NTN UL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: Follow case 1. | Consider an active rate of [20%] for TN. | All active TN cells in central NTN beam |
| 3 | TN with NTN | NTN UL | TN DL | NTN cell: Nadir point NTN UE: Follow case 1. | TN clusters randomly placed in this NTN beam | Follow case 1. |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (45° for GEO and LEO，Interested companies can bring analysis and results for other values). NTN UE: Follow case 1. | TN clusters randomly placed in this NTN beam | Follow case 1. |
| 4 | TN with NTN | TN DL | NTN UL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: Follow case 1. | Consider the active rate of [20%] for TN. | Follow case 1. |
| 5 | TN with NTN | TN DL | NTN DL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: Option 1: NTN UEs dropped at the edge of TN clusters (NTN UE outside of the TN cluster) Option 2: NTN UEs dropped in TN clusters (NTN UE inside TN cluster) Option 3: NTN UEs dropped in or at the edge of TN clusters with a possibility (Random number). FFS on possibility. | One cluster with 19 TN cells (57 sectors) randomly placed in the central NTN beam | Option 1: All active TN clusters which has the NTN UE(s) at its edge. Option 2: Only the active TN clusters which contain the NTN UE(s). Option 3: All active TN clusters no matter whether the NTN UE(s) is (are) included in the TN cluster or at the edge of TN clusters. |
| 6 | TN with NTN | NTN DL | TN DL | NTN cell: Nadir point. NTN UE: Follow case 5 conclusion | TN clusters randomly placed in this NTN beam | All in central NTN beam |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (45° for GEO and LEO，Interested companies can bring analysis and results for other values) NTN UE: Follow case 5 conclusion |  |  |
| 7 | TN with NTN | NTN DL | TN UL | NTN cell: Nadir point NTN UE: Follow case 5 conclusion | TN clusters randomly placed in this NTN beam | All in central NTN beam |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (45° for GEO and LEO，Interested companies can bring analysis and results for other values). NTN UE: Follow case 5 conclusion |  |  |
| 8 | TN with NTN | TN UL | NTN DL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: Follow case 5 conclusion. | One cluster with 19 TN cells (57 sectors) randomly placed in the central NTN beam | Follow case 5 conclusion |

Discussion on ACLR1 and ACLR2 model for NTN UE

In last meeting, RAN4 has the following conclusion for the number of active NTN UE (UL).

- For calibration, the number of active NTN UE (UL) is 10 UEs with divided RBs per UE by channel bandwidth.

However, it seems that RAN4 never discuss the ACLR1/ACLR2 model for Ka band UE. Based on the WF [1], the following proposals are listed.

Issue 4-2: ACLR modelling

- Proposals

  - Option 1 (Qualcomm):

    - To use clause 5.1.1.4.1 and 5.1.1.4.2 as in 3GPP TR 36.942 as baseline for the ACLR modelling.

  - Option 2 (Huawei): RAN4 further discuss how much value the ACLR2 for Ka band UE is higher than ACLR1.

- Recommended WF

  - FFS

For Ka band NTN UE ACLR modeling, ACLR1/ACLR2 model in TR 36.942 can be referred, but the key issue is how much value the ACLR2 for Ka band UE is higher than ACLR1. In FR1, 30 dB ACLR1 was assumed and specified and 43dB ACLR2 was assumed. However, there is no RF evaluation/assumption about the difference between ACLR1 and ACLR2 in Ka band. Generally, the ACLR2 value is higher than ACLR1 since the 2nd adjacent channel is far away from 1st adjacent channel.

Proposal 2: It’s proposed that RAN4 further discuss how much value the ACLR2 for Ka band UE is higher than ACLR1 from RF implementation perspective.

Discussion on Fixed VSAT and L-ESIM

In the table 2.3.1-4 of simulation assumption [2], we have the following NTN UE characteristics for system level simulations.

UE parameters

UE parameters are shown in Table 2.3.1-4

[Table 2.3.1-4 UE characteristics for system level simulations

| Characteristics | fixed VSAT | L-ESIM |
| --- | --- | --- |
| Frequency band | 27GHz | 27GHz |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter | Directional (M,N,P,Mg,Ng) = (TBD,TBD,2,1,1); (dV,dH) = (TBD, TBD)λ with directional antenna element (HPBW=65 deg) |
| Polarisation | circular | circular |
| Rx Antenna gain | 39.7 dBi | TBD dBi per element |
| Antenna temperature | 150 K | TBD K |
| Noise figure | 1.2 dB | TBD dB |
| Tx transmit power | 2 W (33 dBm) | [TBD W (TBD dBm)] |
| Tx antenna gain | 43.2 dBi | TBD dBi per element |

- FFS whether and how to consider phased array antenna pattern, not preclude to consider phased array antenna pattern in the future if needed.

The Antenna type and configurations for L-ESIM was assumed as phased array antenna. However, we have the following way forward for phased array antenna pattern in last RAN4 meeting due to the concerns on RAN4 workloads.

- Agreement: Focus on circular aperture antenna pattern on SAN side and Ka Band NTN UE side from co-existence study perspective - FFS whether and how to consider phased array antenna pattern - Not preclude to consider phased array antenna pattern in the future if needed

Following the way forward in last RAN4 meeting, it seems that RAN4 need to further discuss the assumptions about antenna type and configurations for L-ESIM.

Proposal 3: Since RAN4 has agreed to focus on circular aperture antenna pattern on the Ka band NTN UE side, reflector antenna can be considered for L-ESIM and FFS the detailed parameters.

In the WID [3], “ESIM scenarios for NGSO in Ka band are not considered in this WI.” So, ESIM coexistence study for NGSO scenarios are out of the scope of this WI.

Proposal 4: ESIM coexistence study for NGSO scenarios are out of the scope of this WI.

Summary

Based on the discussion above, the following observations and proposals are listed.

Proposal 1: After aligning the order of Rel-18 scenarios, it’s recommended to discuss the following relationship between TN and NTN for Ka band coexistence study.

| No. | Combination | Aggressor | Victim | Which NTN cell/UE to observe? | Which TN/UE to observe? | Which TN cells in a TN to observe? |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | NTN UL | TN UL | NTN cell: Nadir point. NTN UE: Option 1: NTN UEs dropped at the edge of TN clusters (NTN UE outside of the TN cluster) Option 2: NTN UEs dropped in TN clusters (NTN UE inside TN cluster) Option 3: NTN UEs dropped in or at the edge of TN clusters with a possibility (Random number). FFS on possibility. | TN randomly placed in this NTN beam | Option 1: All active TN clusters which has the NTN UE(s) at its edge. Option 2: Only the active TN clusters which contain the NTN UE(s). Option 3: All active TN clusters no matter whether the NTN UE(s) is (are) included in the TN cluster or at the edge of TN clusters. |
| 2 | TN with NTN | TN UL | NTN UL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: Follow case 1. | Consider an active rate of [20%] for TN. | All active TN cells in central NTN beam |
| 3 | TN with NTN | NTN UL | TN DL | NTN cell: Nadir point NTN UE: Follow case 1. | TN clusters randomly placed in this NTN beam | Follow case 1. |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (45° for GEO and LEO，Interested companies can bring analysis and results for other values). NTN UE: Follow case 1. | TN clusters randomly placed in this NTN beam | Follow case 1. |
| 4 | TN with NTN | TN DL | NTN UL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: Follow case 1. | Consider the active rate of [20%] for TN. | Follow case 1. |
| 5 | TN with NTN | TN DL | NTN DL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: Option 1: NTN UEs dropped at the edge of TN clusters (NTN UE outside of the TN cluster) Option 2: NTN UEs dropped in TN clusters (NTN UE inside TN cluster) Option 3: NTN UEs dropped in or at the edge of TN clusters with a possibility (Random number). FFS on possibility. | One cluster with 19 TN cells (57 sectors) randomly placed in the central NTN beam | Option 1: All active TN clusters which has the NTN UE(s) at its edge. Option 2: Only the active TN clusters which contain the NTN UE(s). Option 3: All active TN clusters no matter whether the NTN UE(s) is (are) included in the TN cluster or at the edge of TN clusters. |
| 6 | TN with NTN | NTN DL | TN DL | NTN cell: Nadir point. NTN UE: Follow case 5 conclusion | TN clusters randomly placed in this NTN beam | All in central NTN beam |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (45° for GEO and LEO，Interested companies can bring analysis and results for other values) NTN UE: Follow case 5 conclusion |  |  |
| 7 | TN with NTN | NTN DL | TN UL | NTN cell: Nadir point NTN UE: Follow case 5 conclusion | TN clusters randomly placed in this NTN beam | All in central NTN beam |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (45° for GEO and LEO，Interested companies can bring analysis and results for other values). NTN UE: Follow case 5 conclusion |  |  |
| 8 | TN with NTN | TN UL | NTN DL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: Follow case 5 conclusion. | One cluster with 19 TN cells (57 sectors) randomly placed in the central NTN beam | Follow case 5 conclusion |

Proposal 2: It’s proposed that RAN4 further discuss how much value the ACLR2 for Ka band UE is higher than ACLR1 from RF implementation perspective.

Proposal 3: Since RAN4 has agreed to focus on circular aperture antenna pattern on the Ka band NTN UE side, reflector antenna can be considered for L-ESIM and FFS the detailed parameters.

Proposal 4: ESIM coexistence study for NGSO scenarios are out of the scope of this WI.

- References

[1] R4-2306002, WF on [311] NR_NTN_enh_Part3, Samsung

[2] R4-2306003, Simulation assumptions for NTN co-existence study in bands above 10GHz, Samsung

[3] RP-223534, Revised WID: NR NTN (Non-Terrestrial Networks) enhancements, Thales