3GPP TSG-RAN WG4 Meeting #107 	R4-2308619

Incheon, Korea, May 22 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Considerations on mobile IAB RF requirements

Agenda item:	8.34.3	RF core requirements	[NR_mobile_IAB-Core]

Document for:	Approval

Introduction

RAN4#106-bis meeting continued discussion on the  work item for Mobile IAB [1]. The WI objectives specific for RAN4 were defined in work item description as follows:

RAN4 is expected to study impact on RF and RRM requirements: - Conduct co-existence study to assess the impact of moving cells. Based on the study outcome, specify RF and RRM requirements and mechanisms for the mobile IAB-node to enable co-existence, if needed. - Specify RRM requirements for the mobile IAB-node to enable IAB-node mobility, if needed.

Agreements from RAN4#106 and RAN4#106-bis meetings are captured in agreed way forwards [2, 3].

In this contribution, we continue discussion on RF core requirements issues related to studies and normative work anticipated for RAN4 considering the scope and targets of the Rel.18 Mobile IAB WI [1].

Discussion

Mobile IAB output power and power control

Following agreement is captured in WF [2] regarding RF core requirements.

Agreement: - FFS if output power and power control requirements for Rel-18 mIAB could be the same as for UE.

In [3] agreements related to mIAB output power and power control is to wait for the outcome of co-existence study.

IAB Rel-16 and Rel-17 specifications support high output power for both IAB-DU and IAB-MT as presented below:

Table 6.2.1-1: IAB-DU type 1-H rated output power limits for IAB-DU classes (from TS38.174)

| IAB-DU class | Prated,c,sys | Prated,c,TABC |
| --- | --- | --- |
| Wide Area IAB-DU | (Note) | (Note) |
| Medium Range IAB-DU | ≤ 38 dBm +10log(NTXU,counted) | ≤ 38 dBm |
| Local Area IAB-DU | ≤ 24 dBm +10log(NTXU,counted) | ≤ 24 dBm |
| NOTE: There is no upper limit for the Prated,c,sys or Prated,c,TABC of the Wide Area IAB-DU. |  |  |

Table 6.2.1-2: IAB-MT type 1-H rated output power limits for IAB-MT classes (from TS38.174)

| IAB-MT class | Prated,c,sys | Prated,c,TABC |
| --- | --- | --- |
| Wide Area IAB-MT | (Note) | (Note) |
| Local Area IAB-MT | ≤ 24 dBm +10log(NTXU,counted) | ≤ 24 dBm |
| NOTE: There is no upper limit for the Prated,c,sys or Prated,c,TABC of the Wide area IAB-MT. |  |  |

In IAB core specification TS 38.174 support very limited output power control dynamics for IAB-MT:

|  | Total power dynamic range |
| --- | --- |
| Wide area IAB-MT | ≥ 5 dB |
| Local area IAB-MT | ≥ 10 dB |

High output power and minimal power control is possible for IAB-MT due to planned network geometry for nodes which are static. However, for mobile IAB with mobility of IAB-node output power should be reconsider. For comparison NR UE output power range is 63 dB: it is from +23 up to -40 dBm.

For mobile IAB limited power control and high output power may need limits for mobile IAB-nodes that are moving physically.

There are at least two options to solve above issue:

- One option to solve this issue would be limit output power in UL to be similar to NR UE output power. It should be noted that in FR2 vehicular UE requirements already exist. One of the benefit of such approach for mobile-IAB would be that total power of IAB-Node would be smaller than total power of multiple UEs inside a vehicle.Second  option would be to specify UE-like output power control for mobile-IAB. Benefit of such solution would be better network performance. Thus it is proposed to reconsider output power and power control requirements for Rel-18 mobile-IAB.

Depends on co-existence studies, it is proposed to consider two options for IAB-MT node with mobility: either limit output power in UL, or to introduce output power control dynamics based on UE approach.

Conclusion

In this contribution, we continue discussion on RF core requirements issues related to studies and normative work anticipated for RAN4 considering the scope and targets of the Rel.18 Mobile IAB WI.

In the paper, the following Observations and Proposals were made:

Observation 1: For mobile IAB limited power control and high output power may need limits for mobile IAB-nodes that are moving physically.

Proposal 1: Depends on co-existence studies,  it is proposed to consider two options for IAB-MT node with mobility: either limit output power in UL, or to introduce output power control dynamics based on UE approach.

- References

- RP-222671, Mobile IAB (Integrated Access and Backhaul) for NR

- R4-2302910, WF for Rel-18 mIAB RF requirements impact and co-existence study, Qualcomm Incorporated

- R4-2305935, WF on mIAB RF requirements and coexistence, Qualcomm Incorporated