3GPP TSG-RAN WG4 Meeting #107	R4-2308658

Incheon, KR, 22 – 26 May, 2023

Title: 	Discussion on joint working of pre-MG and con-MG

Source: 	Huawei, HiSilicon

Agenda item:	8.10.2.2

Document for:	Discussion

Introduction

Joint working of pre-MG and con-MG are discussed in RAN4#106-bis-e and the outcomes are captured in [1]. Based on [1] the following issues need to be further discussed.

- Pre-MGs activation/deactivation procedure

- Collision handling

- Measurement requirements

In this paper we will provide our views on open issues in joint working of pre-MG and con-MG.

Discussion

Pre-MGs activation/deactivation procedure

Issue 3-2-1: [Case 1] Define definitions for simultaneous and non-simultaneous Pre-MGs activation/deactivation for Pre-MG + Pre-MG < Way forward >: FFS the following scenarios: The simultaneous case can be discussed in two scenarios, which are whether they are fully overlapped or partially overlapped, regardless of whether the Pre-MG (de)activation are triggered by the same or different events. Clarification on the fully and partially overlapping scenarios are provided in the figures below. Moderator suggests that companies provide feedback on whether the requirements should be defined for all scenarios captured in the figures. - Fully overlap: - Both Pre-MGs are triggered by the same event, as shown below: - Two Pre-MGs are triggered by 2 events of the same type at the same time, as shown below: - Partially overlap: - Two Pre-MGs are triggered by 2 events of the same type at different time, as shown below: - Two Pre-MGs are triggered by different type of events at the same time, as shown below: - Two Pre-MGs are triggered by different type of events at different time, as shown below: - Two Pre-MGs are triggered by different type of events at different time but the delay finish at the same time, as shown below:

The issue is about for which scenarios requirements for simultaneous pre-MG (de)activation should be defined. In principle, we suggest to consider only scenarios where simultaneous pre-MG (de)activation is triggered by the same event because it is corner case where pre-MG (de)activation triggered by different events overlap in time.

Fully overlapping case

Single event that can trigger fully overlapping simultaneous pre-MG (de)activation includes single CC BWP switch or SCell (de)activation, as in figure 1a. BWP switch and SCell status change in one CC may impact multiple MOs, e.g. when multiple MOs are configured in the CBW of the serving cell.

Single event that can trigger fully overlapping simultaneous pre-MG (de)activation includes also includes simultaneous multi-CC BWP switch. In simultaneous multi-CC BWP switch, no matter it is triggered by DCI, timer or RRC, the requirements assume a common delay for all CCs. This is shown in figure 1b.

Partially overlapping case

Single event that can trigger partially overlapping simultaneous pre-MG (de)activation includes multiple SCell activation with same MAC CE. In this case, the SCell activation delay for multiple CCs is different, which results in a similar case as in figure 2b.

Single event that can trigger partially overlapping simultaneous pre-MG (de)activation includes non-simultaneous timer-based multi-CC BWP switch. In this case, the BWP switch on different CCs are done in sequential order, which results in a similar case as in figure 2b.

We do not support to consider scenario 2a. Multiple SCell activation with separate MAC CEs is only considered for NR-DC case, which is not relevant to pre-MG.

We do not support to consider non-simultaneous DCI or RRC based multi-CC BWP switch. Non-simultaneous multi-CC BWP switch triggered by DCI and RRC are only considered for NR-DC case, which is not relevant to pre-MG.

We do not support to consider scenario 2b, 2c or 2d. As discussed above, they are corner case, and no requirement was defined for the case where different types of events triggers status change of a single pre-MG in overlapping time. We should follow same principle in Rel-18.

Proposal 1: Define delay requirements for simultaneous pre-MG (de)activation in following cases

- Fully overlapping:

  - triggered by single CC BWP switch or SCell (de)activation

  - triggered by simultaneous multi-CC BWP switch

- partially overlapping:

  - triggered by multiple SCell activation with same MAC CE

  - triggered by non-simultaneous timer-based multi-CC BWP switch

Issue 3-2-2: [Case 1] Whether to extend the delay for fully overlapped simultaneous activation/deactivation for Pre-MG+Pre-MG < Agreement >: - The fully overlapped simultaneous multiple Pre-MGs activation/deactivation delay equals the BWPs/SCells/RRC reconfiguration delay plus existing processing time (5ms) plus the additional post-processing time T1, where T1 value is FFS. - An illustration example is captured below: < Way forward >: - FFS the value of T1 with the following options: - Option 1: 0 ms - Option 2: < = 5 ms - Other values are not precluded

Although the status of each pre-MG is independently controlled, what are concerned in pre-MG (de)activation is the final scheduling of data Tx/Rx and measurement related to a MG, and this also depends on the status of the other MG. Within the Rel-17 delay (without any extension), UE cannot consider the new status of the other pre-MG, as shown Figure 1. This is the reason why UE would need another processing time [X]ms (T1 in the WF) to determine the data scheduling and measurement based on the new status of both pre-MGs. As to the exact value of T1, we suggest 2ms.

Figure 1: Example of fully overlapping (de)activation procedure of two pre-MGs

Proposal 2: For fully overlapped simultaneous (de)activation, T1 = 2ms.

Issue 3-2-3: [Case 1] Whether to extend the delay for partially overlapped simultaneous activation/deactivation for Pre-MG+Pre-MG < Way forward >: FFS the options: - Option 1: - RAN4 shall extend the delay for partially overlapped simultaneous Pre-MGs activation/deactivation, when multiple Pre-MGs activation/deactivation processes are overlapped in time, until the end of the last Pre-MG activation/deactivation duration + T2. Where the value of T2 is FFS. - Option 3: - For partially overlapping simultaneous activation/deactivation of two Pre-MGs due to different events, RAN4 shall discuss necessity of defining requirement. The following two options can be considered: - only clarify in spec that extra delay can be expected in high-level. - the completion of activation/deactivation of the first Pre-MG is extended to the end of completion of activation/deactivation of the second Pre-MG. - Option 4: - Since the UE capability of supporting simultaneous pre-MG multiple activation/deactivation procedure has been identified, for the UE capable of such capability, the UE is capable to perform multiple activation/deactivation procedure in parallel, so we prefer to reuse the R17 activation/deactivation switching delay.

We support option 1 for the partially overlapped case. From UE implementation point of view, there is no difference between fully or partially overlapped cases. As shown in Figure 2, during the (de)activation procedure of MG#1, the new status of MG#2 may not be considered and vice versa. From the end of the last pre-MG (de)activation duration (duration for MG#2 in Figure 2), UE would need another processing time [X]ms (T2 in the WF) to determine the data scheduling and measurement based on the new status of both pre-MGs. As to the exact value of T2, we suggest 2ms.

Figure 2: Example of partially overlapping (de)activation procedure of two pre-MGs

Proposal 3: For partially overlapped simultaneous (de)activation, the delay is extended until the end of the last pre-MG (de)activation duration + T2, where T2 = 2ms.

Collision handling

Issue 3-3-1: [Case 1] Required changes for Pre-MG on collision < Way forward >: - Option 1: RAN4 shall stick to agreed baseline that collision and priority rule on Pre-MG are considered only when Pre-MG is activated (deactivated Pre-MG is not considered in collisions). - Option 2: De-activated pre-MG is considered in collisions handling.

We support to consider deactivated Pre-MG in a collision, for the following reasons.

- It can avoid dynamic collision. When deactivated pre-MG is also considered for collision, NW and UE can well determine the drop/keep of each MG occasion based on configuration, as it will not change due to status change of a pre-MG. This will simplify both NW/UE implementation and the spec. As can be seen, many other issues (e.g. 3-2-3, 3-3-5, 3-4-2) will be complicated by dynamic collision.

- It can achieve the goal of pre-MG. The motivation of pre-MG is to retain the resource for data when MOs associated to it does not require MG. If only activated pre-MG is considered for collision, even the pre-MG is deactivated, the resource can still not be used for data as there is colliding MG not dropped.

- It can ensure the measurement performance for MOs associated to the pre-MG. it is noted that the priority is essentially configured for the MO associated to a MG, so when a high priority pre-MG is deactivated, it does not mean the associated measurements should be de-prioritized compared to those associated to the other colliding MG with low priority.

Proposal 4: Deactivated pre-MG is considered in collisions handling.

Issue 3-3-6: [Case 1] Whether to define a new UE capability for dynamic collisions? < Agreement/ Way forward >: - FFS. Wait for outcome of issue 3-3-1.

If proposal 4 is not agreeable, we support to define a new UE capability for dynamic collision.

It is noted that in RAN4#105, it was already agreed that “support of gap combinations including pre-configured MGs (Case 1) that cause dynamic collisions will be subject to new UE capability(ies)”. The FFS is the exact definition of dynamic collision, which was resolved in RAN4#106.

In last meeting some companies proposed introduce a UE capability for UE to indicate whether deactivated pre-MG is considered in collisions handling or not. This is an alternative way to define capability related to dynamic collision because if UE indicates deactivated pre-MG is considered in collision handling, there would be no dynamic collision for this UE. If UE indicates deactivated pre-MG is not considered in collision handling, it means the UE should be able to handle dynamic collision.

Proposal 5: If Proposal 4 is not agreeable, introduce a UE capability for UE to indicate whether deactivated pre-MG is considered in collisions handling or not.

Issue 3-3-2: [Case 1] When the pre-configured MG activation procedure is overlapped with one of concurrent gap occasion during the dynamic collision (i.e. Pre-MG has higher priority than the MG) < Way forward >: - FFS the following options: - Option 1: - A collision between a change in the status of a pre-configured MG and a gap instance happens when the change occurs ≤ 4 ms before the start or ≤ 4 ms after the end of a gap instance of an activated concurrent MG. (UE shall extend the activation procedure) - Option 1a: - If a change in the status of a pre-configured MG collides with a gap instance, the change in status is delayed by (MGL of the gap instance plus 5 ms) to avoid the collision. - Option 2: - For some gap patter configuration, for example when the current gap pattern is gap pattern 4 (MGL = 6ms and MGRP = 20ms), after the activation of Pre-MG, that Pre-MG occasion will collide with the next gap occasion of the concurrent gap and gaps will be dropped based on priority rule. Clarifications may need since for a particular window W there is an extra collision due to the extension of the activation procedure and the collision may not happen again for other time window W. - Option 3: - Alternatively, when the pre-configured MG activation procedure is overlapped with one of concurrent gap occasion, the Pre-MG status shall not be changed immediately and its status shall be changed prior to the next gap occasion. - Option 4: - When Pre-MG activation/deactivation period collides Type-2 gap, - during Pre-MG activation/deactivation period, - the UE is not required to receive or transmit in the corresponding NR serving cells in the Pre-MG occasions. - gap dropping rule won’t be applied. - how to use such gap occasion is up to UE implementation. - the gap dropping rule will be re-applied after the 1st effective MG occasion after Pre-MG activation. - Option 5: - If the priority of pre-MG is higher than the other MG, applying priority rule, i.e. pre-MG activation/deactivation procedure prioritizes the other MG occasion, so the other MG occasion is dropped; - then the MO(s) associated with this MG accordingly be canceled at this MG occasion. - Option 6: - If two MG occasions collide, one of the two MGs is pre-MG and the pre-MG has higher priority, then UE is allowed to keep or drop any of the two occasions if - the (de)activation procedure of pre-MG ends earlier than the start of pre-MG occasion, and - the (de)activation procedure of pre-MG overlaps with time period T, where T starts from 4ms before the other MG occasion and ends at 4ms after the other MG occasion - Option 7: - Agree on option 2 for issue 3-3-1 and close this issue.

When pre-MG is colliding with the other MG and pre-MG has higher priority as in Figure 3, additional UE behaviour may be needed.

- Case 1): Pre-MG status is changed from activated to deactivated

If the deactivation procedure (from B to A) is well before the MG occasion, UE would consider the pre-MG as deactivated and keep the MG occasion. However, in Figure 3 the completion of deactivation is too late and UE cannot start the MG occasion, i.e. occasions of both pre-MG and MG may be dropped.

- Case 2): Pre-MG status is changed from deactivated to activated

If the activation procedure (from B to A) is well before the MG occasion, UE would consider the pre-MG as activated and drop the MG occasion. However, in Figure 3 the completion of activation is too late and UE has already started the MG occasion and cannot drop it, i.e. occasion of pre-MG may be dropped although it is activated and has higher priority, while the occasion of the MG is kept although it is colliding with an occasion of pre-MG which is activated and has higher priority.

Figure 3: Example of (de)activation of pre-MG overlapping with another MG

In essence, the issue is that the keep/drop of the MG occasion depends on the status of the pre-MG, and when the (de)activation of pre-MG overlaps with another MG, it may be too late for UE to start or to drop the MG occasion, and to start the pre-MG when it should be activated. It is noted that the issue occurs when the (de)activation procedure of the pre-MG ends earlier than the start of the pre-MG occasion; otherwise the status change does not take effect for the colliding occasion, and UE could determine the keep/drop of the MG occasion based on the old status of the pre-MG.

Considering Figure 3 is a very specific case that may not happen very often in real deployment, we suggest to simply leave it to UE implementation to decide which MG occasion to keep or drop. Optimization by defining particular UE behaviour on keep/drop is not preferred.

Proposal 6: If Proposal 4 is not agreeable, define additional UE behaviour as follows.

If two MG occasions collide, one of the two MGs is pre-MG and the pre-MG has higher priority, then UE is allowed to keep or drop any of the two occasions if

- the activation procedure of pre-MG ends earlier than the start of pre-MG occasion, and

- the activation procedure of pre-MG overlaps with time period T, where T starts from 4ms before the other MG occasion and ends at 4ms after the other MG occasion.

Issue 3-3-3: [Case 1] When the pre-configured MG deactivation procedure is overlapped with one of concurrent gap occasion during the dynamic collision (i.e. Pre-MG has higher priority than the MG) < Agreement/ Way forward >: - FFS. Wait for outcome of issue 3-3-1.

Above Proposal 6 can be also used when the pre-configured MG deactivation procedure is overlapped with one of concurrent gap occasion. Since we are discussing a corner case, we suggest to use the same principle to deal with pre-MG activation and pre-MG deactivation.

Proposal 7: Re-use same rule as in Proposal 6 for the case when the pre-configured MG deactivation procedure is overlapped with one of concurrent gap occasion.

Issue 3-3-4: [Case 1] Definition for Pre-MG activation/deactivation collisions with MG < Way forward>: - Proposed definition: ‘The collision between the pre-MG activation/deactivation procedure and the other MG occasion can be defined as: the pre-MG activation/deactivation procedure is physically overlapping with the other MG occasion fully or partially in time domain’. - Proponent company of this proposal to provide further comment on the differences between this definition and option 1 in issue 3-3-2.

A definition for collision between pre-MG (de)activation and a MG instance is needed to define if additional UE behaviour needs to be defined for issue 3-3-2 and 3-3-3. We do not support the proposed definition in WF. Instead, a collision between a change in the status of a pre-MG and a MG instance happens when the change occurs ≤ 4 ms before the start or ≤ 4 ms after the end of a MG instance.

As shown in Figure 3, even the (de)activation procedure (from B to A) does not physically overlap with the MG instance, it will also cause late starting or cancelling of the MG because UE may have already decided to cancel or start the MG instance before the completion of the (de)activation procedure (time point A).

Proposal 8: A collision between a change in the status of a pre-MG and a MG instance happens when the change occurs ≤ 4 ms before the start or ≤ 4 ms after the end of a gap instance of an activated concurrent MG.

Issue 3-3-5: [Case 1] When the pre-configured MG activation procedure is overlapped with one of concurrent gap occasion where the MG has higher priority than the Pre-MG < Agreement/ Way forward >: - FFS. Wait for outcome of issue 3-3-1.

When the MG has higher priority, the colliding pre-MG will be always dropped, no matter it is considered as activated or deactivated. The existing priority rule applies without any change

Proposal 9: When the pre-MG (de)activation procedure is overlapped with one of concurrent gap occasion and the MG has higher priority, existing priority rule applies without any change.

Measurement requirements

Issue 3-4-1: [Case 1] Measurement delay requirements due to change in Pre-MG status < Way forward >: - Postpone the discussion until it is clear whether to support dynamic collision or not. - FFS the options: - Option 1: - When gap combinations including pre-configured MGs (Case 1) are provided to the UE, measurement requirements do not apply if the following parameters change during the measurement period due to changes in the status of any pre-configured MGs: - Kp for intra-frequency and inter-frequency measurements without gaps - Kgap for intra-frequency and inter-frequency measurements with gaps - Kgap_EUTRA for inter-RAT measurements - Kp_CSI-RS for CSI-RS L3 measurements - Kp,PRS,i for NR positioning measurements - CSSFintra for intra-frequency measurements - CSSFinter for intra-frequency measurements - CSSFinterRAT for intra-RAT measurements - P scaling factor for L1-RSRP and L1-SINR measurements - Option 2: - R17 measurement delay requirements for concurrent MG can be reused for case 1 without any status change. - Option 3: - RAN4 to investigate whether measurement requirements in case of changes of Pre-MG status are based on more relaxed requirements between both Pre-MG status.

When the status of pre-MG is changed, it can result at changes to measurements, including

- measurement performed with the pre-MG when it is activated

- measurement outside MG and L1 measurement, and

- measurement with the other component MG.

The first one was already addressed in Rel-17 with the following requirements.

If the Pre-MG status changes during a measurement period of a measurement that can be performed without and within measurement gaps, the UE is allowed to restart the measurement.

If the Pre-MG status changes from activated to deactivated during a measurement period of a measurement that can only be performed within measurement gaps, the measurement requirements do not apply.

For the other two, we understand that when the measurement period requirements, e.g. Kp, Kgap, CSSF or P factors, for a measurement is changed due to status change of the pre-MG, UE may re-schedule the measurement, and the measurement period requirements should not apply. In other words, UE should be allowed to restart the measurement and fulfill the measurement period requirements with new Kp, Kgap, CSSF or P factors.

Proposal 10: When the measurement period requirements for a measurement are changed due to status change of a pre-MG, the measurement period requirements should not apply, and UE is allowed to restart the measurement.

Issue 3-4-2: [Case 1] Measurement delay requirements < Way forward >: - Postpone the discussion until it is clear whether to support dynamic collision or not. - FFS the options: - Option 1: - No need to consider the delayed Pre-MG activation in measurement requirements - Option 2: - Measurement delay requirements should also consider the case of delayed Pre-MG activation due to overlap between Pre-MG and concurrent MG. - Option 2a: - RAN4 to add the Pre-MG activation delay of 5 ms to the measurement delay in case of overlapping of Pre-MG with concurrent MG.

We support option 1. The same issue has been discussed in Rel-17 pre-MG, and the conclusion was to not add pre-MG (de)activation delay in the measurement period requirements. Instead, the change of pre-MG status will impact the applicability of the requirements as in option 1 of issue 3-4-1.

Proposal 11: Pre-MG (de)activation delay is not considered in measurement requirements.

Conclusions

In this paper we provided our views on open issues in joint working of pre-MG and con-MG.

Proposal 1: Define delay requirements for simultaneous pre-MG (de)activation in following cases

- Fully overlapping:

  - triggered by single CC BWP switch or SCell (de)activation

  - triggered by simultaneous multi-CC BWP switch

- partially overlapping:

  - triggered by multiple SCell activation with same MAC CE

  - triggered by non-simultaneous timer-based multi-CC BWP switch

Proposal 2: For fully overlapped simultaneous (de)activation, T1 = 2ms.

Proposal 3: For partially overlapped simultaneous (de)activation, the delay is extended until the end of the last pre-MG (de)activation duration + T2, where T2 = 2ms.

Proposal 4: Deactivated pre-MG is considered in collisions handling.

Proposal 5: If Proposal 4 is not agreeable, introduce a UE capability for UE to indicate whether deactivated pre-MG is considered in collisions handling or not.

Proposal 6: If Proposal 4 is not agreeable, define additional UE behaviour as follows.

If two MG occasions collide, one of the two MGs is pre-MG and the pre-MG has higher priority, then UE is allowed to keep or drop any of the two occasions if

- the activation procedure of pre-MG ends earlier than the start of pre-MG occasion, and

- the activation procedure of pre-MG overlaps with time period T, where T starts from 4ms before the other MG occasion and ends at 4ms after the other MG occasion.

Proposal 7: Re-use same rule as in Proposal 6 for the case when the pre-configured MG deactivation procedure is overlapped with one of concurrent gap occasion.

Proposal 8: A collision between a change in the status of a pre-MG and a MG instance happens when the change occurs ≤ 4 ms before the start or ≤ 4 ms after the end of a gap instance of an activated concurrent MG.

Proposal 9: When the pre-MG (de)activation procedure is overlapped with one of concurrent gap occasion and the MG has higher priority, existing priority rule applies without any change.

Proposal 10: When the measurement period requirements for a measurement are changed due to status change of a pre-MG, the measurement period requirements should not apply, and UE is allowed to restart the measurement.

Proposal 11: Pre-MG (de)activation delay is not considered in measurement requirements.

Reference

- R4-2306330, WF on Rel-18 NR MG enhancements, MediaTek inc.