3GPP TSG-RAN WG4 Meeting #107	R4-2308666

Incheon, KR, 22 – 26 May, 2023

Title: 	Reply LS on measurement definitions for positioning with bandwidth aggregation

Source: 	Huawei, HiSilicon

Agenda item:	8.23.3.1

Document for:	Approval

Introduction

RRM requirements for PRS/SRS CA for positioning are discussed in RAN4#106-bis-e, and outcomes are captured in WF [1]. In addition, RAN4 received LS from RAN1 [2] related to definition of measurements.

In this paper we will provide our views on definition of measurements with PRS/SRS CA for positioning.

Discussion

Conclusion The legacy definition of DL RSTD, UL RTOA, UE Rx-Tx time difference, gNB Rx-Tx time difference is reused with the assumption that the subframe timings of the intra-band contiguous carriers are the same. - Note: multiple PRS/SRS resources which can be used to determine the start of subframe can be from multiple intra-band continuous carriers, - Note: no RAN1 spec impact - Send an LS to RAN4 to confirm RAN1’s understanding

In [2] RAN1 asks RAN4 to confirm RAN1’s understanding about definition of timing related measurement in the context of PRS/SRS CA.

In our view, there is no problem in the RAN1 assumption that the subframe timings of the intra-band contiguous carriers are the same because the PRS and SRS for CA are sent with same antenna. Since all the timing related measurements are defined based on subframe timing, there is no problem to re-use the existing definition with PRS/SRS CA.

Proposal: RAN4 to confirm that legacy definition of DL RSTD, UL RTOA, UE Rx-Tx time difference, gNB Rx-Tx time difference can be reused with PRS/SRS CA.

Conclusions

In this paper we provided our views on definition of measurements with PRS/SRS CA for positioning.

Proposal: RAN4 to confirm that legacy definition of DL RSTD, UL RTOA, UE Rx-Tx time difference, gNB Rx-Tx time difference can be reused with PRS/SRS CA.

Reference

- R4-2306349, WF on RRM requirements for RedCap positioning and PRS/SRS BW aggregation, Ericsson

- R1-2304082, LS on measurement definitions for positioning with bandwidth aggregation, RAN1

Annex: draft Reply LS on measurement definitions for positioning with bandwidth aggregation

3GPP TSG-RAN WG4 Meeting #107 R4-22xxxxx Incheon, KR, 22 – 26 May, 2023 Title: Reply LS on measurement definitions for positioning with bandwidth aggregation Response to: R1-2304082 Release: Release 18 Work Item: NR_pos_enh2 Source: RAN4 To: RAN1 Cc: Contact Person: #### Name: Li Zhang - E-mail Address: zhangli164@huawei.com Attachments: - 1. Overall Description: RAN4 thanks RAN1 for the LS R1-2304082. RAN4 would like to confirm that RAN1’s understanding correct, i.e. the legacy definition of DL RSTD, UL RTOA, UE Rx-Tx time difference, gNB Rx-Tx time difference can be reused with PRS and SRS bandwidth aggregation. RAN4 respectfully asks RAN1 to take the above information into account. 2. Actions: To RAN1: RAN4 respectfully asks RAN1 to take the above information into account. 3. Date of Next TSG-RAN4 Meetings: RAN WG4 Meeting #108 Aug 21 – Aug 25, 2023 Toulouse, France RAN WG4 Meeting #108-bis Oct 09 – Oct 13, 2023 Xiamen, China