3GPP TSG-RAN WG4 Meeting #107	R4-2308701

Incheon, KR, 22th – 26th May, 2023

Title: 	Discussion on RLM and BFD/CBD measurements for R18 FR2 multi-Rx chain DL reception

Source: 	Huawei, HiSilicon

Agenda item:	8.8.3.3

Document for:	Discussion

Introduction

In this contribution, we will provide the further discussion on RLM and BFD/CBD measurements for FR2 multi-Rx DL reception.

Discussion

Simultaneous reception for RLM and cell specific BFD/CBD

For RLM and cell specific BFD procedure, the followings are specified in TS38.133:

- 8.1 Radio Link Monitoring - 8.1.1 Introduction The requirements in clause 8.1 apply for radio link monitoring on: - PCell in SA NR, NR-DC and NE-DC operation mode, - PSCell in NR-DC and EN-DC operation mode, - Deactivated PSCell in NR-DC and EN-DC operation mode, when configured. The UE shall monitor the downlink radio link quality based on the reference signal configured as RLM-RS resource(s) in order to detect the downlink radio link quality of the PCell, PSCell and deactivated PSCell if configured with bfd-and-RLM with value true as specified in TS 38.213 [3]. The configured RLM-RS resources can be all SSBs, or all CSI-RSs, or a mix of SSBs and CSI-RSs. UE is not required to perform RLM outside the active DL BWP. On each RLM-RS resource, the UE shall estimate the downlink radio link quality and compare it to the thresholds Qout and Qin for the purpose of monitoring downlink radio link quality of the cell. … - 8.5 Link Recovery Procedures - 8.5.1 Introduction The UE shall assess the downlink radio link quality of a serving cell based on the reference signal in the set as specified in TS 38.213 [3] in order to detect beam failure on: - PCell in SA, NR-DC, or NE-DC operation mode, - PSCell in NR-DC and EN-DC operation mode, - SCell in SA, NR-DC, NE-DC or EN-DC operation mode, - Deactivated PSCell in NR-DC and EN-DC operation mode The RS resource configurations in the set on PCell, PSCell or deactivated PSCell (if configured with bfd-and-RLM with value true) can be periodic CSI-RS resources and/or SSBs. RS resource configuration in the set on SCell shall be periodic CSI-RS. UE is not required to perform beam failure detection outside the active DL BWP. UE is not required to meet the requirements in clause 8.5.2 and 8.5.3 if UE does not have set . UE is not required to perform beam failure detection on a deactivated SCell, and also not required to perform beam failure detection on resources which is implicitly configured for a deactivated SCell. When more than 2 periodic CSI-RS resources on a CC are configured in the set for current SCell or implicitly configured in the set for other SCell, it is up to UE implementation to select two of CSI-RS resources in active BWP in current CC to perform beam failure detection. UE is not required to perform beam failure detection on a SCell on which is not configured. On each RS resource configuration in the set , the UE shall estimate the radio link quality and compare it to the threshold Qout_LR for the purpose of accessing downlink radio link quality of the serving cell beams. …

It can be observed that UE perform RLM/BFD measurements on the each RLM/BFD RS resource and compare it to the threshold independently. When any one RLM-RS is no worse better than the threshold Qout, then no OOS indication is detected. When any one BFD-RS is no worse than the threshold Qout_LR, then no BFI indication is detected. For recovery, when the link quality of any one of RLM/CBD RS resource is better than the threshold Qin/Qin_LR, then the cell or beam level quality recovery are considered.

Observation 1: For RLM and cell specific BFD/CBD, UE performs downlink radio link quality estimation on each RLM/BFD/CBD RS resource independently.

Hence, the best Rx beam used for estimating the link quality of each RLM/BFD/CBD RS resource needs to be assumed from single RS perspective rather than a RS pair perspective. Then, simultaneous RLM/BFD measurements on two RS resources are not necessary for UE to monitor the serving cell/beam quality.

Proposal 1: For RLM and cell specific BFD/CBD, there is no need to consider simultaneous RLM/BFD/CBD measurements on two RS resources for monitoring the cell or link quality, and the best Rx beam for each RLM/BFD/CBD RS resource is assumed to be selected from single RS perspective.

Simultaneous reception for TRP specific BFD/CBD

For TRP specific BFD/CBD, two RS resources sets are configured for BFD and two RS resources sets are configured for CBD. Each BFD/CBD resources set can be considered to be associated to a TRP. BFD resources sets and CBD resources sets are 1-to-1 mapping. UE monitors the link quality of each TRP based on the corresponding BFD-RS resource set. When beam failure is detected for one TRP, then UE will select new beam based on the associated CBD resource sets. For TRP specific BFD/CBD, UE performs BFD/CBD measurements as two independent TRPs, not as one pair of TRPs.

Observation 2: For TRP specific BFD/CBD procedure, UE performs BFD (or CBD) measurements on two BRD (or CBD) resource sets as two independent TRPs, not as a TRP pair.

For TRP specific BFD procedure, the followings are specified in TS38.133:

- 8.18 TRP specific Link Recovery Procedures - 8.18.1 Introduction The UE shall assess the downlink radio link quality of a serving cell based on the reference signal provided in the two sets q0,0 and q0,1 as specified in TS 38.213 [3] in order to detect beam failure on: - PCell in SA, NR-DC, or NE-DC operation mode, - PSCell in NR-DC and EN-DC operation mode, - SCell in SA, NR-DC, NE-DC or EN-DC operation mode. The RS resource configurations in the two sets q0,0 and q0,1 on PCell or PSCell can be periodic CSI-RS resources and/or SSBs. RS resource configuration in the two sets q0,0 and q0,1 on SCell shall be periodic CSI-RS. UE is not required to perform beam failure detection outside the active DL BWP. UE is not required to meet the requirements in clause 8.18.2 and 8.18.3 if UE does not have configured sets q0,0 and q0,1. UE is not required to perform beam failure detection on a deactivated SCell, and also not required to perform beam failure detection on resources which is implicitly configured for a deactivated SCell. UE is not required to perform beam failure detection on a SCell on which q1,0 and q1,1 is not configured. If additionalPCIList is configured for the serving cell, the SSBs in sets q0,1 and q1,1 can be associated to the same additional PCI other than serving cell PCI. On each RS resource configuration in the two sets q0,0 and q0,1, the UE shall estimate the radio link quality and compare it to the threshold Qout_LR for the purpose of accessing downlink radio link quality of the serving cell. …

So, the best Rx beam for BFD/CBD-RS is still selected from single RS perspective. Simultaneous receptions on two BFD/CBD RSs from different resource sets are not necessary for TRP specific BFD/CBD.

Observation 3: For TRP specific BFD/CBD, UE performs downlink radio link quality estimation on each BFD/CBD RS resource independently.

Proposal 2: For TRP specific BFD/CBD, there is no need to consider simultaneous BFD/CBD measurements on two RS resources for monitoring the beam level link quality, and the best Rx beam for each BFD/CBD RS resource is still assumed to be selected from single RS perspective.

Beam sweeping for RLM and BFD/CBD

For RLM and BFD measurement, the beam sweeping factor reduction is defined as N=8 for SSB resource and N=1 for CSI-RS resource. For CBD measurements, the beam sweeping factor reduction is defined as N=8 for both SSB resource and CSI-RS resource. So, there is no need to consider beam sweeping factor reduction for CSI-RS based RLM/BFD. For SSB resource, there is no TCI configuration and UE needs to perform beam sweeping for searching best Rx beam. For CSI-RS based on CBD measurements, beam sweeping operation is needed due to that beam information from source RS is not reliable when beam failure is detected.

Proposal 3: There is no need to consider faster beam sweeping for CSI-RS based RLM and BFD measurements since no beam sweeping operation is assumed.

In RAN4, it was agreed to consider faster beam sweeping with multi-Rx chains based UE capabilities, and beam sweeping factor reduction is one of the options. The beam sweeping factor reduction for RLM/BFD/CBD measurements could lead to the direction sweeping range reduction. If beam sweeping factor was reduced for faster beam sweeping, then the beam direction coverage would also be impacted. Compared with legacy UE, the reduction of direction sweeping range for beam sweeping operation may lead that UE could not select to the best Rx beam and cause the performance degradation.

Figure 1: Example of legacy beam sweeping operation

Figure 2: Example of beam sweeping with beam sweeping factor reduction

Figure 1 and Figure 2 provide the example of beam sweeping operation without and with beam sweeping factor reduction. When the UE is rotated with 90 degrees, the UE is still able to select the best Rx beam based on legacy beam sweeping operation but not based on the beam sweeping operation with reduced beam sweeping factor. Hence, beam sweeping factor reduction will degrade the robustness of the UE against beam variation caused by UE rotation.

Observation 4: For SSB based RLM/BFD/CBD and CSI-RS based CBD, beam sweeping factor reduction for faster beam sweeping will lead to reduce the beam sweeping coverage compared with legacy UE, which will degrade the robustness of the UE against beam variation caused by UE rotation.

Proposal 4: For SSB based RLM/BFD/CBD and CSI-RS based CBD, RAN4 needs to investigate the performance degradation due to beam sweeping factor reduction for faster beam sweeping.

Conclusions

In this paper we provided our discussion on RLM and BFD/CBD measurements for FR2 multi-Rx DL reception. The followings are provided.

Observation 1: For RLM and cell specific BFD/CBD, UE performs downlink radio link quality estimation on each RLM/BFD/CBD RS resource independently.

Proposal 1: For RLM and cell specific BFD/CBD, there is no need to consider simultaneous RLM/BFD/CBD measurements on two RS resources for monitoring the cell or link quality, and the best Rx beam for each RLM/BFD/CBD RS resource is assumed to be selected from single RS perspective.

Observation 2: For TRP specific BFD/CBD procedure, UE performs BFD (or CBD) measurements on two BRD (or CBD) resource sets as two independent TRPs, not as a TRP pair.

Observation 3: For TRP specific BFD/CBD, UE performs downlink radio link quality estimation on each BFD/CBD RS resource independently.

Proposal 2: For TRP specific BFD/CBD, there is no need to consider simultaneous BFD/CBD measurements on two RS resources for monitoring the beam level link quality, and the best Rx beam for each BFD/CBD RS resource is still assumed to be selected from single RS perspective.

Proposal 3: There is no need to consider faster beam sweeping for CSI-RS based RLM and BFD measurements since no beam sweeping operation is assumed.

Observation 4: For SSB based RLM/BFD/CBD and CSI-RS based CBD, beam sweeping factor reduction for faster beam sweeping will lead to reduce the beam sweeping coverage compared with legacy UE, which will degrade the robustness of the UE against beam variation caused by UE rotation.

Proposal 4: For SSB based RLM/BFD/CBD and CSI-RS based CBD, RAN4 needs to investigate the performance degradation due to beam sweeping factor reduction for faster beam sweeping.

Reference

- R4-2306318, “WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 1)”, vivo