3GPP TSG-RAN WG4 Meeting #107	R4-2308702

Incheon, KR, 22th – 26th May, 2023

Title: 	Discussion on scheduling/measurement restrictions for R18 FR2 multi-Rx chain DL reception

Source: 	Huawei, HiSilicon

Agenda item:	8.8.3.4

Document for:	Discussion

Introduction

In this contribution, we will provide the discussion on scheduling restrictions and measurement restrictions for FR2 multi-Rx DL reception.

Discussion

Scheduling Restrictions

The scheduling restrictions requirements in TS 38.133 specifies the conditions when data receptions and L1/L3 measurements cannot be performed simultaneously.

For data receptions, the beam information is derived based on the active TCI state and non-beam sweeping operation is assumed for data receptions. However, UE is assumed to perform L3 measurements with beam sweeping operation. If simultaneous of data reception and L3 measurement is supported, which requires that UE activates one antenna panel for data reception and activates another antenna panel for L3 measurement, which cannot be guaranteed by UE. Besides, rough beam is assumed for L3 measurements and fine beam is assumed for data receptions. It is impossible to pair a rough beam and a fine beam for simultaneous receptions. Hence, the scheduling restrictions due to L3 measurements still need to be applied for FR2 multi-Rx DL receptions.

Proposal 1: For R18 multi-Rx DL receptions, it is suggested not to consider simultaneous data reception and L3 measurement, and the existing scheduling restrictions requirements due to L3 measurements still need to be applied.

For L1 measurements, the existing scheduling restrictions requirements define that UE is not able to perform data receptions when RS for L1 measurement is not QCL with the TCI state of data.

For SSB or CSI-RS with repetition on, beam sweeping operation is assumed and UE needs to try multiple beam directions on this RS resource. However, there is no beam sweeping operation for data receptions. Simultaneous of data reception and L1 RS with beam sweeping operation is not considered. The scheduling restrictions due to L1 measurements on SSB or CSI-RS with repetition on still need to be applied.

Proposal 2: For R18 multi-Rx DL receptions, the scheduling restrictions on SSB for L1 measurements or and on CSI-RS with repetition=on still need to be applied.

For FR2 multi-Rx DL reception, the two activated TCI states used for data receptions from two TRPs are based on the results of group-based L1-RSRP beam reporting. The UE will be configured with multiple RSs for L1 measurements.

Figure 1: Example of RSs configured L1 measurements

For example, as shown in Figure 1, the data receptions from two TRPs are based on the reported beam pair of CSI-RS1+CSI-RS2. The beam pair used for simultaneous data receptions from two TRPs is derived from the measured best beam pair of CSI-RS1+CSI-RS2, which means that CSI-RS1 or CSI-RS2 is QCLed with one of the activated TCI states for data receptions, UE can be assumed to perform simultaneous data reception and L1 measurement on CSI-RS1/CSI-RS2. For other CSI-RS resources, UE cannot perform simultaneous data reception and L1 measurement.

When only one TCI state is activated for data receptions and the best Rx beam for data receptions is derived from non-group based L1-RSRP reporting, there is no necessary to consider simultaneous data reception and L1 measurement. When one TCI state is activated for data receptions and the Rx beam for data receptions is derived from group-based L1-RSRP reporting, then simultaneous data reception and CSI-RS based L1 measurements can be considered provided that the CSI-RS and the QCL source of data have been reporting as beam pair and UE is under multi-Rx ON state.

Proposal 3: For R18 multi-Rx DL receptions, the scheduling restriction relaxation for L1 measurements can be considered under the following conditions:

- Conditions for case 1: single PDCCH/PDSCH is transmitted

  - CSI-RS and QCL source of PDCCH/PDSCH have been reported via GBBR in a pair.

  - UE is in multi-Rx ON state (e.g. UE indicates or is activated for multi-panel receptions) .

  - UE is using the beam in the reported beam pair for CSI-RS and PDCCH/PDSCH (e.g. only GBBR is configured and non-GBBR is not configured).

- Conditions for case 2: dual PDCCH/PDSCH are transmitted

  - CSI-RS has the same QCL source of any one of the PDCCH/PDSCH.

  - CSI-RS is not in a resource set with repetition.

Measurement Restrictions

The measurement restrictions requirements in TS 38.133 specifies the conditions when two RSs configured for L1 measurements cannot be measured simultaneously.

When SSB is used for L1 measurements, beam sweeping operation is assumed on the SSB resource for beam training. UE cannot guarantee the simultaneous L1 measurements on an SSB resource and another RS due to the dynamic beam change on the SSB resource. When CSI-RS with repetition on is used for L1 measurements, beam sweeping operation is also assumed for beam training. Similarly, simultaneous L1 measurements on an CSI-RS resource repetition on and another RS are not considered.

RAN4 agreed that group-based reporting is considered for enabling simultaneous DL receptions with two different beams. So, simultaneous data receptions can be enabled by activating two TCI states according to the results of group-based L1-RSRP reporting. When two RS resources without beam sweeping for L1 measurements has been reported as a beam pair and UE is also using beam in the reported beam pair for these RS receptions, then UE can be assumed to perform simultaneous L1 measurements on the two RS resources. Besides, UE also need to be indicated or activated for multi-Rx DL receptions, otherwise there is no need to consider simultaneous L1 measurements.

Proposal 4: For R18 multi-Rx DL receptions, the measurements restriction relaxation for L1 measurements can be considered under the following conditions:

- UE is in multi-Rx ON state (e.g. UE indicates or is activated for multi-panel receptions)

- Both CSI-RSs are not included in a resource set with repetition ON.

- UE is using the beam in the reported beam pair for the two CSI-RSs (e.g. only GBBR is configured and non-GBBR is not configured).

Conclusions

In this paper we provided our discussion on scheduling/measurement restrictions for FR2 multi-Rx DL reception. The followings are provided.

Proposal 1: For R18 multi-Rx DL receptions, it is suggested not to consider simultaneous data reception and L3 measurement, and the existing scheduling restrictions requirements due to L3 measurements still need to be applied.

Proposal 2: For R18 multi-Rx DL receptions, the scheduling restrictions on SSB for L1 measurements or and on CSI-RS with repetition=on still need to be applied.

Proposal 3: For R18 multi-Rx DL receptions, the scheduling restriction relaxation for L1 measurements can be considered under the following conditions:

- Conditions for case 1: single PDCCH/PDSCH is transmitted

  - CSI-RS and QCL source of PDCCH/PDSCH have been reported via GBBR in a pair.

  - UE is in multi-Rx ON state (e.g. UE indicates or is activated for multi-panel receptions) .

  - UE is using the beam in the reported beam pair for CSI-RS and PDCCH/PDSCH (e.g. only GBBR is configured and non-GBBR is not configured).

- Conditions for case 2: dual PDCCH/PDSCH are transmitted

  - CSI-RS has the same QCL source of any one of the PDCCH/PDSCH.

  - CSI-RS is not in a resource set with repetition.

Proposal 4: For R18 multi-Rx DL receptions, the measurements restriction relaxation for L1 measurements can be considered under the following conditions:

- UE is in multi-Rx ON state (e.g. UE indicates or is activated for multi-panel receptions)

- Both CSI-RSs are not included in a resource set with repetition ON.

- UE is using the beam in the reported beam pair for the two CSI-RSs (e.g. only GBBR is configured and non-GBBR is not configured).

Reference

- R4-2306318, “WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 1)”, vivo