3GPP TSG-RAN WG4 Meeting # 107	R4-2308742
Incheon, KR, May 22 – May 26, 2023

Agenda item:	8.17.3

Source:	CAICT

Title:	On FR1 MIMO OTA test with hand phantom

Document for:	Approval

1	Introduction

In recent RAN4 meetings, the topic on FR1 MIMO OTA testing for smartphone with hand phantom was discussed, it is recommended to further discuss and focus more on the necessity of FR1 MIMO OTA test with hand phantom as captured in the WF [1].

Issue 1-3-2: Necessity of FR1 MIMO OTA test with hand phantom <Agreement>: - FFS the necessity of FR1 MIMO OTA test with hand phantom at next meetings. - Focus more on the necessity than the feasibility of FR1 MIMO OTA test with hand phantom at the next meeting.

In this paper, we further collect and analyse measurement results, to demonstrate the necessity of FR1 MIMO OTA test methodology enhancement for smartphones with hand phantoms.

2	Discussion

At previous RAN4 meetings, we presented and analysed measurement results of 12 commercial smartphones at band n78 to demonstrate the impact of hand phantom on MIMO OTA performance [2, 3], with the following observations:

Observations in our contribution at RAN4 #106 [2]: Observation 1: It is observed from the measurement results that the hand phantoms will affect the MIMO OTA performance of smartphones, and the effects on different smartphones are diverse. More than half smartphones suffer MIMO OTA performance degradation due to the presence of hand phantoms, but 30%~40% smartphones show similar or even better performance in hand phantoms. The performance gap between FS and hand phantom scenarios can reach 2.8dB. Observation 2: The effects of HL and HR phantoms are different. Among the tested 12 smartphones, most (66.7%) show better MIMO OTA performance in HL. The performance gap between HL and HR scenarios can reach 1.6dB. Observation 3: The measured sensitivity values are different in different DUT positions with hand phantom. Observations in our contribution at RAN4 #106-bis-e [3]: Observation 1: It can be observed from the measurement results that MIMO OTA performance in FS and TRS in hand phantom cannot reflect MIMO OTA performance for smartphones in browsing mode. Observation 2: It can be observed from the measurement results that the trends of MIMO OTA performance with DMP and DMSU positions are similar for smartphones in browsing mode.

To further investigate and provide more insights, we further collected measurement results at different bands. The total test cases are summarized in Table 1. New test cases compared to our last contribution [2] are highlighted in bule.

Table 1. Test cases for data collection in this contribution

| Operating mode |  | NR standalone (SA) |  |  |
| --- | --- | --- | --- | --- |
| Scenario |  | Free space (FS); Hand left (HL); Hand right (HR) |  |  |
| Hand phantom |  | Wide Grip Hand |  |  |
| Frequency: 3550MHz (band n78) | DUT | 12 commercial smartphones with different models from 7 venders (UE 1~12) |  |  |
|  | MIMO OTA test | Channel model | FR1 CDL-C Uma |  |
|  |  | MIMO layer | 4x4 |  |
|  |  | DUT position | - FS scenario: Data mode portrait (DMP); Data mode landscape (DML); Data mode screen up (DMSU) - Hand phantom scenario: HL_DMP; HL_DMSU; HR_DMP; HR_DMSU |  |
|  | SISO OTA test | TRS in FS, HL, HR |  |  |
| Frequency: 2593MHz (band n41) | DUT | 6 commercial smartphones from 6 venders (UE 1,3,4,5,6,12) |  |
|  | MIMO OTA test | Channel model | FR1 CDL-C Uma |
|  |  | MIMO layer | 4x4 |
|  |  | DUT position | - FS scenario: DMP; DML; DMSU - Hand phantom scenario: HL_DMP; HL_DMSU; HR_DMP; HR_DMSU |
|  | SISO OTA test | TRS in FS, HL, HR |  |
| Frequency: 2140MHz (band n1) | DUT | 6 commercial smartphones from 6 venders (UE 1,3,4,5,6,12) |  |
|  | MIMO OTA test | Channel model | FR1 CDL-C Umi |
|  |  | MIMO layer | 2x2 |
|  |  | DUT position | - FS scenario: DMP; DML; DMSU - Hand phantom scenario: HL_DMP; HL_DMSU; HR_DMP; HR_DMSU |
|  | SISO OTA test | TRS in FS, HL, HR |  |

Fig. 1 illustrates DUT positions with right hand phantom in the measurements. The measurements were performed inside CAICT’s 16-probe MIMO OTA chamber with a range length of 3.2 m that is relatively larger than the minimum range length of 1.2m. The QZ of the chamber is large enough to accommodate the DUT with hand phantoms.

(a)                                                                      (b)

Fig. 1.  DUT positions with right hand phantom. (a) HR_DMP, (b) HR_DMSU. (Note: the photos are for illustration only, the UE in the photos is not the DUT)

(1) Measurement results at band n78

The MIMO OTA and TRS measurement results of the 12 smartphones in different scenarios at band n78 are presented in Fig. 2.  In Fig. 2,

TRS_hand=10log2/(110PHL10+110PHR10)

MIMO_hand=10log4110SHL_DMP,7010+110SHR_DMP,7010+110SHL_DMSU,7010+110SHR_DMSU,7010

Fig. 2. MIMO OTA and TRS measurement results of 12 UEs in different scenarios at Band n78.

Fig. 3 presents the comparison of TRMS, TRS_hand, and MIMO_hand of the 12 UEs. For each performance metric, the measurement results of different UEs are normalized to the UE with the best performance, e.g.,

TRS_hand_nol for UEx = TRS_hand_UE(best)-TRS_hand_UEx

In Fig. 3, the UEs are ranked by TRMS. UE rankings by TRMS, TRS_hand, and MIMO_hand are listed in Table 2. It can be observed that although the MIMO OTA performance of some phones has a similar trend with TRS, the overall ranking of MIMO OTA is different from TRS. That is, the DUT with better TRS may not show better MIMO OTA performance, and vice versa. For example, UE8 has very good MIMO OTA performance (ranked 1/2nd, 8%/17% percentile) but its TRS performance ranks 8th (67% percentile), which is much lower than the MIMO ranking. Comparing the rankings of TRMS and MIMO_hand, they are also not totally aligned. Thus, TRS with hand phantoms and MIMO OTA performance in free space cannot reflect MIMO OTA performance of smartphones in browsing mode.

Observation 1: At band n78, the rankings of 12 UEs by TRMS, TRS_hand, and MIMO_hand are not totally aligned.

Fig. 3. Comparison of TRMS, TRS_hand, and MIMO_hand of the 12 UEs at Band n78.

Table 2. UE rankings by TRMS, TRS_hand, and MIMO_hand at Band n78

| Range | Ranked by TRMS | Ranked by TRS_hand | Ranked by MIMO_hand |
| --- | --- | --- | --- |
| Top 25% | UE8 | UE1 | UE5 |
|  | UE5 | UE4 | UE8 |
|  | UE4 | UE10 | UE4 |
| Middle | UE10 | UE5 | UE10 |
|  | UE12 | UE11 | UE1 |
|  | UE1 | UE7 | UE11 |
|  | UE11 | UE12 | UE7 |
|  | UE3 | UE3 | UE12 |
|  | UE7 | UE8 | UE3 |
| Bottom 25% | UE9 | UE6 | UE2 |
|  | UE2 | UE9 | UE9 |
|  | UE6 | UE2 | UE6 |

(2) Measurement results at band n41

The MIMO OTA and TRS measurement results of the 6 smartphones in different scenarios at band n41 are presented in Fig. 4.  Fig. 5 presents the comparison of TRMS, TRS_hand, and MIMO_hand of the 6 UEs. UE rankings by TRMS, TRS_hand, and MIMO_hand are listed in Table 3.

Different with band n78, it can be seen that the UE rankings by TRMS, TRS_hand, and MIMO_hand are almost the same.

Observation 2: At band n41, the rankings of 6 UEs by TRMS, TRS_hand, and MIMO_hand are almost the same.

Fig. 4. MIMO OTA and TRS measurement results of 6 UEs in different scenarios at Band n41.

Fig. 5. Comparison of TRMS, TRS_hand, and MIMO_hand of the 6 UEs at Band n41.

Table 3. UE rankings by TRMS, TRS_hand, and MIMO_hand at Band n41

| Range | Ranked by TRMS | Ranked by TRS_hand | Ranked by MIMO_hand |
| --- | --- | --- | --- |
| Top 33% | UE4 | UE1 | UE1 |
|  | UE1 | UE4 | UE4 |
| Middle | UE5 | UE5 | UE5 |
|  | UE12 | UE12 | UE12 |
| Bottom 33% | UE3 | UE3 | UE3 |
|  | UE6 | UE6 | UE6 |

(3) Measurement results at band n1

Similarly, Fig. 6 presents the comparison of TRMS, TRS_hand, and MIMO_hand of the 6 UEs in different scenarios at band n1. UE rankings by TRMS, TRS_hand, and MIMO_hand are listed in Table 2.

It can be observed that the UE rankings by TRMS and MIMO_hand are the same, while the ranking by TRS_hand is different.

Observation 3: At band n1, the rankings of 6 UEs by TRMS and MIMO_hand are the same, but are different from the ranking by TRS_hand.

Fig. 6. MIMO OTA and TRS measurement results of 6 UEs in different scenarios at Band n1.

Table 4. UE rankings by TRMS, TRS_hand, and MIMO_hand at Band n41

| Range | Ranked by TRMS | Ranked by TRS_hand | Ranked by MIMO_hand |
| --- | --- | --- | --- |
| Top 33% | UE4 | UE1 | UE4 |
|  | UE5 | UE4 | UE5 |
| Middle | UE1 | UE5 | UE1 |
|  | UE12 | UE3 | UE12 |
| Bottom 33% | UE3 | UE6 | UE3 |
|  | UE6 | UE12 | UE6 |

Based on the above observations, it can be concluded that MIMO OTA performance with hand phantoms cannot be reflected by MIMO OTA performance in free space and TRS with hand phantoms. Thus, it is necessary to enhance the FR1 MIMO OTA test methodology with hand phantom to evaluate the real performance of smartphone in browsing mode.

Observation 4: Based on Observations 1~3, it can be concluded that MIMO OTA performance in browsing mode cannot be reflected by MIMO OTA performance in free space and TRS with hand phantoms.

Proposal 1: It is necessary to enhance the FR1 MIMO OTA test methodology for smartphone with hand phantoms.

3	Conclusion

In this paper, we further collect and analyse measurement results, to demonstrate the necessity of FR1 MIMO OTA test methodology enhancement for smartphones with hand phantoms.

Observation 1: At band n78, the rankings of 12 UEs by TRMS, TRS_hand, and MIMO_hand are not totally aligned.

Observation 2: At band n41, the rankings of 6 UEs by TRMS, TRS_hand, and MIMO_hand are almost the same.

Observation 3: At band n1, the rankings of 6 UEs by TRMS and MIMO_hand are the same, but are different from the ranking by TRS_hand.

Observation 4: Based on Observations 1~3, it can be concluded that MIMO OTA performance in browsing mode cannot be reflected by MIMO OTA performance in free space and TRS with hand phantoms.

Proposal 1: It is necessary to enhance the FR1 MIMO OTA test methodology for smartphone with hand phantoms.

References

- R4-2305910, “WF on Rel-18 MIMO OTA”, CAICT, 3GPP RAN4#106-bis-e, Apr. 2023.

- R4-2301047, “On FR1 MIMO OTA test methodology enhancement for smartphone with hand phantom”, CAICT, SAICT, 3GPP RAN4#106, Feb. 2023.

- R4-2305122, “Further views on FR1 MIMO OTA test for smartphone with hand phantom”, CAICT, SACIT, 3GPP RAN4#106-bis-e, Apr. 2023.