3GPP TSG-RAN WG4 Meeting # 107 	    R4-2308796

Incheon, South Korea, 22nd – 26th May 2023

Agenda Item:	8.22.1

Source: 	Ericsson

Title:	Update on terminologies for AI/ML

Document for:	Approval

# Introduction

RAN4#106bis-e agreed on the following:

Agreement:

- Terminology as given in R4-2305779 is agreed (included in the Annex) together with the following principles:

  - If needed, the description of terminologies in Table 1 can be updated. The changes are then liaised to RAN1 through an LS.

  - If needed, new terminology with an appropriate description can be added to Table 1. The changes are then liaised to RAN1 through an LS.

  - If RAN1 agrees on new terminology not listed in Table 1, then RAN4 also updates the list of terminologies in Table 1 with the RAN1 agreed description.

# Terminologies

Based on RAN1#112bis-e agreement terminology table agreed in RAN4#106bis-e needs to be updated. In the table below proposed update is marked with changes.

Table 1. Terminologies agreed in RAN1 and to be used for RAN4 discussion on use of AI/ML for CSI feedback enhancements, beam management, and positioning accuracy enhancements.

| Terminology | Description |
| --- | --- |
| AI/ML Model | A data driven algorithm that applies AI/ML techniques to generate a set of outputs based on a set of inputs. |
| AI/ML model delivery | A generic term referring to delivery of an AI/ML model from one entity to another entity in any manner. Note: An entity could mean a network node/function (e.g., gNB, LMF, etc.), UE, proprietary server, etc. |
| AI/ML model Inference | A process of using a trained AI/ML model to produce a set of outputs based on a set of inputs |
| AI/ML model parameter update | Process of updating the model parameters of a model. |
| AI/ML model testing | A subprocess of training, to evaluate the performance of a final AI/ML model using a dataset different from one used for model training and validation. Differently from AI/ML model validation, testing does not assume subsequent tuning of the model. |
| AI/ML model training | A process to train an AI/ML Model [by learning the input/output relationship] in a data driven manner and obtain the trained AI/ML Model for inference |
| AI/ML model transfer | Delivery of an AI/ML model over the air interface in a manner that is not transparent to 3GPP signalling, either parameters of a model structure known at the receiving end or a new model with parameters. Delivery may contain a full model or a partial model. |
| AI/ML model update | Process of updating the model parameters and/or model structure of a model. |
| AI/ML model validation | A subprocess of training, to evaluate the quality of an AI/ML model using a dataset different from one used for model training, that helps selecting model parameters that generalize beyond the dataset used for model training. |
| Data collection | A process of collecting data by the network nodes, management entity, or UE for the purpose of AI/ML model training, data analytics and inference. |
| Federated learning / federated training | A machine learning technique that trains an AI/ML model across multiple decentralized edge nodes (e.g., UEs, gNBs) each performing local model training using local data samples. The technique requires multiple interactions of the model, but no exchange of local data samples. |
| Functionality identification | A process/method of identifying an AI/ML functionality for the common understanding between the NW and the UE. Note: Information regarding the AI/ML functionality may be shared during functionality identification, where AI/ML functionality resides depends on the specific use cases and sub use cases. |
| Lifecycle management (LCM) | LCM refers to the process of developing, deploying and maintaining AI models. |
| Model activation | Enable an AI/ML model for a specific function. |
| Model deactivation | Disable an AI/ML model for a specific function. |
| Model download | Model transfer from the network to UE. |
| Model generalization | Using one model that is generalizable to different scenarios/configurations/sites. |
| Model identification | A process/method of identifying an AI/ML model for the common understanding between the NW and the UE. Note: The process/method of model identification may or may not be applicable. Note: Information regarding the AI/ML model may be shared during model identification. |
| Model monitoring | A procedure that monitors the inference performance of the AI/ML model. |
| Model selection | The process of selecting an AI/ML model for activation among multiple models for the same AI/ML enabled feature. Note: Model selection may or may not be carried out simultaneously with model activation. |
| Model switching | Deactivating a currently active AI/ML model and activating a different AI/ML model for a specific function. |
| Model upload | Model transfer from UE to the network. |
| Network-side (AI/ML) model | An AI/ML Model whose inference is performed entirely at the network. |
| Offline field data | The data collected from field and used for offline training of the AI/ML model. |
| Offline training | An AI/ML training process where the model is trained based on collected dataset, and where the trained model is later used or delivered for inference. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as offline training by commonly accepted conventions. |
| One-sided (AI/ML) model | A UE-side (AI/ML) model or a Network-side (AI/ML) model. |
| Online field data | The data collected from field and used for online training of the AI/ML model. |
| Online training | An AI/ML training process where the model being used for inference is (typically continuously) trained in (near) real-time with the arrival of new training samples. Note: the notion of (near) real-time vs. non-real-time is context-dependent and is relative to the inference time-scale. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as online training by commonly accepted conventions. Note: Fine-tuning/re-training may be done via online or offline training. (This note could be removed when we define the term fine-tuning.) |
| Open-format models | ML models of specified format that are mutually recognizable across vendors and allow interoperability, from 3GPP perspective. |
| Proprietary-format models | ML models of vendor-/device-specific proprietary format, from 3GPP perspective NOTE: An example is a device-specific binary executable format |
| Reinforcement Learning (RL) | A process of training an AI/ML model from input (a.k.a. state) and a feedback signal (a.k.a. reward) resulting from the model’s output (a.k.a. action) in an environment the model is interacting with. |
| Semi-supervised learning | A process of training a model with a mix of labelled data and unlabelled data. |
| Supervised learning | A process of training a model from input and its corresponding labels. |
| Two-sided (AI/ML) model | A paired AI/ML Model(s) over which joint inference is performed, where joint inference comprises AI/ML Inference whose inference is performed jointly across the UE and the network, i.e., the first part of inference is firstly performed by UE and then the remaining part is performed by gNB, or vice versa. |
| UE AI/ML capability | UE capability to support AI/ML models for CSI enhancements, beam management, and positioning accuracy enhancements. |
| UE-side (AI/ML) model | An AI/ML Model whose inference is performed entirely at the UE. |
| Unsupervised learning | A process of training a model without labelled data. |

Proposal 1: Terminology and description table in R4-2305779 is updated as in Table 1.

Table 1. Terminologies agreed in RAN1 and to be used for RAN4 discussion on use of AI/ML for CSI feedback enhancements, beam management, and positioning accuracy enhancements.

| Terminology | Description |
| --- | --- |
| AI/ML Model | A data driven algorithm that applies AI/ML techniques to generate a set of outputs based on a set of inputs. |
| AI/ML model delivery | A generic term referring to delivery of an AI/ML model from one entity to another entity in any manner. Note: An entity could mean a network node/function (e.g., gNB, LMF, etc.), UE, proprietary server, etc. |
| AI/ML model Inference | A process of using a trained AI/ML model to produce a set of outputs based on a set of inputs |
| AI/ML model parameter update | Process of updating the model parameters of a model. |
| AI/ML model testing | A subprocess of training, to evaluate the performance of a final AI/ML model using a dataset different from one used for model training and validation. Differently from AI/ML model validation, testing does not assume subsequent tuning of the model. |
| AI/ML model training | A process to train an AI/ML Model [by learning the input/output relationship] in a data driven manner and obtain the trained AI/ML Model for inference |
| AI/ML model transfer | Delivery of an AI/ML model over the air interface in a manner that is not transparent to 3GPP signalling, either parameters of a model structure known at the receiving end or a new model with parameters. Delivery may contain a full model or a partial model. |
| AI/ML model update | Process of updating the model parameters and/or model structure of a model. |
| AI/ML model validation | A subprocess of training, to evaluate the quality of an AI/ML model using a dataset different from one used for model training, that helps selecting model parameters that generalize beyond the dataset used for model training. |
| Data collection | A process of collecting data by the network nodes, management entity, or UE for the purpose of AI/ML model training, data analytics and inference. |
| Federated learning / federated training | A machine learning technique that trains an AI/ML model across multiple decentralized edge nodes (e.g., UEs, gNBs) each performing local model training using local data samples. The technique requires multiple interactions of the model, but no exchange of local data samples. |
| Lifecycle management (LCM) | LCM refers to the process of developing, deploying and maintaining AI models. |
| Model activation | Enable an AI/ML model for a specific function. |
| Model deactivation | Disable an AI/ML model for a specific function. |
| Model download | Model transfer from the network to UE. |
| Model generalization | Using one model that is generalizable to different scenarios/configurations/sites. |
| Model identification | A process/method of identifying an AI/ML model for the common understanding between the NW and the UE. Note: The process/method of model identification may or may not be applicable. Note: Information regarding the AI/ML model may be shared during model identification. |
| Model monitoring | A procedure that monitors the inference performance of the AI/ML model. |
| Model selection | The process of selecting an AI/ML model for activation among multiple models for the same AI/ML enabled feature. Note: Model selection may or may not be carried out simultaneously with model activation. |
| Model switching | Deactivating a currently active AI/ML model and activating a different AI/ML model for a specific function. |
| Model upload | Model transfer from UE to the network. |
| Network-side (AI/ML) model | An AI/ML Model whose inference is performed entirely at the network. |
| Offline field data | The data collected from field and used for offline training of the AI/ML model. |
| Offline training | An AI/ML training process where the model is trained based on collected dataset, and where the trained model is later used or delivered for inference. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as offline training by commonly accepted conventions. |
| One-sided (AI/ML) model | A UE-side (AI/ML) model or a Network-side (AI/ML) model. |
| Online field data | The data collected from field and used for online training of the AI/ML model. |
| Online training | An AI/ML training process where the model being used for inference is (typically continuously) trained in (near) real-time with the arrival of new training samples. Note: the notion of (near) real-time vs. non-real-time is context-dependent and is relative to the inference time-scale. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as online training by commonly accepted conventions. Note: Fine-tuning/re-training may be done via online or offline training. (This note could be removed when we define the term fine-tuning.) |
| Open-format models | ML models of specified format that are mutually recognizable across vendors and allow interoperability, from 3GPP perspective. |
| Proprietary-format models | ML models of vendor-/device-specific proprietary format, from 3GPP perspective NOTE: An example is a device-specific binary executable format |
| Reinforcement Learning (RL) | A process of training an AI/ML model from input (a.k.a. state) and a feedback signal (a.k.a. reward) resulting from the model’s output (a.k.a. action) in an environment the model is interacting with. |
| Semi-supervised learning | A process of training a model with a mix of labelled data and unlabelled data. |
| Supervised learning | A process of training a model from input and its corresponding labels. |
| Two-sided (AI/ML) model | A paired AI/ML Model(s) over which joint inference is performed, where joint inference comprises AI/ML Inference whose inference is performed jointly across the UE and the network, i.e., the first part of inference is firstly performed by UE and then the remaining part is performed by gNB, or vice versa. |
| UE AI/ML capability | UE capability to support AI/ML models for CSI enhancements, beam management, and positioning accuracy enhancements. |
| UE-side (AI/ML) model | An AI/ML Model whose inference is performed entirely at the UE. |
| Unsupervised learning | A process of training a model without labelled data. |

# Summary

In this contribution we proposed to update terminology and description table in R4-2305779 based on recent RAN1 agreement.

Proposal 1: Terminology and description table in R4-2305779 is updated as in Table 1.

Table 1. Terminologies agreed in RAN1 and to be used for RAN4 discussion on use of AI/ML for CSI feedback enhancements, beam management, and positioning accuracy enhancements.

| Terminology | Description |
| --- | --- |
| AI/ML Model | A data driven algorithm that applies AI/ML techniques to generate a set of outputs based on a set of inputs. |
| AI/ML model delivery | A generic term referring to delivery of an AI/ML model from one entity to another entity in any manner. Note: An entity could mean a network node/function (e.g., gNB, LMF, etc.), UE, proprietary server, etc. |
| AI/ML model Inference | A process of using a trained AI/ML model to produce a set of outputs based on a set of inputs |
| AI/ML model parameter update | Process of updating the model parameters of a model. |
| AI/ML model testing | A subprocess of training, to evaluate the performance of a final AI/ML model using a dataset different from one used for model training and validation. Differently from AI/ML model validation, testing does not assume subsequent tuning of the model. |
| AI/ML model training | A process to train an AI/ML Model [by learning the input/output relationship] in a data driven manner and obtain the trained AI/ML Model for inference |
| AI/ML model transfer | Delivery of an AI/ML model over the air interface in a manner that is not transparent to 3GPP signalling, either parameters of a model structure known at the receiving end or a new model with parameters. Delivery may contain a full model or a partial model. |
| AI/ML model update | Process of updating the model parameters and/or model structure of a model. |
| AI/ML model validation | A subprocess of training, to evaluate the quality of an AI/ML model using a dataset different from one used for model training, that helps selecting model parameters that generalize beyond the dataset used for model training. |
| Data collection | A process of collecting data by the network nodes, management entity, or UE for the purpose of AI/ML model training, data analytics and inference. |
| Federated learning / federated training | A machine learning technique that trains an AI/ML model across multiple decentralized edge nodes (e.g., UEs, gNBs) each performing local model training using local data samples. The technique requires multiple interactions of the model, but no exchange of local data samples. |
| Lifecycle management (LCM) | LCM refers to the process of developing, deploying and maintaining AI models. |
| Model activation | Enable an AI/ML model for a specific function. |
| Model deactivation | Disable an AI/ML model for a specific function. |
| Model download | Model transfer from the network to UE. |
| Model generalization | Using one model that is generalizable to different scenarios/configurations/sites. |
| Model identification | A process/method of identifying an AI/ML model for the common understanding between the NW and the UE. Note: The process/method of model identification may or may not be applicable. Note: Information regarding the AI/ML model may be shared during model identification. |
| Model monitoring | A procedure that monitors the inference performance of the AI/ML model. |
| Model selection | The process of selecting an AI/ML model for activation among multiple models for the same AI/ML enabled feature. Note: Model selection may or may not be carried out simultaneously with model activation. |
| Model switching | Deactivating a currently active AI/ML model and activating a different AI/ML model for a specific function. |
| Model upload | Model transfer from UE to the network. |
| Network-side (AI/ML) model | An AI/ML Model whose inference is performed entirely at the network. |
| Offline field data | The data collected from field and used for offline training of the AI/ML model. |
| Offline training | An AI/ML training process where the model is trained based on collected dataset, and where the trained model is later used or delivered for inference. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as offline training by commonly accepted conventions. |
| One-sided (AI/ML) model | A UE-side (AI/ML) model or a Network-side (AI/ML) model. |
| Online field data | The data collected from field and used for online training of the AI/ML model. |
| Online training | An AI/ML training process where the model being used for inference is (typically continuously) trained in (near) real-time with the arrival of new training samples. Note: the notion of (near) real-time vs. non-real-time is context-dependent and is relative to the inference time-scale. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as online training by commonly accepted conventions. Note: Fine-tuning/re-training may be done via online or offline training. (This note could be removed when we define the term fine-tuning.) |
| Open-format models | ML models of specified format that are mutually recognizable across vendors and allow interoperability, from 3GPP perspective. |
| Proprietary-format models | ML models of vendor-/device-specific proprietary format, from 3GPP perspective NOTE: An example is a device-specific binary executable format |
| Reinforcement Learning (RL) | A process of training an AI/ML model from input (a.k.a. state) and a feedback signal (a.k.a. reward) resulting from the model’s output (a.k.a. action) in an environment the model is interacting with. |
| Semi-supervised learning | A process of training a model with a mix of labelled data and unlabelled data. |
| Supervised learning | A process of training a model from input and its corresponding labels. |
| Two-sided (AI/ML) model | A paired AI/ML Model(s) over which joint inference is performed, where joint inference comprises AI/ML Inference whose inference is performed jointly across the UE and the network, i.e., the first part of inference is firstly performed by UE and then the remaining part is performed by gNB, or vice versa. |
| UE AI/ML capability | UE capability to support AI/ML models for CSI enhancements, beam management, and positioning accuracy enhancements. |
| UE-side (AI/ML) model | An AI/ML Model whose inference is performed entirely at the UE. |
| Unsupervised learning | A process of training a model without labelled data. |

# References

- R4-2305779, “On terminologies for AI/ML”, Ericsson.