3GPP TSG-RAN WG4 Meeting #107              	R4-2308828

Incheon, South Korea, 22th May-26th May, 2023

Agenda item:	9.7.7.2

Source:	Samsung

Title:	Discussion and initial simulation results for eMTC and NB-IoT over NTN

Document for:	Discussion

- Introduction

In the previous RAN4 meeting, the remaining issue for the test setup for SAN demodulation requirement on eMTC and NB-IoT over NTN was under discussion. The related agreement for the test setup for SAN demodulation was captured into the WF [1].

In this contribution, the view on the remaining issue for SAN demodulation requirement on IoT over NTN is provided. Meanwhile, based on the agreed simulation assumption, the simulation results are provided for alignment and performance requirement derivation.

# 2	Discussion

## 2.1	Test setup of eMTC and NB-IoT for NTN

Additional Doppler Shift and timing offset modeling for channel model

Regarding the additional Doppler shift modeling, as agreed in the last meeting, fixed (the largest) frequency offset value for the SAN demodulation requirement was applied for the simulation assumption with following values based on different channels

| Channel | Repetition | Tx duration in one segment | Largest frequency offset value (Hz) |
| --- | --- | --- | --- |
| eMTC PUSCH CE mode A | 8 | 8ms | 4 |
| eMTC PUSCH CE mode B | 256 | 256ms | 128 |
| eMTC PUCCH format 1a | 8 | 8ms | 4 |
| NPUSCH format 1, 3.75KHz, 1 tone | 4 | 256ms | 160 |
| NPUSCH format 1, 15KHz, 12 tones | 16 | 16ms | 8 |
| NPUSCH format 2, 3.75KHz, 1 tone | 16 | 128ms | 64 |
| NPUSCH format 2, 15KHz, 1 tone | 16 | 32ms | 16 |

As indicated in TP 38.821, the maximum Doppler shift variation for a UE with fixed position on Earth is 0.27ppm/s for a LEO satellite with orbit altitude of 600km and 0.13 ppm/s for a LEO satellite as 1200km. Considering 0.27ppm/s as the maximum doppler shift variation, the related additional doppler shift can be calculated based on the duration of segments. With the assuming of 2GHz carrier frequency, the related the additional doppler shift can be calculated. Considering the largest doppler value is agreed for requirement, and CE mode B and NPUSCH 1 with3.75KHz has the same transmission duration, we suggest to update the value as 128 for NPUSCH 1 with 3.75KHz in stead of 160.

Meanwhile, based on initial results, minor different for largest doppler offset between 160 and 128.

Observation 1: The same duration for CE mode B and NPUSCH format 1 with 3.75KHz as 256ms

Observation 2: Minor performance different between the largest doppler offset between 160 and 128.

Proposal 1: Update the value of largest doppler offset as 128 for NPUSCH format 1 with 3.75KHz to align with CE mode B with same transmission duration

PRACH preamble test parameters

For PRACH preamble test parameters, such as Ncs, logical sequence index, and v, can be reused from legacy LTE eMTC PRACH

Proposal 2: apply the same PRACH preamble test parameters for eMTC PRACH requirement in NTN scenario as following

| Burst format | Ncs | Logical sequence index | v |
| --- | --- | --- | --- |
| 0 | 13 | 22 | 32 |
| 1 | 167 | 22 | 2 |
| 2 | 167 | 22 | 0 |
| 3 | 0 | 22 | 0 |

PRACH configuration

For BL/CE UEs, for each PRACH coverage enhancement level, there is a PRACH configuration configured by higher layers with a PRACH configuration index (prach-ConfigurationIndex),

For eMTC PRACH requirements, the PRACH detection performance can be different with different PRACH configuration index. For simulation results alignment and performance requirement derivation, a note was added into spec as

Note 1: Under fading channels, the PRACH detection performance may be significantly different with different PRACH Configuration Indexes. The requirements in this table are defined based on the simulation results with PRACH Configuration Indexes (3, 19, 35, 51) for Format 0, Format 1, Format 2, and Format 3 respectively following LTE

Based on the simulation results provided by companies, if the results can not be aligned well with large gap existed, the same PRACH configuration indexes for each format can be considered for alignment

Proposal 3: Apply the same PRACH Configuration indexes (3,9,35, 51) for Format 0, Format 1, Format 2 and Format 3 requirements for IoT over NTN

NPRACH test configuration

For NPRACH, the hopping pattern with different preamble groups within one NPRACH transmission is related with the initial subcarrier index, also the hopping pattern across different NPRACH repetition transmission is related with the NB-IoT cell id.  In LTE, the test parameters with following value are applied for requirement definition. Since the legacy NB-IoT is reused for NTN, the same parameters can be applied for simulation assumption

Proposal 4: apply the same NPRACH preamble test parameters for NPRACH requirement in NTN scenario as following

| Parameters | Value |
| --- | --- |
| Narrowband physical layer cell identity | 0 |
| Initial subcarrier index | 0 |

Channel Model

In the last meeting, Considering the typical use cases of IoT, low doppler frequency shall be considered. Meanwhile, based on companies’ simulation results, large performance degradation for high doppler scenario can be observed. Therefore, only low doppler is applied similar as LTE.

Regarding different channel models to cover both LOS and NLOS channel condition. In Rel-17 NR NTN, RAN4 introduce two channel models for SAN PUSCH requirement as

- NTN-TDL-A with delay spread 100ns - NTN TDL-C with delay spread 5ns

In our view, for Cat-M1/NB1/NB2 UE, it will connect to the same type of satellites discussed in NR NTN WI. Therefore, we think the same channel model can be applied for IoT UE.  Then, the SAN requirement with both channels can be considered similar as NR NTN scenario. But we are also open to only consider one channel for requirement to reduce the effort if the simulation results can be aligned well among companies.

As for channel model for SAN PUCCH and PRACH requirement, as agreed, only channel is considered with NTN-TDLA 100 to reduce the test efforts.

Proposal 5: The following channel model for requirement definition of SAN demodulation requirement can be considered

- NTN-TDLA100-5Hz and NTN-TDLC5-5Hz for eMTC CE mode A/B - NTN-TDLA100-5Hz for eMTC PUCCH and PRACH - NTN_TDLA100-1Hz and NTN-TDLC5-1Hz for NB1/NB2 UE for NPUSCH1 - NTN_TDLC5-1Hz for NB1/NB2 UE for NPUSCH1 - NTN-TDLA100-5Hz for NPRACH

# 3	Simulation results for IoT over NTN

Based on agreed simulation assumption, the simulation results are provided for IoT over NTN for alignment and performance requirement derivation

## 3.1	eMTC over NTN

In this section, the initial results for eMTC PUSCH CE mode A, CE mode B, PUCCH and PRACH. The detail simulation assumptions for them are indicated in the following tables

Table 1:  simulation assumption for eMTC PUSCH CE mode A

| Parameters | CE Mode A |
| --- | --- |
| Max number of HARQ transmissions | 4 |
| RV sequences | 0, 2, 3, 1, 0, 2, 3, 1 |
| Number of PUSCH repetitions | 8 |
| Frequency hopping | OFF |
| Duplex mode | FDD only |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 1 and 2 |
| Propagation channel | Option 1: NTN-TDLA100-5 Option 2: NTN-TDLC5-5 |
| FRC | TS36.104 A3-2 |
| System BW | 1.4MHz only |
| Doppler shift offset | 4Hz |
| Transmit timing offset | [0.01] us per subframe |
| Fraction of maximum throughput | 70% |

Table 2:  simulation assumption for eMTC PUSCH CE mode B

| Parameters | CE Mode B |
| --- | --- |
| Max number of HARQ transmissions | 2 |
| RV sequences | 0,0,0,0,2,2,2,2, 3,3,3,3,1,1,1,1 |
| Number of PUSCH repetitions | 256 |
| Frequency hopping | OFF |
| Duplex mode | FDD only |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 1 and 2 |
| Propagation channel | Option 1: NTN-TDLA100-5 Option 2: NTN-TDLC5-5 |
| FRC | TS36.104 A3-1 |
| System BW | 1.4MHz only |
| Doppler shift offset | 128 |
| Transmit timing offset | [0.01] us per subframe |
| Fraction of maximum throughput | 70% |

Table 3:  simulation assumption for eMTC PUCCH

| Parameters | Values |
| --- | --- |
| Number of PUCCH repetitions | 8 |
| Frequency hopping | OFF |
| Duplex mode | FDD only |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 1 and 2 |
| Propagation channel | NTN-TDLA100-5 |
| System BW | 1.4MHz |
| ACK missed detection probability | 1% |

Table 4:  simulation assumption for eMTC PRACH

| Parameters | Values |  |
| --- | --- | --- |
| Formats | Format 0/1/2/3 |  |
| Number of PRACH repetitions | Format 0 | 8 |
|  | Format 1 | 8 |
|  | Format 2 | 16 |
|  | Format 3 | 16 |
| Frequency hopping | OFF |  |
| Duplex mode | FDD only |  |
| Number of Tx antennas | 1 |  |
| Number of Rx antennas | 1 and 2 |  |
| Propagation channel | NTN-TDLA100-5 |  |
| System BW | 1.4MHz |  |
| Missed detection probability | 1% |  |
| False alarm | 0.1% |  |

eMTC CE mode A and CE mode B

Table 5:  Ideal and impairment results for eMTC CE mode A and B

| Case | Antenna configuration | Rep | Channel | CE mode | Additional Doppler | Additional Timing offset | FRC | SNR@70% Ideal | SNR@70% Impair |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1T1R | 8 | NTN TDLAC5-5 | A | 4Hz | [0.01:0.01:0.08]us | 36.104 A3-2 | -7.1 | -5.1 |
| 2 | 1T1R | 8 | NTN TDLA100-5 | A | 4Hz | [0.01:0.01:0.08]us | 36.104 A3-2 | -4.5 | -2.5 |
| 3 | 1T2R | 8 | NTN TDLAC5-5 | A | 4Hz | [0.01:0.01:0.08]us | 36.104 A3-2 | -10.2 | -8.2 |
| 4 | 1T2R | 8 | NTN TDLA100-5 | A | 4Hz | [0.01:0.01:0.08]us | 36.104 A3-2 | -8.7 | -6.7 |
| 5 | 1T1R | 256 | NTN TDLAC5-5 | B | 128Hz | [0.01:0.01:2.56]us | 36.104 A3-1 | -13.9 | -11.9 |
| 6 | 1T1R | 256 | NTN TDLA100-5 | B | 128Hz | [0.01:0.01:2.56]us | 36.104 A3-1 | -12.1 | -10.1 |
| 7 | 1T2R | 256 | NTN TDLAC5-5 | B | 128Hz | [0.01:0.01:2.56]us | 36.104 A3-1 | -15.9 | -13.9 |
| 8 | 1T2R | 256 | NTN TDLA100-5 | B | 128Hz | [0.01:0.01:2.56]us | 36.104 A3-1 | -14.9 | -12.9 |

eMTC PUCCH format 1

Table 6:  Ideal and impairment results for eMTC PUCCH format 1

| Case | Format | Frequency hopping | Antenna configuration | BW | Repetition | Channel | Additional Doppler | Additional Timing offsets | SNR@1% ACK missed | SNR@70% Impair |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Format 1a | Off | 1T1R | 1.4MHz | 8 | NTN-TDLA100-5 | 4Hz | [0.01:0.01:0.08]us | -3.6 | -1.6 |
| 2 | Format 1a | off | 1T2R | 1.4MHz | 8 | NTN-TDLA100-5 | 4Hz | [0.01:0.01:0.08]us | -9.8 | -7.8 |

eMTC PRACH

Table 7:  Ideal and impairment results for eMTC PRACH

| Case |  | Format | Antenna configuration | Repetition | Channel | Frequency offset | Timing estimation error | 1% MD ideal | 1% MD impair |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Format 0 |  | 1T1R | 8 | NTN-TDLA100-5 | 270Hz | 2.08us | -12.0 | -10.0 |
| 2 | Format 0 |  | 1T2R | 8 | NTN-TDLA100-5 | 270Hz | 2.08us | -17.8 | -15.8 |
| 3 | Format 1 |  | 1T1R | 8 | NTN-TDLA100-5 | 270Hz | 2.08us | -11.4 | -9.4 |
| 4 | Format 1 |  | 1T2R | 8 | NTN-TDLA100-5 | 270Hz | 2.08us | -17.2 | -15.2 |
| 5 | Format 2 |  | 1T1R | 16 | NTN-TDLA100-5 | 270Hz | 2.08us | -17.8 | -15.8 |
| 6 | Format 2 |  | 1T2R | 16 | NTN-TDLA100-5 | 270Hz | 2.08us | -22.7 | -20.7 |
| 7 | Format 3 |  | 1T1R | 16 | NTN-TDLA100-5 | 270Hz | 2.08us | -17.4 | -15.4 |
| 8 | Format 3 |  | 1T2R | 16 | NTN-TDLA100-5 | 270Hz | 2.08us | -22.3 | -20.3 |

## 3.2	NB-IoT over NTN

In this section, similar as eMTC over NTN, the simulation results for NPUSCH1/2 and NPRACH are provided.

NPUSCH format 1

Table 8:  Ideal and impairment results for NPUSCH1

| Case | Antenna configuration | Repetition | Channel | Tone | SCS | Additional Doppler | Additional Timing offset | FRC | SNR@70% ideal | SNR@70% Impair |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1T1R | 4 | NTN TDLC5-1 | 1 | 3.75KHz | [160] | [0.32:0.32:2.56] | A16-1 | -3.3 | -1.3 |
| 2 | 1T1R | 4 | NTN TDLA-1 | 1 | 3.75KHz | [160] | [0.32:0.32:2.56] | A16-1 | -3.3 | -1.3 |
| 3 | 1T2R | 4 | NTN TDLC5-1 | 1 | 3.75KHz | [160] | [0.32:0.32:2.56] | A16-1 | -6.7 | -4.7 |
| 4 | 1T2R | 4 | NTN TDLA-1 | 1 | 3.75KHz | [160] | [0.32:0.32:2.56] | A16-1 | -6.7 | -4.7 |
| 5 | 1T1R | 4 | NTN TDLC5-1 | 1 | 3.75KHz | 128 | [0.32:0.32:2.56] | A16-1 | -3.3 | -1.3 |
| 6 | 1T1R | 4 | NTN TDLA-1 | 1 | 3.75KHz | 128 | [0.32:0.32:2.56] | A16-1 | -3.3 | -1.3 |
| 7 | 1T2R | 4 | NTN TDLC5-1 | 1 | 3.75KHz | 128 | [0.32:0.32:2.56] | A16-1 | -6.6 | -4.6 |
| 8 | 1T2R | 4 | NTN TDLA-1 | 1 | 3.75KHz | 128 | [0.32:0.32:2.56] | A16-1 | -6.6 | -4.6 |
| 9 | 1T1R | 16 | NTN TDLC5-1 | 12 | 15KHz | 8 | [0.01:0.01:0.16] | A16-5 | -6.0 | -4.0 |
| 10 | 1T1R | 16 | NTN TDLA-1 | 12 | 15KHz | 8 | [0.01:0.01:0.16] | A16-5 | -6.0 | -4.0 |
| 11 | 1T2R | 16 | NTN TDLC5-1 | 12 | 15KHz | 8 | [0.01:0.01:0.16] | A16-5 | -10.3 | -8.3 |
| 12 | 1T2R | 16 | NTN TDLA-1 | 12 | 15KHz | 8 | [0.01:0.01:0.16] | A16-5 | -10.3 | -8.3 |

NPUSCH format 2

Table 9:  Ideal and impairment results for NPUSCH2

| Case | Antenna configuration | Repetition | Channel | Tone | SCS | Additional Doppler | Additional Timing offset | SNR@70% ideal | SNR@70% Impair |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1T1R | 16 | NTN TDLA100-1 | 1 | 3.75KHz | 64Hz | [0.08:0.08:1.28] | 0.84 | 2.84 |
| 2 | 1T2R | 16 | NTN TDLA100-1 | 1 | 3.75KHz | 64Hz | [0.08:0.08:1.28] | -4.91 | -2.91 |
| 3 | 1T1R | 16 | NTN TDLA100-1 | 1 | 15KHz | 16Hz | [0.02:0.02:0.32] | 3.74 | 5.74 |
| 4 | 1T2R | 16 | NTN TDLA100-1 | 1 | 15KHz | 16Hz | [0.02:0.02:0.32] | -4.8 | -2.8 |

NPRACH

Table 10:  Ideal and impairment results for NPRACH

| Case |  | Format | Antenna configuration | Repetition | Channel | Frequency offset | Timing estimation error | 1% MD ideal | 1% MD impair |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Format 0 |  | 1T1R | 8 | NTN-TDLA100-1 | 200Hz | 3.646us | 12.79 | 14.79 |
| 2 | Format 0 |  | 1T2R | 8 | NTN-TDLA100-1 | 200Hz | 3.646us | 3.65 | 5.65 |
| 3 | Format 1 |  | 1T1R | 8 | NTN-TDLA100-1 | 200Hz | 3.646us | 12.65 | 14.65 |
| 4 | Format 1 |  | 1T2R | 8 | NTN-TDLA100-1 | 200Hz | 3.646us | 3.56 | 5.56 |
| 5 | Format 0 |  | 1T1R | 16 | NTN-TDLA100-1 | 200Hz | 3.646us | 8.84 | 10.84 |
| 6 | Format 0 |  | 1T2R | 16 | NTN-TDLA100-1 | 200Hz | 3.646us | 1.06 | 3.06 |
| 7 | Format 1 |  | 1T1R | 16 | NTN-TDLA100-1 | 200Hz | 3.646us | 8.77 | 10.77 |
| 8 | Format 1 |  | 1T2R | 16 | NTN-TDLA100-1 | 200Hz | 3.646us | 1.06 | 3.06 |

# 4	Conclusion

In this contribution, the view on remaining issue of test setup for SAN requirement for IoT over NTN was provided. Meanwhile, the ideal and impairment simulation results are provided for alignment and performance requirement derivation.

Observation 1: The same duration for CE mode B and NPUSCH format 1 with 3.75KHz as 256ms

Observation 2: Minor performance different between the largest doppler offset between 160 and 128.

Proposal 1: Update the value of largest doppler offset as 128 for NPUSCH format 1 with 3.75KHz to align with CE mode B with same transmission duration

Proposal 2: apply the same PRACH preamble test parameters for eMTC PRACH requirement in NTN scenario as following

| Burst format | Ncs | Logical sequence index | v |
| --- | --- | --- | --- |
| 0 | 13 | 22 | 32 |
| 1 | 167 | 22 | 2 |
| 2 | 167 | 22 | 0 |
| 3 | 0 | 22 | 0 |

Proposal 3: Apply the same PRACH Configuration indexes (3,9,35, 51) for Format 0, Format 1, Format 2 and Format 3 requirements for IoT over NTN

Proposal 4: apply the same NPRACH preamble test parameters for NPRACH requirement in NTN scenario as following

| Parameters | Value |
| --- | --- |
| Narrowband physical layer cell identity | 0 |
| Initial subcarrier index | 0 |

Proposal 5: The following channel model for requirement definition of SAN demodulation requirement can be considered

- NTN-TDLA100-5Hz and NTN-TDLC5-5Hz for eMTC CE mode A/B - NTN-TDLA100-5Hz for eMTC PUCCH and PRACH - NTN_TDLA100-1Hz and NTN-TDLC5-1Hz for NB1/NB2 UE for NPUSCH1 - NTN_TDLC5-1Hz for NB1/NB2 UE for NPUSCH1 - NTN-TDLA100-1Hz for NPRACH

# References

R4-2305965 WF on SAN demodulation requirements for IoT over NTN