3GPP TSG-RAN WG4 Meeting # 107	R4-2308858

# Incheon, KR, May 22 – May 26, 2023

Title: 	Discussions on remaining issues on FR2-2 PUSCH requirements

Source: 	Huawei, HiSilicon

Agenda item:       5.2.9.6.2

Document for:	Discussion

Background

In last meeting, one issue on MCS selection for 2x2 was left [1]. In this paper, we provide our related views.

Discussions

In February meeting, RAN4 agreed to use MCS16 as temporary value to define 16QAM PUSCH requirements which can be replaced by MCS12 based on further evaluation:

MCS and number Tx/Rx branches for 16 QAM PUSCH requirements <Agreement> - For 1x2 Low, use TDLA30-650 for 100MHz CBW, TDLA10-650 for 400MHz CBW. - For 2x2 Low, use TDLA30-200 for 100MHz CBW, TDLA10-200 for 400MHz CBW - Consider MCS16 for the requirements - Update requirements on CRs with - TDLA30-650 - 120 kHz and 100 MHz– add requirements in [] - TDLA10-650 - 120 kHz and 400 MHz – add requirements in [] - TDLA10-650 - 480 kHz and 400 MHz – add requirements in [] - Bring simulation results next meeting for - MCS16 - TDLA30-200 - 120 kHz and 100 MHz - TDLA10-200 - 120 kHz and 400 MHz - TDLA10-200 - 480 kHz and 400 MHz - MCS12 - TDLA30-650 - 120 kHz and 100 MHz - TDLA10-650 - 120 kHz and 400 MHz - TDLA10-650 - 480 kHz and 400 MHz - Decide between MCS12 and MCS16 for 2x2 requirements based on the results provided

The simulation results are captured in Table 2-1:

Table 2-1: Simulation results for CP-OFDM for minimum bandwidth for each SCS

| SCS (kHz) | CBW (MHz) | MCS | Channel model | Antenna configuration | Ideal results | Impairment results |
| --- | --- | --- | --- | --- | --- | --- |
| 120 | 100 | 16 | TDLA30-200 | 2x2 Low | Inf | Inf |
| 120 | 400 | 16 | TDLA10-200 | 2x2 Low | Inf | Inf |
| 480 | 400 | 16 | TDLA10-200 | 2x2 Low | Inf | Inf |
| 120 | 100 | 12 | TDLA30-650 | 2x2 Low | 13.6 | 16.1 |
| 120 | 400 | 12 | TDLA10-650 | 2x2 Low | 14.4 | 16.9 |
| 480 | 400 | 12 | TDLA10-650 | 2x2 Low | 11.7 | 14.2 |

Obviously, MCS 16 is not feasible to define the requirements, hence we suggest using choosing MCS12 instead.

Proposal 1: Use MCS12 to define 2T2R, 16QAM requirements.

Conclusion

In this contribution we provide our views on remaining issues on FR2-2 PUSCH requirements. The proposal is:

Proposal 1: Use MCS12 to define 2T2R, 16QAM requirements.

Reference

[1]  R4-2302971_ WF on BS demodulation requirements for FR2-2