3GPP TSG-RAN WG4 Meeting # 107	R4-2308864

# Incheon, KR, May 22 – May 26, 2023

Title: 	Test parameters and simulation results on advanced receiver for MU-MIMO

Source: 	Huawei, HiSilicon

Agenda item:	8.19.1.2

Document for:	Discussion

Background

In last meeting, a WF [1] was agreed and one part is about simulation assumptions and interested companies can bring simulation results based on initial simulation assumptions to study the performance gain of advanced receiver compared to baseline receiver. In this paper, we provide our related simulation results and views on test parameters.

Simulation results

The simulation assumptions are summarized in Table 2-1:

Table 2-1: Simulation assumptions

| Parameter |  | Unit | Value |  |
| --- | --- | --- | --- | --- |
|  |  |  | Target UE | Co-scheduled UE |
| Channel Bandwidth/SCS |  | MHz/KHz | 10/15 |  |
| Duplex mode |  |  | FDD |  |
| MCS |  |  | 13,17 | Rank 1+1: QPSK Rank 2+2: QPSK and 64QAM |
| Allocation for interference UE and target UE | Rank allocation |  | 1 | 1 |
|  |  |  | 2 | 2 |
|  | Scrambling ID |  | Same scrambling ID for both UEs |  |
| MIMO configuration |  |  | For Rank 1+1: 2T2R Medium For Rank 2+2: 4T4R Low |  |
| Number of CDM groups without data |  |  | 1 for paired UE allocated in same CDM groups and 2 for paired UE allocated in different CDM groups |  |
| HARQ process number |  |  | 4 |  |
| Precoding model | Target UE |  | Random precoding with Single panel Type 1 per PRB bundling size per slot | - Option 1: Select the precoding matrix to ensure orthogonality with target UE - Option 2: Select the precoding matrix randomly ensuring the selected precoding matrix shall not be identical to the precoding matrix of target UE |
| PDSCH configuration | Mapping type |  | Type A |  |
|  | Starting symbol (S) |  | 2 |  |
|  | Length (L) |  | 12 |  |
|  | PRB bundling size |  | 2 |  |
|  | PRB bundling type |  | Static |  |
| PDSCH DMRS configuration | DMRS Type |  | DMRS Type 1 |  |
|  | Number of additional DMRS |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |
| Propagation conditions |  |  | TDLC300-100,TDLA30-10 |  |
| Receiver type |  |  | MMSE-IRC,E-MMSE-IRC, R-ML | N/A |
| Test metric |  |  | SNR @ %70 of maximum Throughput | N/A |
| Note: It is noted that all information of co-scheduled UE is known to target UE |  |  |  |  |

Rank 1+1

Normalized TP-SNR curve is captured in Figure 2-2

Figure 2-2: Normalized TP-SNR curve for Rank 1+1

Rank 2+2

Normalized TP-SNR curve is captured in Figure 2-3

Figure 2-3: Normalized TP-SNR curve for Rank 2+2

Summary of simulation results are captured in Table 2-2:

Table 2-2: Summary of simulation results

| Rank allocation | Propagation conditions | Serving UE:MCS13 (16QAM) |  | E-MMSE-IRC(Gain) | R-ML(Gain) |
| --- | --- | --- | --- | --- | --- |
| Rank 1+1 | TDLC300-100 | Interference UE: QPSK | Random PMI | 1.0 | 11.0 |
|  |  |  | Orthogonal | 0.8 | 9.2 |
| Rank 2+2 | TDLC300-100 | Interference UE: QPSK | Random PMI | 4.0 | 8.6 |
|  |  |  | Orthogonal | 1.2 | 3.8 |
|  |  | Interference UE: 64QAM | Random PMI | 4.0 | 4.9 |
|  |  |  | Orthogonal | 1.2 | 1.6 |
|  | TDLA30-10 | Interference UE: QPSK | Random PMI | 0.0 | 4.2 |
|  |  |  | Orthogonal | 0.0 | 2.8 |
|  |  | Interference UE: 64QAM | Random PMI | 0.0 | 0.5 |
|  |  |  | Orthogonal | 0.0 | 0.4 |
| Rank allocation | Propagation conditions | Serving UE:MCS17 (64QAM) |  | E-MMSE-IRC(Gain) | R-ML(Gain) |
| Rank 2+2 | TDLA30-10 | Interference UE: QPSK | Orthogonal | 0.0 | 3.3 |
|  |  | Interference UE: 64QAM | Orthogonal | 0.1 | 0.2 |

We can make following conclusions based on the observations from above simulation results:

- Observation1: R-ML performs much better than E-MMSE-IRC when modulation order of interference UE is smaller than serving UE.

- Observation2: E-MMSE-IRC has no performance gain over MMSE-IRC for TDLA30-10, even for TDLC300-100, the gain is limited.

- Observation3: Random PMI increases the performance gain significantly compared to orthogonal PMI.

Discussions

Number of co-scheduled UEs

One issue is number of co-scheduled UEs, options are listed as follows:

Issue 2-1-1: The number of co-scheduled UEs Candidate options: - Option 1: 1 co-scheduled UE as higher priority - Option 2: Cover 2 co-scheduled UEs at least in phase I study

Number of Co-scheduled UEs is agnostic to target UE and have no impact on baseband processing, we think only 1 co-scheduled UE is enough.

MCS for the target UE

In last meeting, one key issue is MCS for the target UE.

Issue 2-1-4: MCS for the target UE Way forward - Cover MCS 13 for rank 1 and rank 2 for initial simulation - Cover MCS 17 for rank 2 for initial simulation - Further discuss whether to cover MCS 4 for rank 1 in the next meeting - The assumption can be updated later based on available results.

Based on our understanding, R-ML receiver only has advantage on high SNR and relative low modulation order of co-scheduled UE. MCS4 may not fit this condition, we are fine to only consider MCS17 and MCS19

Modulation order for the co-scheduled UE

One issue is modulation order of co-scheduled UE, options are listed as follows:

- Maintain last meeting status from the modulation order perspective: For R-ML, E-IRC and IRC (baseline in Rel-17, for performance comparison purpose) for initial simulation - For rank 1+1: QPSK (high priority) - For rank 2+2: 64QAM (high priority) - For rank 2+2: QPSK (high priority) - For rank 1+1 (64QAM) +1 (QPSK) (lower priority) - For rank 1+1 (64QAM) +1 (16QAM) (lowest priority) - Other options on the modulation order for co-scheduled UE are not precluded. - These assumptions can be updated in the next meeting based on available simulation results.

Based on our understanding, multiple modulation order is unrealistic because it is impossible to make UE distinguish which modulation order belongs to which UE since either complicate blind detection algorithm or signalling with large overhead is needed. Therefore, we suggest not considering multiple modulation order in phase 1.

PDSCH resource allocation for the target and co-scheduled UE

Way forward - For initial simulation in phase I, cover scenario 1 - FFS whether to cover scenario 2, based on the discussion outcome in the NWA part.

Scenario 2 can be summarized as that different UE with different modulation order are located across the whole bandwidth scheduled for target UE. Similar views as modulation order, this is unrealistic due to the fact it is hard to design the signalling with partial bandwidth granularity with limited bit overhead.

We suggest focusing on following conditions in Phase 1:

Proposal 1: Focus on following conditions in Phase 1:

- Only 1 co-scheduled UE which has same bandwidth with target UE

- Rank 1+1:

  - Target UE: MCS13， co-scheduled UE: QPSK, Random PMI, TDLC300-100, 2T2R, ULA Medium

- Rank 2+2:

  - Target UE: MCS13， co-scheduled UE: QPSK, Orthogonal PMI, TDLA30-10, 4T4R, ULA Low

  - Target UE: MCS13， co-scheduled UE: 64QAM, Orthogonal PMI, TDLA30-10, 4T4R, ULA Low

  - Target UE: MCS17， co-scheduled UE: QPSK, Orthogonal PMI, TDLA30-10, 4T4R, ULA Low

  - Target UE: MCS17， co-scheduled UE: 64QAM, Orthogonal PMI, TDLA30-10, 4T4R, ULA Low

Recommendations in phase 2

Due to the higher up-bound performance gain, it is more feasible to define R-ML requirements in phase 2. Meanwhile, to trade off the implementation complexity and performance gain, it’s better to focus on conditions that modulation order of co-scheduled UE is smaller than target UE. If a target UE detects a co-scheduled UE with higher modulation order, this target UE may fallback to E-MMSE-IRC receiver.

Proposal 2: RAN 4 to focus on R-ML receiver for requirements definition in phase 2 and focus on conditions that modulation order of co-scheduled UE is smaller than target UE. E.g. (64QAM+QPSK)

Conclusion

In this contribution we provide our simulation results based on agreed assumptions and views on receiver assumptions. The observations and proposals are:

- Observation1: R-ML performs much better than E-MMSE-IRC when modulation order of interference UE is smaller than serving UE.

- Observation2: E-MMSE-IRC has no performance gain over MMSE-IRC for TDLA30-10, even for TDLC300-100, the gain is limited.

- Observation3: Random PMI increases the performance gain significantly compared to orthogonal PMI.

Proposal 1: Focus on following conditions in Phase 1:

- Only 1 co-scheduled UE which has same bandwidth with target UE

- Rank 1+1:

  - Target UE: MCS13， co-scheduled UE: QPSK, Random PMI, TDLC300-100, 2T2R, ULA Medium

- Rank 2+2:

  - Target UE: MCS13， co-scheduled UE: QPSK, Orthogonal PMI, TDLA30-10, 4T4R, ULA Low

  - Target UE: MCS13， co-scheduled UE: 64QAM, Orthogonal PMI, TDLA30-10, 4T4R, ULA Low

  - Target UE: MCS17， co-scheduled UE: QPSK, Orthogonal PMI, TDLA30-10, 4T4R, ULA Low

  - Target UE: MCS17， co-scheduled UE: 64QAM, Orthogonal PMI, TDLA30-10, 4T4R, ULA Low

Proposal 2: RAN 4 focus on R-ML receiver for requirements definition in phase 2 and focus on conditions that modulation order of co-scheduled UE is smaller than target UE. E.g. (64QAM+QPSK)

Reference

- R4-2305914 WF on advanced receiver for MU-MIMO scenario. China Telecom.