3GPP TSG-RAN WG4 Meeting #107	R4-2308880

# Incheon, 22nd - 26th May, 2023

Title: 	Discussion on general aspects for NR ATG demodulation requirements

Source: 	Huawei, HiSilicon

Agenda item:	8.14.5.1

Document for:	Discussion

Background

During RAN4#106-bis-e meeting, WF [1] for ATG demodulation requirements was approved. In this contribution, we share our views about general aspects for NR ATG demodulation requirements.

Discussion

Doppler shift assumption

Agreement from GTW: - Align with RF session agreement on frequency error requirements with below values as starting point - For FDD channel model, set Doppler as [220Hz] - For TDD channel model, set Doppler as [400/500Hz]. FFS: - For FDD channel model, - Option 1: set Doppler as 220Hz - Option 2: Set Doppler as 200Hz for UL, 220Hz for DL - For TDD channel model: - Option 1: set Doppler as 400Hz. - Option 2: set Doppler as 500Hz

As per RF agreement achieved in last meeting, 0.1 ppm is defined for the frequency error. Considering the NR operating band n1 and n79 as shown in the following Table 2.1-1, the maximum frequency error is 198Hz for UL and 217Hz for DL for FDD case and 500Hz for TDD case.

Table 2.1-1: NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n1 | 1920 MHz – 1980 MHz | 2110 MHz – 2170 MHz | FDD |
| n79 | 4400 MHz – 5000 MHz | 4400 MHz – 5000 MHz | TDD |

For FDD, select Doppler as 200Hz for UL, 220Hz for DL.

For TDD, select Doppler as 500Hz.

Proposals

In this contribution, we discuss general aspects for NR ATG demodulation requirements. Our observations and proposals are:

- For FDD, select Doppler as 200Hz for UL, 220Hz for DL.

For TDD, select Doppler as 500Hz.

Reference

R4-2305899, WF on IoT-NTN UE demodulation requirements, RAN4#106-bis-e, CMCC