3GPP TSG-RAN WG4 Meeting #107	R4-2308881

# Incheon, 22nd - 26th May, 2023

Title: 	Discussion on NR BS ATG demodulation requirements

Source: 	Huawei, HiSilicon

Agenda item:	8.14.5.3

Document for:	Discussion

Background

During RAN4#106-bis-e meeting, WF [1] for ATG demodulation requirements was approved. In this contribution, we share our views about NR BS ATG demodulation requirements.

Discussion

Test scope

- FFS For PUSCH and PUCCH - Option 1: - PUSCH - Normal PUSCH with CP-OFDM with down selection - UCI multiplexed on PUSCH - PUSCH for 2-step RA type - PUCCH - DTX to ACK probability - Normal PUCCH format 0/1/2/3/4 - Option 2: Define new dedicated requirements for PUSCH/PUCCH for AGWN channel model - Option 3: Define new incremental requirements dedicated for ATG BS, i.e. a BS supporting ATG feature should pass both the existing requirements and the new dedicated requirements for ATG based on manufacture declaration. - FFS For PRACH - Option 1: - False alarm probability - Normal mode PRACH detection - Reuse legacy TN BS PRACH format 0, A1/A2/A3/B4/C0/C2 demodulation requirements for ATG BS PRACH. - Option 2: - Consider both long and short formats for ATG. Format 0, A2, B4 and C2 can be the starting point. - Option 3: Only consider long format 0 for PRACH. - Option 4: - Define new incremental requirements dedicated for ATG BS, i.e. a BS supporting ATG feature should pass both the existing requirements and the new dedicated requirements for ATG based on manufacture declaration. - Select the same formats as NTN for ATG BS demodulation requirements, i.e. preamble format 0, 2, B4 and C2. - Option 5: Existing requirement with format 0 with 625Hz doppler frequency can be applied for ATG scenario

For PUSCH and PUCCH, consider single path AWGN channel with doppler channel model is assumed for the ATG scenario, that is simpler than the fading channel from UE processing point of view, we don’t think it necessary to define new incremental requirements dedicated for ATG BS, the existing legacy TN requirements can be reused. In addition, we don’t think it necessary to add limitation on the detail test scope, all features should be allowed for the ATG BS and which cases can be tested should be based on BS manufacturer declaration.

For PUSCH and PUCCH, no incremental requirements dedicated for ATG BS should be defined, the existing legacy TN requirements can be reused. All features should be allowed for the ATG BS and which cases can be tested should be based on BS manufacturer declaration.

For PRACH, it is not reasonable to select Option 3 or Option 5 since it is not enough to only consider PRACH format 0 from test coverage point of view. Considering UE pre-compensation on UL frequency shift and UL timing shift as baseline, all preamble formats are feasible. We propose to select the same format as NTN for ATG BS demodulation requirements, i.e. preamble format 0, 2, B4 and C2. In addition, the existing requirement with preamble format 0 with 625Hz doppler frequency is defined for restricted set type A. We do not think it necessary to define PRACH requirements with restricted set since there is no very larger doppler frequency after UL frequency shift pre-compensation.

For PRACH, select the same format as NTN for ATG BS demodulation requirements, i.e. preamble format 0, 2, B4 and C2.

Do not define PRACH requirements with restricted set.

Bandwidth & SCS

- For FDD 15kHz - Option 1: 5MHz, 20MHz and 40MHz - Option 2: 20MHz - Option 3: 5MHz - For TDD 30kHz - Option 1: 10MHz, 40MHz, 60MHz and 100MHz - Option 2: 100MHz - Option 3: 10MHz

For the bandwidth and SCS, minimum bandwidth for each SCS can be selected to reduce the simulation effort and ensure the test coverage as much as possible. From our understanding, there is negligible performance difference for the cases with same configurations but different bandwidth.

Select minimum bandwidth for each SCS for ATG BS performance requirements definition.

TDD pattern

- Option 1: Reusing legacy TDD pattern - Option 2: Define a new TDD pattern 30D4S6U which only applied for ATG scenario. - Option 3: The requirement of TDD with existing Patten can be applied for TDD with different UL-DL pattern in ATG scenario. FFS on introducing a new TDD pattern only applied for ATG scenario. - Option 4: If new dedicated TDD pattern is considered, note should be added in the specification that this is pattern is for ATG scenario used only.

Currently, there is no TDD patterns defined in TS 38.104 but only default TDD pattern or example TDD pattern is specified for BS performance requirements. However, all existing default TDD pattern or example TDD pattern are not feasible since that there is large timing difference between DL and UL due to large BS-UE RTT delay in ATG scenario. In last meeting, a new TDD pattern is proposed by companies to set large guard period for the switching from DL to UL. From our understanding, if new dedicated TDD pattern is considered, note should be added in the specification that this is pattern is for ATG scenario used only.

In addition, we would like to further study the feasibility for the new proposed TDD pattern. In current specification, the maximum feasible DL-DataToUL-ACK-r16 value is 15. In Rel-17 there is some enhancement but the dl-DataToUL-ACK-v1700 is applicable for NTN and dl-DataToUL-ACK-r17 is applicable for up to 71 GHz. It is not sure whether it can be directly used for ATG.

DL-DataToUL-ACK-r16 ::=                    SEQUENCE (SIZE (1..8)) OF INTEGER (-1..15)

DL-DataToUL-ACK-r17 ::=                    SEQUENCE (SIZE (1..8)) OF INTEGER (-1..127)

DL-DataToUL-ACK-v1700 ::=                  SEQUENCE (SIZE (1..8)) OF INTEGER (16..31)

It is not sure Rel-17 enhancement on dl-DataToUL-ACK can be directly used for ATG.

For the maximum HARQ process number, we think additional UE capability is needed, since the current UE capability is not applicable for ATG bands.

Current UE capability for the maximum HARQ process number is not applicable for ATG bands.

Further study the feasibility for the new proposed TDD pattern.

If new dedicated TDD pattern is considered, note should be added in the specification that this is pattern is for ATG scenario used only.

Antenna Configuration

- 1T2R - FFS: - Option 1: 1/2Tx and 2/4/8 Rx for 1-C/1-H; and 1/2Tx and 2Rx for 1-O. - Option 2: 1/2Tx and 2/4 Rx for 1-C/1-H; and 1/2Tx and 2Rx for 1-O.

For the antenna configuration, we propose to use the same antenna configurations and manufacture declarations as TN BS for ATG demodulation requirements, e.g. 1/2Tx and 2/4/8 Rx for 1-C/1-H; and 1/2Tx and 2Rx for 1-O (2Tx is only for PUSCH requirements).

Consider 1/2Tx and 2/4/8 Rx for 1-C/1-H; and 1/2Tx and 2Rx for 1-O (2Tx is only for PUSCH requirements) for ATG BS demodulation requirements.

Transform precoding

- Option 1: Only consider CP-OFDM. - Option 2: Consider CP-OFDM and DFT-S-OFDM waveform

From our understanding, we should focus on ATG feature for the testing and select only one certain typical transform precoding configuration, i.e. CP-OFDM like other WIs did. We don’t think there is necessity to cover both DFT-S-OFDM and CP-OFDM for ATG since the performance for different transform precoding configuration has been verified by the existing BS performance requirements.

Only consider CP-OFDM waveform for ATG BS performance requirements.

Test metric

- Option 1: Consider 70% and 30% throughput requirements for ATG PUSCH demodulation. - Option 2: 70% throughput requirements

We think both 70% and 30% throughput requirements are necessary. As we discussed in clause 2.1, no incremental requirements dedicated for ATG BS should be defined, the existing legacy TN requirements can be reused. So both 70% and 30% throughput requirements can be reused.

Consider both 70% and 30% throughput requirements reused from the existing legacy TN requirements.

Proposals

In this contribution, we discuss on NR BS ATG demodulation requirements. Our observations and proposals are:

- For PUSCH and PUCCH, no incremental requirements dedicated for ATG BS should be defined, the existing legacy TN requirements can be reused. All features should be allowed for the ATG BS and which cases can be tested should be based on BS manufacturer declaration.

For PRACH, select the same format as NTN for ATG BS demodulation requirements, i.e. preamble format 0, 2, B4 and C2.

Do not define PRACH requirements with restricted set.

Select minimum bandwidth for each SCS for ATG BS performance requirements definition.

- It is not sure Rel-17 enhancement on dl-DataToUL-ACK can be directly used for ATG.

- Current UE capability for the maximum HARQ process number is not applicable for ATG bands.

Further study the feasibility for the new proposed TDD pattern.

If new dedicated TDD pattern is considered, note should be added in the specification that this is pattern is for ATG scenario used only.

Consider 1/2Tx and 2/4/8 Rx for 1-C/1-H; and 1/2Tx and 2Rx for 1-O (2Tx is only for PUSCH requirements) for ATG BS demodulation requirements.

Only consider CP-OFDM waveform for ATG BS performance requirements.

Consider both 70% and 30% throughput requirements reused from the existing legacy TN requirements.

Reference

R4-2305899, WF for ATG demodulation requirements, RAN4#106-bis-e, CMCC