3GPP TSG-RAN WG4 Meeting #107	R4-2308882

# Incheon, 22nd - 26th May, 2023

Title: 	Discussion on NR UE ATG demodulation requirements

Source: 	Huawei, HiSilicon

Agenda item:	8.14.5.2

Document for:	Discussion

Background

During RAN4#106-bis-e meeting, WF [1] for ATG demodulation requirements was approved. In this contribution, we share our views about NR UE ATG demodulation requirements.

Discussion

Test scope

- FFS For PDSCH: - Option 1: Introduce PDSCH requirements with AWGN+Doppler channel model - Option 2: FFS - Decide whether to introduce new PDSCH requirements after channel model and other parameters (e.g., bandwidth, TDD pattern and so on) are agreed. - If no new PDSCH requirements for ATG, further decide which legacy PDSCH requirements shall be applied for ATG UE. - FFS For PDCCH: - Option 1: Introduce PDCCH requirements with AWGN+Doppler channel model - Option 2: No need to introduce additional PDCCH requirements. But the legacy PDCCH mandatory requirements shall be met. - Option 3: FFS - Decide whether to introduce new PDCCH requirements after channel model is agreed. - If no new PDCCH requirements for ATG, whether and which legacy PDCCH requirements shall be applied for ATG UE - For CSI reporting: - Do not define new CSI reporting requirements for ATG UE. - FFS - Option 1: legacy CSI reporting requirements shall be met. - Option 2: FFS whether and which legacy UE CSI reporting requirements shall be applied for ATG UE.

For PDSCH/PDCCH demodulation and CSI reporting requirements, consider single path AWGN channel with doppler channel model is assumed for the ATG scenario, that is simpler than the fading channel from UE processing point of view, we don’t think it necessary to define new incremental requirements dedicated for ATG UE, the existing legacy TN requirements can be reused. In addition, we don’t think it necessary to add limitation on the detail test scope, all features should be allowed for the ATG UE and which cases can be tested should be based on UE capability.

For PDSCH/PDCCH demodulation and CSI reporting requirements, no incremental requirements dedicated for ATG UE should be defined, the existing legacy TN requirements can be reused. All features should be allowed for the ATG UE and which cases can be tested should be based on UE capability.

TDD pattern

- Option 1: Define a new TDD pattern 30D4S6U which only applied for ATG scenario. - Option 2: Proceed with typical TDD pattern for requirement, further check the performance if the new TDD pattern is agreed to be introduced - RF, RRM and Demod sessions should keep the consistency on the agreement for the necessary of new TDD pattern for ATG scenario. - Start with FDD requirement configuration if companies cannot get consensus on TDD pattern issue easily - Option 3: If new dedicated TDD pattern is considered, note should be added in the specification that this is pattern is for ATG scenario used only.

Currently, all existing TDD pattern are not feasible since that there is large timing difference between DL and UL due to large BS-UE RTT delay in ATG scenario. In last meeting, a new TDD pattern is proposed by companies to set large guard period for the switching from DL to UL. From our understanding, if new dedicated TDD pattern is considered, note should be added in the specification that this is pattern is for ATG scenario used only.

In addition, we would like to further study the feasibility for the new proposed TDD pattern. In current specification, the maximum feasible DL-DataToUL-ACK-r16 value is 15. In Rel-17 there is some enhancement but the dl-DataToUL-ACK-v1700 is applicable for NTN and dl-DataToUL-ACK-r17 is applicable for up to 71 GHz. It is not sure whether it can be directly used for ATG.

DL-DataToUL-ACK-r16 ::=                    SEQUENCE (SIZE (1..8)) OF INTEGER (-1..15)

DL-DataToUL-ACK-r17 ::=                    SEQUENCE (SIZE (1..8)) OF INTEGER (-1..127)

DL-DataToUL-ACK-v1700 ::=                  SEQUENCE (SIZE (1..8)) OF INTEGER (16..31)

It is not sure Rel-17 enhancement on dl-DataToUL-ACK can be directly used for ATG.

For the maximum HARQ process number, we think additional UE capability is needed, since the current UE capability is not applicable for ATG bands.

Current UE capability for the maximum HARQ process number is not applicable for ATG bands.

Further study the feasibility for the new proposed TDD pattern.

If new dedicated TDD pattern is considered, note should be added in the specification that this is pattern is for ATG scenario used only.

Bandwidth & SCS

- For TDD: - Option 1: 40MHz for TDD 30kHz. - Option 2: 100MHz for TDD 30kHz. - For FDD: - Option 1: 20Mhz for FDD 15kHz - Option 2: 10MHz for FDD 15kHz

For the bandwidth and SCS, we propose to select 10MHz for FDD 15kHz and 40MHz for ATG UE performance requirements that is the same value as legacy UE performance requirements.

Select 10MHz for FDD 15kHz and 40MHz for TDD 30kHz SCS for ATG UE performance requirements.

Antenna Configuration

- 2T2R - FFS: - 2T4R (for UE supporting 4R) - 4T4R (for UE supporting 4R)

As per agreement achieved in RF session in last meeting, 4Rx is not mandate for ATG UE in the legacy 4Rx-mandated-bands, so 4Rx requirements are only applied for UE supporting 4Rx. We propose to use the same antenna configurations as legacy UE for ATG UE demodulation requirements, i.e. 2x2, 2x4 and 4x4 for both FDD and TDD.

Consider 2x2, 2x4 and 4x4 for both FDD and TDD for ATG UE demodulation requirements. 4Rx requirements are only applied for UE supporting 4Rx.

Proposals

In this contribution, we discuss on NR UE ATG demodulation requirements. Our observations and proposals are:

- For PDSCH/PDCCH demodulation and CSI reporting requirements, no incremental requirements dedicated for ATG UE should be defined, the existing legacy TN requirements can be reused. All features should be allowed for the ATG UE and which cases can be tested should be based on UE capability.

- It is not sure Rel-17 enhancement on dl-DataToUL-ACK can be directly used for ATG.

Current UE capability for the maximum HARQ process number is not applicable for ATG bands.

Further study the feasibility for the new proposed TDD pattern.

If new dedicated TDD pattern is considered, note should be added in the specification that this is pattern is for ATG scenario used only.

Select 40MHz for TDD 30kHz SCS for ATG UE performance requirements.

Consider 2x2, 2x4 and 4x4 for both FDD and TDD for ATG UE demodulation requirements. 4Rx requirements are only applied for UE supporting 4Rx.

Reference

R4-2305899, WF for ATG demodulation requirements, RAN4#106-bis-e, CMCC