3GPP TSG-RAN WG4 Meeting # 107	R4-2308945

Incheon, KR, May 22 – May 26, 2023

Agenda Item:	8.8.4.2

Source:	Ericsson

Title:	PDSCH demodulation requirements for FR2 multi-Rx chain

Document for:	Discussion

# 1	Introduction

Last meeting #106bis-e, it was the second meeting for this WI where further discussions have been conducted around general aspects. In this meeting, we might further progress on PDSCH demodulation requirements’ parameters. Below, we summarize the PDSCH related open issues as stated in [1].

Issue 2-1-1: AoA assumption - Agreement: - Do not define demodulation requirements for all AoA offsets considered in RF requirements. - Define demodulation requirements for 1 or 2 correlation types based on feasibility and performance evaluation. Issue 2-1-2: Power imbalance between TRPs - Agreement: - Do not introduce power imbalance between the TRPs for FR2 multi-Rx demod requirements. Issue 2-2-1: Channel model - Proposals - Option 1: Consider including TDLA30-300 for 120kHz/200MHz requirements, if introduced - Option 2: Consider TDLA30-300 for 100 MHz/120 kHz. - Option 3: Consider TDLA30-75 for 100MHz/120KHz Issue 2-2-2: SCS/BW - Agreement: - Define 120kHz/100MHz for SCS/Bandwidth as first priority. Issue 2-2-3: MIMO layers per TRP - Agreement: - Consider the following for initial evaluation- - Single-DCI based SDM scheme using 1+1- and 2+2-layer combination - Multi-DCI based fully-overlapping resource allocation using 2+2-layer combination - Multi-DCI with non-overlapping resource allocation using 2+2-layer combination. Issue 2-2-4: MCS - Agreement: - Consider the following MCS for initial evaluation- - Use MCS 13 or lower for defining rank 2+2, higher inter-TRP interference cases with SDM/fully-overlapping scheduling. - Use MCS 17 for rank 1+1 cases or lower inter-TRP interference cases with SDM or non-overlapping case. Issue 2-1-5: PTRS Port - Proposals - Option 1: One PTRS port per TRP for both sDCI and mDCI schemes - Option 2: One port PTRS for s-DCI SDM scheme Issue 2-1-6: PTRS Pattern - Agreement: - For PTRS pattern, consider K=2, L=1 as the frequency and time density, respectively. Issue 2-2-7: Time offset between TRPs - Proposals - Option 1: Use the time offset between the two TRPs transmission points as {0.25us, -0.0625us}. - Option 2: Use time offset of TRP1 with respect to TRP2 as {0.2us, -0.05us} for different test cases. - Option 3: For NR FR2-1 multi-Rx chain DL reception, consider sDCI and mDCI based SDM scheme with timing offset between TRxPs as follow - Timing offset of the second TRxP from the first TRxP [us] as -0.0625 or 0.25. - Option 4: Select following value for timing offset of the second TRxP from the first TRxP for demodulation cases. Timing offset[us]. Multi-DCI based multi-TRP; non-overlapping; -0.0625. full-overlapping; -0.0625. Single-DCI based multi-TRP; SDM; 0.25. Issue 2-2-8: Frequency offset between TRPs - Proposals - Option 1: Select 3000Hz frequency offset for all demodulation cases as starting point. - Option 2: 600Hz - Option 3: 400Hz Issue 2-2-9: TDD slot pattern - Agreement: - Define the FR2 multi-Rx UE demodulation requirements for PDSCH using FR2.120-1 TDD UL-DL pattern, given by DDDSU and S: 10D+2G+2U. Issue 2-2-10: Test cases and simulation parameters for sDCI SDM - Agreement: - Further discuss after relevant open issues are finalized. Issue 2-2-11: Test cases and simulation parameters for mDCI fully overlapping - Agreement: - Further discuss after relevant open issues are finalized. Issue 2-2-12: Test cases and simulation parameters for mDCI non-overlapping - Agreement: - Further discuss after relevant open issues are finalized. Issue 2-2-13: Performance metric - Agreement: - Define the FR2 multi-Rx UE demodulation requirements for PDSCH with 70% of the peak throughput metric only. Issue 2-2-14: Whether to consider additional margin for 47GHz carrier frequency - Agreement: - Discuss later when requirements are defined.

# 2	PDSCH Demodulation Requirements

## 2.1 PTRS

NR operations in FR2-1 band are susceptible to phase noise (PN) effects that should be assessed and addressed. In WF [1], we agreed to use Rel-15 pattern with K = 2 and L = 1. Hereafter, we provide our views on PTRS ports to be used for both sDCI and mDCI schemes.

For this WI, we need to consider one PTRS port per TRP for mDCI schemes.

Proposal 1: For mDCI, consider one port per TRxP for PTRS.

In TS. 38.306, it was shown that the capability parameter ‘supportTwoPortDL-PTRS-r16’ is optional, as

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| supportTwoPortDL-PTRS-r16 Indicates whether UE supports 2-port DL PT-RS. UE supports this feature should indicate support singleDCI-SDM-scheme-r16 for the band. | Band | No | N/A | N/A |

Consequently, we can consider only one PTRS port for sDCI SDM based scheme. This is captured in Proposal 2 as follow.

Proposal 2: Considering that the capability parameter ‘supportTwoPortDL-PTRS-r16’ is not mandatory (optional), we can consider only one PTRS port for sDCI SDM based scheme.

## 2.2 Time/Frequency offsets and power imbalance between TRxPs

In FR1 TDD, the timing and frequency offsets of the second TRxP from the first TRxP have been considered in TS 38.101-4 [3] for sDCI and mDCI based SDM schemes as follow

| Timing offset of the second TRxP from the first TRxP | us | -0.25 or 1 |
| --- | --- | --- |
| Frequency offset of the second TRxP from the first TRxP | Hz | 300 or 0 |

In this WI, we should consider reasonable time and frequency offsets between TRxPs that meet FR2 conditions. Considering 120 kHz SCS [2], the proposed time and frequency offsets are summarized in the following proposal to be considered as a starting point.

Proposal 3: For NR FR2-1 multi-Rx chain DL reception, consider sDCI SDM based scheme and mDCI with full or non-overlapping resource allocation scheme with timing and frequency offsets between TRxPs as follow

- Timing offset of the second TRxP from the first TRxP [us] as -0.0625 or 0.25.

- Frequency offset of the second TRxP from the first TRxP [Hz] as 3000 or 0.

## 2.3 PDSCH demodulation parameters

To save efforts, it is preferrable to limit our requirements to PDSCH Type A and focus on sDCI based SDM scheme, mDCI with full overlapping resource allocation scheme and mDCI with non-overlapping resource allocation scheme.

Proposal 4: Define PDSCH Type A demodulation requirements for

- Single-DCI based SDM scheme using 1+1- and 2+2-layer combination

- Multi-DCI with full overlapping resource allocation using 2+2-layer combination

- Multi-DCI with non-overlapping resource allocation using 2+2-layer combination

Considering both sDCI and m-DCI based transmission, we summarize the needed parameters to define the PDSCH requirements in the following proposals.

Proposal 5: Consider PDSCH test cases for sDCI based SDM scheme as follow

Test Parameters

| Parameter |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  | TCI State #1 |  |
|  |  | CORESETPoolIndex |  | 0 |  |
| Duplex mode |  |  |  | TDD |  |
| Active DL BWP index |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  | Type A |  |
|  | k0 |  |  | 0 |  |
|  | Starting symbol (S) |  |  | 2 |  |
|  | Length (L) |  |  | 12 |  |
|  | PRB bundling type |  |  | Static |  |
|  | PRB bundling size |  |  | 2 |  |
|  | Resource allocation type |  |  | Type 1 |  |
|  | RBG size |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  | 1000 | 1002 |
|  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |  |
| Resource allocation |  |  |  | Full-overlappling |  |
| Timing offset of the second TRxP from the first TRxP |  |  | us | -0.0625 for test 1-1 and test 1-3 0.25 for test 1-2 and test 1-4 |  |
| Frequency offset of the second TRxP from the first TRxP |  |  | Hz | 3000 for test 1-1 and test 1-3 0 for test 1-2 and test 1-4 |  |
| Precoding configuration |  |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity. |  |
| Note 1: PDSCH transmission is done from both TRxPs (PDSCH Layer 0 is transmitted from TRxP #1 and PDSCH layer 1 is transmitted from TRxP #2) Note 2: PDSCH transmission is done from both TRxPs (PDSCH Layers 0 and 1 are transmitted from TRxP #1 and PDSCH layers 2 and 3 are transmitted from TRxP #2) |  |  |  |  |  |

Minimum performance

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition (Note 1) | Correlation matrix and antenna configuration (Note 2) | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) (Note 3) |
| 1-1 | R.PDSCH.5-3.2 TDD | 40 / 30 | 64QAM, 0.43 | FR2.120-1 | TDLA30-300 | 2x4, ULA Low | 70 | TBD |
| 1-2 | R.PDSCH.5-3.2 TDD | 40 / 30 | 64QAM, 0.43 | FR2.120-1 | TDLA30-300 | 2x4, ULA Low | 70 | TBD |
| 1-3 | R.PDSCH.5-2.2 TDD | 40/30 | 16QAM, 0.48 | FR2.120-1 | TDLA30-300 | 2x4, ULA Medium | 70 | TBD |
| 1-4 | R.PDSCH.5-2.2 TDD | 40/30 | 16QAM, 0.48 | FR2.120-1 | TDLA30-300 | 2x4, ULA Medium | 70 | TBD |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent Note 2: Correlation matrix and antenna configuration parameters apply to each of TRxP #1 and TRxP #2 Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 [3] with scaling factor as 1/sqrt(2) for transmitted signal from each TRxP |  |  |  |  |  |  |  |  |

Proposal 6: Consider PDSCH test cases for mDCI with full overlapping resource allocation as follow

Test Parameters

| Parameter |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  | TCI State #1 | TCI State #2 |
|  |  | CORESETPoolIndex |  | 0,1 |  |
| Duplex mode |  |  |  | TDD |  |
| Active DL BWP index |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  | Type A |  |
|  | k0 |  |  | 0 |  |
|  | Starting symbol (S) |  |  | 2 |  |
|  | Length (L) |  |  | 12 |  |
|  | PRB bundling type |  |  | Static |  |
|  | PRB bundling size |  |  | 2 |  |
|  | Resource allocation type |  |  | Type 1 |  |
|  | RBG size |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  | {1000,1001} | {1002,1003} |
|  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |  |
| Resource allocation |  |  |  | Full overlapping |  |
| Timing offset of the second TRxP from the first TRxP |  |  | us | -0.0625 for test 1-1 0.25 for test 1-2 |  |
| Frequency offset of the second TRxP from the first TRxP |  |  | Hz | 3000 for test 1-1 0 for test 1-2 |  |
| Precoding configuration |  |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity |  |
| Note 1: PDSCH transmission is done from both TRxPs. Transmission from TRxP #1 uses CORESETPoolIndex 0 and transmission from TRxP #2 uses CORESETPoolIndex 1 |  |  |  |  |  |

Minimum performance

| Test num. | Reference channel |  | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition(Note 1) | Correlation matrix and antenna configuration(Note 2) | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB)(Note 3) |
|  | TRxP #1 | TRxP #2 |  |  |  |  |  |  |  |
| 1-1 | R.PDSCH.5-2.2 TDD | R.PDSCH.5-2.2 TDD | 100/120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-300 | 2x4, ULA Low | 70 | TBD |
| 1-2 | R.PDSCH.5-2.2 TDD | R.PDSCH.5-2.2 TDD | 100 / 120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-300 | 2x4, ULA Low | 70 | TBD |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent Note 2: Correlation matrix and antenna configuration parameters apply to each of TRxP #1 and TRxP #2 Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 [3] |  |  |  |  |  |  |  |  |  |

Proposal 7: Consider PDSCH test cases for mDCI with non-overlapping scheme as follow

Test Parameters

| Parameter |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  | TCI State #1 | TCI State #2 |
|  |  | CORESETPoolIndex |  | 0,1 |  |
| Duplex mode |  |  |  | TDD |  |
| Active DL BWP index |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  | Type A |  |
|  | k0 |  |  | 0 |  |
|  | Starting symbol (S) |  |  | 2 |  |
|  | Length (L) |  |  | 12 |  |
|  | PRB bundling type |  |  | Static |  |
|  | PRB bundling size |  |  | 2 |  |
|  | Resource allocation type |  |  | Type 1 |  |
|  | RBG size |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  | {1000,1001} | {1002,1003} |
|  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |  |
| Resource allocation |  |  |  | Non-overlapping |  |
| Timing offset of the second TRxP from the first TRxP |  |  | us | -0.0625 |  |
| Frequency offset of the second TRxP from the first TRxP |  |  | Hz | 3000 |  |
| Precoding configuration |  |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity |  |
| Note 1: PDSCH transmission is done from both TRxPs. Transmission from TRxP #1 uses CORESETPoolIndex 0 and transmission from TRxP #2 uses CORESETPoolIndex 1 |  |  |  |  |  |

Minimum performance

| Test num. | Reference channel |  | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition(Note 1) | Correlation matrix and antenna configuration(Note 2) | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB)(Note 3) |
|  | TRxP #1 | TRxP #2 |  |  |  |  |  |  |  |
| 1-1 | R.PDSCH.5-2.2 TDD | R.PDSCH.5-2.2 TDD | 100/120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-300 | 2x4, ULA High | 70 | TBD |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent Note 2: Correlation matrix and antenna configuration parameters apply to each of TRxP #1 and TRxP #2 Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 [3] |  |  |  |  |  |  |  |  |  |

# 3	Summary

In this paper we tried to contribute with the following proposals for further discussions:

Proposal 1: For mDCI, consider one port per TRxP for PTRS.

Proposal 2: Considering that the capability parameter ‘supportTwoPortDL-PTRS-r16’ is not mandatory (optional), we can consider only one PTRS port for sDCI SDM based scheme.

Proposal 3: For NR FR2-1 multi-Rx chain DL reception, consider sDCI SDM based scheme and mDCI with full or non-overlapping resource allocation scheme with timing and frequency offsets between TRxPs as follow

- Timing offset of the second TRxP from the first TRxP [us] as -0.0625 or 0.25.

- Frequency offset of the second TRxP from the first TRxP [Hz] as 3000 or 0.

Proposal 4: Define PDSCH Type A demodulation requirements for

- Single-DCI based SDM scheme using 1+1- and 2+2-layer combination

- Multi-DCI with full overlapping resource allocation using 2+2-layer combination

- Multi-DCI with non-overlapping resource allocation using 2+2-layer combination

Proposal 5: Consider PDSCH test cases for sDCI based SDM scheme as follow

Test Parameters

| Parameter |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  | TCI State #1 |  |
|  |  | CORESETPoolIndex |  | 0 |  |
| Duplex mode |  |  |  | TDD |  |
| Active DL BWP index |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  | Type A |  |
|  | k0 |  |  | 0 |  |
|  | Starting symbol (S) |  |  | 2 |  |
|  | Length (L) |  |  | 12 |  |
|  | PRB bundling type |  |  | Static |  |
|  | PRB bundling size |  |  | 2 |  |
|  | Resource allocation type |  |  | Type 1 |  |
|  | RBG size |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  | 1000 | 1002 |
|  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |  |
| Resource allocation |  |  |  | Full-overlappling |  |
| Timing offset of the second TRxP from the first TRxP |  |  | us | -0.0625 for test 1-1 and test 1-3 0.25 for test 1-2 and test 1-4 |  |
| Frequency offset of the second TRxP from the first TRxP |  |  | Hz | 3000 for test 1-1 and test 1-3 0 for test 1-2 and test 1-4 |  |
| Precoding configuration |  |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity. |  |
| Note 1: PDSCH transmission is done from both TRxPs (PDSCH Layer 0 is transmitted from TRxP #1 and PDSCH layer 1 is transmitted from TRxP #2) Note 2: PDSCH transmission is done from both TRxPs (PDSCH Layers 0 and 1 are transmitted from TRxP #1 and PDSCH layers 2 and 3 are transmitted from TRxP #2) |  |  |  |  |  |

Minimum performance

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition (Note 1) | Correlation matrix and antenna configuration (Note 2) | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) (Note 3) |
| 1-1 | R.PDSCH.5-3.2 TDD | 40 / 30 | 64QAM, 0.43 | FR2.120-1 | TDLA30-300 | 2x4, ULA Low | 70 | TBD |
| 1-2 | R.PDSCH.5-3.2 TDD | 40 / 30 | 64QAM, 0.43 | FR2.120-1 | TDLA30-300 | 2x4, ULA Low | 70 | TBD |
| 1-3 | R.PDSCH.5-2.2 TDD | 40/30 | 16QAM, 0.48 | FR2.120-1 | TDLA30-300 | 2x4, ULA Medium | 70 | TBD |
| 1-4 | R.PDSCH.5-2.2 TDD | 40/30 | 16QAM, 0.48 | FR2.120-1 | TDLA30-300 | 2x4, ULA Medium | 70 | TBD |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent Note 2: Correlation matrix and antenna configuration parameters apply to each of TRxP #1 and TRxP #2 Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 [3] with scaling factor as 1/sqrt(2) for transmitted signal from each TRxP |  |  |  |  |  |  |  |  |

Proposal 6: Consider PDSCH test cases for mDCI with full overlapping resource allocation as follow

Test Parameters

| Parameter |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  | TCI State #1 | TCI State #2 |
|  |  | CORESETPoolIndex |  | 0,1 |  |
| Duplex mode |  |  |  | TDD |  |
| Active DL BWP index |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  | Type A |  |
|  | k0 |  |  | 0 |  |
|  | Starting symbol (S) |  |  | 2 |  |
|  | Length (L) |  |  | 12 |  |
|  | PRB bundling type |  |  | Static |  |
|  | PRB bundling size |  |  | 2 |  |
|  | Resource allocation type |  |  | Type 1 |  |
|  | RBG size |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  | {1000,1001} | {1002,1003} |
|  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |  |
| Resource allocation |  |  |  | Full overlapping |  |
| Timing offset of the second TRxP from the first TRxP |  |  | us | -0.0625 for test 1-1 0.25 for test 1-2 |  |
| Frequency offset of the second TRxP from the first TRxP |  |  | Hz | 3000 for test 1-1 0 for test 1-2 |  |
| Precoding configuration |  |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity |  |
| Note 1: PDSCH transmission is done from both TRxPs. Transmission from TRxP #1 uses CORESETPoolIndex 0 and transmission from TRxP #2 uses CORESETPoolIndex 1 |  |  |  |  |  |

Minimum performance

| Test num. | Reference channel |  | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition(Note 1) | Correlation matrix and antenna configuration(Note 2) | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB)(Note 3) |
|  | TRxP #1 | TRxP #2 |  |  |  |  |  |  |  |
| 1-1 | R.PDSCH.5-2.2 TDD | R.PDSCH.5-2.2 TDD | 100/120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-300 | 2x4, ULA Low | 70 | TBD |
| 1-2 | R.PDSCH.5-2.2 TDD | R.PDSCH.5-2.2 TDD | 100 / 120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-300 | 2x4, ULA Low | 70 | TBD |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent Note 2: Correlation matrix and antenna configuration parameters apply to each of TRxP #1 and TRxP #2 Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 [3] |  |  |  |  |  |  |  |  |  |

Proposal 7: Consider PDSCH test cases for mDCI with non-overlapping scheme as follow

Test Parameters

| Parameter |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  | TCI State #1 | TCI State #2 |
|  |  | CORESETPoolIndex |  | 0,1 |  |
| Duplex mode |  |  |  | TDD |  |
| Active DL BWP index |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  | Type A |  |
|  | k0 |  |  | 0 |  |
|  | Starting symbol (S) |  |  | 2 |  |
|  | Length (L) |  |  | 12 |  |
|  | PRB bundling type |  |  | Static |  |
|  | PRB bundling size |  |  | 2 |  |
|  | Resource allocation type |  |  | Type 1 |  |
|  | RBG size |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  | {1000,1001} | {1002,1003} |
|  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |  |
| Resource allocation |  |  |  | Non-overlapping |  |
| Timing offset of the second TRxP from the first TRxP |  |  | us | -0.0625 |  |
| Frequency offset of the second TRxP from the first TRxP |  |  | Hz | 3000 |  |
| Precoding configuration |  |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity |  |
| Note 1: PDSCH transmission is done from both TRxPs. Transmission from TRxP #1 uses CORESETPoolIndex 0 and transmission from TRxP #2 uses CORESETPoolIndex 1 |  |  |  |  |  |

Minimum performance

| Test num. | Reference channel |  | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition(Note 1) | Correlation matrix and antenna configuration(Note 2) | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB)(Note 3) |
|  | TRxP #1 | TRxP #2 |  |  |  |  |  |  |  |
| 1-1 | R.PDSCH.5-2.2 TDD | R.PDSCH.5-2.2 TDD | 100/120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-300 | 2x4, ULA High | 70 | TBD |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent Note 2: Correlation matrix and antenna configuration parameters apply to each of TRxP #1 and TRxP #2 Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 [3] |  |  |  |  |  |  |  |  |  |

# References

- R4-2306000, “WF on UE demodulation and CSI requirements for FR2 multi-Rx DL reception”, Qualcomm Inc.

- 3GPP TS 38.101-4, “NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements”, v.17.8.0.