3GPP TSG-RAN WG4 Meeting #107                                          R4-2308949

Incheon, Korea, May 22 – 26, 2023

Source: 	Huawei, HiSilicon

Title: 	On enhancements of increasing UE power high limit for CA and DC

Agenda Item:	8.28.1

Document for:	Approval

Introduction

In previous meeting, opinions on whether and how to consider SAR issue mitigation enhancement have been further exchanged among companies. The FFS points from WF [1] are excerpted as below:

- Issue 1: Whether to introduce PHR reporting for the carrier that is configured for DL but no UL < Agreement >: FFS whether to introduce PHR reporting for the carrier that is configured for DL but no UL. - The target scenario and details like how to calculate the PHR are encouraged to provide by proponent in the next meeting. - Issue 2: Proposals based on power class related information reporting for FR1 carrier < Agreement >: FFS on the following power class related information reporting solutions for FR1 carrier: - Option 1: Power class being used by the UE instead of ΔPPowerClass. - Option 2: Report power-class fallback ΔPPowerClass in the PHR per serving cell using spare bits in the existing PHR MAC-CE; reporting by UEs implementing SAR mitigation by P-MPR and duty cycle averaging (the ‘P-MPR method’) with finer granularity than the ΔPPowerClass is supported using the said spare bits of the PHR in combination with the P-bit. - Power-capability change, power-class fallback or return to declared power class, can trigger an aperiodic report. - Report power-class fallback ΔPPowerClass,CA in the multi-entry PHR for the BC; any BC power-class change, fallback or return to advertised BC power class, also trigger an aperiodic PHR. - Option 3: Report both power-class fallback ΔPPowerClass and the full power transmission capability for the current fall-back power class. - Issue 3: Whether to introduce P-MPR reporting for FR1 carrier < Agreement >: FFS whether to introduce P-MPR reporting for FR1 carrier. - Issue 4: Proposals based on ‘duration’ information reporting for FR1 carrier < Agreement >: FFS whether to consider the following duration related information reporting solutions for FR1 carrier. - Option 1: On top of the power class related information reporting, also consider report evaluation period, the starting time and estimated time for return to higher power class. - Option 2: Consider UE reporting on the following information as a ‘forecast’. - For current UEs, information about how long the gNB should not expect better performance: - For UEs that fall-back in power class: how long a duty-cycle related power class fallback is expected to persist. - For UEs that use P-MPR: how long the reported range of P-MPR is expected to persist. - For all enhanced UEs, information about how long the gNB can expect un-degraded performance: - How long a UE can execute UL grants based on duty-cycle capability and last reported Pcmax. - Alternatively, how long a duty cycle can be sustained without triggering additional P-MPR. - Option 3: Sustainable duty cycle.

In this contribution, we would like to share our views regarding the remaining issues.

Discussion

On the “PHR reporting” for the carrier that is configured for DL but no UL

This is one aspect that has been proposed according to the contributions [2, 3] from previous meetings. Despite the details about what specifically should be reported and how the whole reporting scheme works etc., are still shrouded in mystery, the motivation has been proposed that for specific antenna arrangement, the RF exposure capability for different antenna (may cover different CC) may be different, which can be well monitored from UE but not from gNB perspective. The claimed benefit is that the gNB could make more informed scheduling decision by enabling such UE reporting on the RF exposure capability in case better UL performance can be provided especially for the un-configured UL CCs. We think at least the following information from proponent, if possible, can be constructive for the determination on whether that is a reasonable evolution avenue for UL coverage enhancement:

- Since it has been claimed that such demand is rooted from various RF exposure capability of UE, then the target scenario under specific UE architecture (e.g., example band combination of intra/inter-band CA with specific antenna arrangement) should be provided.

Only based on the above information, the benefits comparing to the existing specified solution can be further justified. For instance, SRS carrier switching with type 3 PHR reporting could be a solution which is supported by specification to fulfil such purpose.

Observation 1: It seems that one specified solution, which is SRS carrier switching with type 3 PHR reporting, can meet the measurement and reporting demand.

Besides, how the gain would be weighed considering that it is still gNB decision on whether to configure UL CC to another band or not and the time consumption of the RRC reconfiguration procedure comparing to the time-variant of RF exposure, needs to be further discussed.

Observation 2: It is not clear how the advertised gain would be weighed considering that it is still gNB decision on whether to configure UL CC to another band or not and the time consumption of the RRC reconfiguration procedure comparing to the time-variant of RF exposure.

Proposal 1: Regarding “PHR reporting for DL CC”, at least the following aspects should be clarified:

- Whether SRS carrier switching with type 3 PHR reporting can meet the same measurement and reporting demand.

- The target scenario under specific UE architecture e.g., example band combination of intra/inter-band CA with specific antenna arrangement should be provided along with the details of such UE reporting to prove the potential gain since it has been claimed that such demand is rooted from various RF exposure capability of antenna.

On the proposals based on power class related information reporting for FR1 carrier

For this part, the identified issue is that the gNB may have limited information on power class fall-back of a HPUE under the current specification design for maxDutyCycle. The following content of TS 38.101-1 can be referenced as a start to better grasp the root cause of this issue.

If a UE supports a different power class than the default UE power class for the band and the supported power class enables the higher maximum output power than that of the default power class: - if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 50% (The exact evaluation period is no less than one radio frame); or - if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or - if the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and half the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or - if the IE P-Max as defined in TS 38.331 [7] is provided and set to the maximum output power of the default power class or lower; - shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified in clause 6.2.4; - else if the UE does not support a power class with higher maximum output power than PC2; or - if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 25% (The exact evaluation period is no less than one radio frame); or - if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 0.5*maxUplinkDutyCycle-PC2-FR1 (The exact evaluation period is no less than one radio frame); or - if the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or - if the IE P-Max as defined in TS 38.331 [7] is provided and set to the maximum output power of the power class 2 or lower; - shall apply all requirements for power class 2 to the supported power class and set the configured transmitted power as specified in clause 6.2.4; - else shall apply all requirements for the supported power class and set the configured transmitted power as specified in clause 6.2.4.

Based on the above description, when a HPUE finds out that the ratio of UL symbols over the total symbols within a certain evaluation period exceeds the indicated maxUplinkDutyCycle, power class fall-back would happen, i.e., such HPUE would set non-zero ΔPPowerClass to reduce the high limit for configured transmission power calculation. This behaviour is captured in another part of TS 38.101-1.

PCMAX_L,f,c ≤ PCMAX,f,c ≤ PCMAX_H,f,c with PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c, (PPowerClass – ΔPPowerClass) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) } PCMAX_H,f,c = MIN {PEMAX,c, PPowerClass – ΔPPowerClass } ΔPPowerClass = - 3 dB for a power class 2 capable UE or 6 dB for a power class 1.5 UE when P-max of 23 dBm or lower is indicated; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 50%; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or when the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and half the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame). - 3 dB for a power class 1.5 capable UE when P-max of between 23 dBm and 26 dB is indicated; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is between 25% and 50%; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is between maxUplinkDutyCycle-PC2-FR1 and maxUplinkDutyCycle-PC2-FR1/2 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or when the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame). - 3dB when the UE is configured with SUL configurations and the requirements of default power class are applied as specified in sub-clause 6.2C.1 on the band where UE indicates power class 2;

Note that PCMAX,f,c for a specific transmission occasion, which can be conveyed to network through PHR report, is determined by UE based on a floating range bounded by PCMAX_L,f,c and PCMAX_H,f,c. But from PHR report, the network cannot deduce the exact power class of which the UE shall apply the corresponding requirements.

Observation 3: When power class fall-back happens for a HPUE, the network cannot deduce the exact power class, of which such HPUE shall apply the corresponding requirements, by PHR report.

Consequently, misalignment could happen when power class fall-back happens for a HPUE since at least the following configurations can be different depending on the current power class.

Full power transmission mode

Considering a UE with dual PC2 PAs to support PC1.5 MOP, it can report ul-FullPwrMode1-r16 (mode 1) to facilitate full power transmission for PUSCH. When power class fall-back happens, it would be better to report ul-FullPwrMode-r16 (mode 0) instead since each Tx chain is capable of 26 dBm MOP. Because it would provide the flexibility for selecting the most suitable Tx chain rather than force the UE, especially for the one with non-coherent Tx chains’ implementation, to maintain both Tx chains activated.

This issue can be better illustrated if we further consider a 4Tx non-coherent UE with PC1.5 MOP and 2*PC3+2*PC2 PA configuration. Such UE can only report ul-FullPwrMode1-r16 to support full power transmission. But when power class is falling back to PC2, ul-FullPwrMode2-r16 and g0g1g3 TPMI groups (utilizing single PC2 Tx chain, dual PC3 Tx chains or other combination to realize PC2 transmission for PUSCH) could be better choice based on similar reasons as we explained in the previous paragraph.

Observation 4: The full power transmission capability of HPUE can be different from that under advertised power class when power class fall-back happens.

Low-MSD capability

In Rel-18, the low-MSD feature is under study and it is RAN4 consensus that there are benefits to allow UE report its low-MSD capability to help network make more reasonable scheduling decisions. It is worthy to note that from UE perspective the actual low-MSD capability can be different depending on the current MOP (with given DL/UL carrier frequencies and RB allocations) and may not be linearly decreasing when power class fall-back happens.

Observation 5: The low-MSD capability of HPUE can be different and may not be linearly decreasing when power class fall-back happens.

As proposed in [4], reusing PHR report process to enable UE report non-zero ΔPPowerClass could be a way to align the understanding of power class fall-back timing between gNB and UE. We think it would be a complete solution to also include the full power transmission mode capability for the current fall-back power class into the enhanced PHR report.

As for low-MSD capability, dynamic report will not be considered in this release according to the latest agreement in WF [5]. So let the network to be aware of power class fall-back and the pre-reported e.g., per power class low-MSD capability from UE can solve this issue to a certain extend.

However, we also understand that RAN4 may be lack of interests of this solution because maxDutyCycle has not been widely implemented yet, but the issue is identified based on the existing specification solution.

Proposal 2: For FR1 PHR enhancement, allow the UE to report ΔPPowerClass, in order to align the understanding of power class fall-back timing between gNB and UE, and the full power transmission capability for the current fall-back power class to guarantee more reasonable UL scheduling.

On whether to introduce P-MPR report for FR1

During the discussion, the concern on the necessity of introducing P-MPR report for FR1 has not been addressed yet. Comparing to FR2, we are still not convinced that it can reach similar or even severer risk of link failure due to P-MPR for FR1 carrier.

Proposal 3: Do not consider P-MPR report for FR1 carrier.

On the proposals based on ‘duration’ information reporting for FR1 carrier

Regarding Option 2, from UE perspective, it is hard to understand how UE could anticipate the time duration with unchanged P-MPR considering that the reason to cause P-MPR e.g., proximity of human tissue, may not be easy to forecast. From gNB perspective, whether such information, very likely to be UE specific, could be beneficial for scheduling without additional implementation complexity needs further clarification.

Proposal 4: Do not consider ‘duration’ information reporting.

Conclusion

In this contribution we discussed on the first objective of the Rel-18 further enhancement for NR UL coverage WI, we have the following observations and proposals:

Observation 1: It seems that one specified solution, which is SRS carrier switching with type 3 PHR reporting, can meet the measurement and reporting demand.

Observation 2: It is not clear how the advertised gain would be leveraged considering that it is still gNB decision on whether to configure UL CC to another band or not and the time consumption of the RRC reconfiguration procedure comparing to the time-variant of RF exposure.

Observation 3: When power class fall-back happens for a HPUE, the network cannot deduce the exact power class, of which such HPUE shall apply the corresponding requirements, by PHR report.

Observation 4: The full power transmission capability of HPUE can be different from that under advertised power class when power class fall-back happens.

Observation 5: The low-MSD capability of HPUE can be different and may not be linearly decreasing when power class fall-back happens.

Proposal 1: Regarding “PHR reporting for DL CC”, at least the following aspects should be clarified:

- Whether SRS carrier switching with type 3 PHR reporting can meet the same measurement and reporting demand.

- The target scenario under specific UE architecture e.g., example band combination of intra/inter-band CA with specific antenna arrangement should be provided along with the details of such UE reporting to prove the potential gain since it has been claimed that such demand is rooted from various RF exposure capability of antenna.

Proposal 2: For FR1 PHR enhancement, allow the UE to report ΔPPowerClass, in order to align the understanding of power class fall-back timing between gNB and UE, and the full power transmission capability for the current fall-back power class to guarantee more reasonable UL scheduling.

Proposal 3: Do not consider P-MPR report for FR1 carrier.

Proposal 4: Do not consider ‘duration’ information reporting.

- References

- R4-2306625, “WF on NR further coverage enhancement part 1”, Huawei, HiSilicon, 3GPP TSG-RAN4 Meeting #106bis-e, Electronic Meeting, April 17 – 26, 2023.

- R4-2300711, “On UE signaling to enhance ULCA and DC”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #106, Athens, Greece, February 27 – March 3, 2023.

- R4-2304605, “Draft LS on UE signaling to enhance UL reliability”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #106bis-e, Electronic Meeting, April 17 – 26, 2023.

- R4-2300753, Power-class fallback reporting in the PHR for improved scheduling and enhanced performance with and without the high-power limit, Ericsson, 3GPP TSG-RAN4 Meeting #106, Athens, Greece, February 27 – March 3, 2023.

- R4-2303695, “WF on study for lower MSD”, Huawei, HiSilicon, 3GPP TSG-RAN4 Meeting #106, Athens, Greece, February 27 – March 3, 2023.