3GPP TSG-RAN WG4 Meeting #107	    R4-2308973

Incheon, KR, May 22 – May 26, 2023

Agenda item:	8.22.1

Source:	OPPO

Title:	On general issue

Document for:	Approval

- Introduction

In 3GPP RAN#94e meeting, a new study item (SI) on AI/ML for NR air interface in Rel-18 [1] was agreed. According to the SID, the study will focus on the general framework, evaluations for three typical use cases and other aspects relate to specification impacts.

Use cases to focus on:

- Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

RAN4 scope in the SID is listed as below:

- Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2 - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable - Consider the need and implications for AI/ML processing capabilities definition

In this contribution, we will discuss the RAN4 related specific issues

- Discussion

2.1 General issue

Based on the progress in RAN1 and RAN2, as well as the RAN4 scope in the SID, it is necessary to:

- Focus on cases and issues with sufficient research progress and consensus in RAN1 and RAN2

For cases and issues that have not been fully discussed and agreed in RAN1 and RAN2, it is not appropriate to initiate discussions in RAN4

- Establish a common testing framework for AI/ML based features,

To support one-sided AI/ML model testing, two-sided AI/ML model testing, common LCM procedure, etc.

Within the common testing framework, the feasibility and methods of constructing sub use case level tests can be studied based on the characteristics and requirements of different sub use cases

- Studying performance requirement and core requirement at the sub use case level

Proposal 1: Based on the progress in RAN1 and RAN2, and the RAN4 scope in the SID, following aspects should be considered:

- Focus on cases and issues with sufficient research progress and consensus in RAN1 and RAN2

- Establish a common testing framework for AI/ML features

- Studying performance requirement and core requirement at the sub use case level

Figure 1: A common framework relates to RAN1 AI/ML discussion

Figure 1 summarizes a common framework relates to RAN1 R18 AI/ML discussions, from our point of view, in RAN4 study:

- Performance requirements and core requirements should focus on

Model inference(agreed in RAN4 #106bis)

LCM - model monitoring, model activation/deactivation/switch/fallback

Model monitoring is the procedure that UE/NW monitors the inference performance of the AI/ML model.

With model monitoring, UE or NW could evaluate whether a given AI/ML model is still feasible for a given scenario or configuration, and then trigger following actions, e.g. model activation/deactivation /switch/fallback.

In RAN1 discussion, model monitoring and related procedures are discussed in detail. In RAN4, we think tests on model monitoring and model activation/deactivation/switch/fallback should be studied as well, e.g. performance/core requirements on monitoring input/output metrics, monitoring accuracy, LCM latency, etc.

- RAN4 does not need to study requirements/tests for

Training data collection (agreed in RAN4#106bis)

AI/ML model update

According to RAN1 definition, model update is the procedure of updating the model parameters and/or model structure of a model. It includes model finetuning, retraining, and re-development via online/offline training, e.g. using one model whose parameters are flexibly updated as the scenario/configuration/site that the device experiences changes over time.

Based on the above definition, it can be seen that model update is some kind of model training, which is more challenging and complex, e.g. by online training/tuning. Considering we have agreed that model training is not included in the scope of RAN4 R18, there is no need to continue discussing model update in this release.

AI/ML model transfer/delivery

According to current definition, AI/ML model delivery/transfer is the procedure to delivery an AI/ML model from one entity to another entity. The difference between model delivery and transfer lies in whether the model delivery should be done through 3GPP air interface.

For AI/ML model delivery, model could be delivery in any manner.

For AI/ML model transfer, delivery of an AI/ML model should be done over the air interface

According to the definition, only model transfer may involve the impact in RAN, so model delivery does not require further discussion in RAN4.

Regarding the model transfer, except for the definition of term, there was no more conclusion formed in RAN1 until last meeting. From our understanding, we do not need to consider the study on model transfer in RAN4 until RAN1 or RAN2 R18 achieves sufficient progress.

Proposal 2: RAN4 does not need to study requirements/tests for AI/ML model delivery.

Proposal 3: RAN4 does not need to study requirements/tests for AI/ML model update, AI/ML model transfer.

Note: if other WG defines the model update procedure or model transfer procedure, RAN4 may need study to define the requirements for it.

For performance requirements and tests, it is necessary to clarify

- Whether the test is for a task(e.g. for CSI feedback, for BM, for positioning)? or for an AI/ML based solution to the task(e.g. for AI/ML based CSI feedback, AI/ML based BM, AI/ML based positioning)?

- Whether the test is for a generalized performance evaluation? or for a scenario-based performance evaluation?

Based on the different distinctions, 4 options are listed below which may lead to different follow-up research routes:

- Option 1: Task level test

AI/ML method is one of the candidate solutions for addressing tasks such as CSI, BM, and positioning in R18 and possibly in the following release. When considering the performance test of use cases in RAN4, no need to distinguish detailed solutions (such as AI/ML based solutions or non-AI/ML based solutions), only need to focus on the task level, e.g. measurement accuracy and performance requirement on task input/output.

- Option 2: Model level test

The solution to be tested should be an AI/ML based solution. The performance test focus on the model level, e.g. measurement accuracy and performance requirement on model input and output.

Note, model input/output may be different from the task input/output in some cases, e.g. consider the pre-process/post-process, the relationship between intermediate KPIs and system KPIs.

- Option 3: Test for a generalized performance

To conduct tests to ensure the basic performance of the AI/ML based solution is not lower than a baseline.

- Option 4: Test for a scenario-based performance

Considering test cases for AI/ML performance in different scenarios/configurations.

Proposal 4: For RAN4 AI/ML performance requirements and tests, following options should be considered,

Option 1: Task level test

Option 2: Model level test

Note, model input/output may be different from the task input/output in some cases, e.g. consider the pre-	process/post-process, the relationship between intermediate KPIs and system KPIs.

Option 3: Test for generalized performance

Option 4: Test for scenario-based performance

Compared to previous works in RAN4, performance and core requirements related to AI/ML cases have new characteristics in the following aspects, including

- Performance and core requirement on AI/ML life cycle management(LCM), e.g.

    - Regarding the performance metrics, intermediate KPIs and eventual KPIs should be considered for further study, e.g.

- Intermediate KPIs as monitoring metrics. e.g., SGCS, CSI prediction accuracy, BM prediction accuracy, AI assisted positioning measurement

- Eventual KPIs as monitoring metrics, e.g., Throughput, BLER, Positioning Accuracy

From our understanding, similar to the RLF/BFR procedures defined in previous releases, for the AI/ML LCM, eventual KPIs(e.g., hypothetical BLER) could be utilized for the performance monitoring. Other options can be used to equivalent convert into the eventual KPI by implementation if needed. For example, UE can estimate the system performance by measuring/monitoring some intermediate results, e.g. UE can estimate the change of BLER/ throughput through the measurement of SGCS in CSI cases.

    - Regarding the performance evaluation and decision-making, the stability of the performance evaluating and decision-making mechanism should be further studied to avoid the interference of random effects on the evaluation results.

Multiple attempts, e.g. within an evaluation window both in PHY and high layers, would be helpful to obtain a relatively stable evaluation result.

Multi-user involved AI/ML model performance test should be addressed as well, e.g. it is necessary to consider whether the model should be switched/updated if only a part of users report the AI/ML model is failed. If NW makes a model failure judgment until receiving model failure indications from a large amount of UE, it may cause a delayed model switch/update. While if NW directly switch/updates the AI/ML model after receiving a few users’ model failure indication, it may lead to ping-pong switching/updating of AI/ML models and cause unnecessary control overheads.

- Testability of two-sided model testing, e.g. in CSI compression,

    - CSI report: for two-sided models, the information transmitted through air-interface (e.g. CSI reports) may be generated by AI/ML models rather than that predefined by protocols. The impact of testability and interoperability may be different from that before.

    - Reference model: for two-sided model testing, TE may need a reference model (e.g. reference CSI decoder/decoder) to complete the testing process[2].

- Details of core requirement for sub use cases should follow the progress in RAN1/RAN2, and define detailed test methodologies accordingly

Proposal 5: Regarding the AI/ML life cycle management impacts and performance monitoring, following aspects should be studied.

- KPIs, including

- Eventual KPI (e.g., Throughput, BLER, Positioning accuracy)

- Intermediate KPI (e.g., SGCS, CSI prediction accuracy, BM prediction accuracy, AI assisted positioning measurement)

- Mechanisms to avoid the interference of random effects on the evaluation results, including

-  Multi-sample involved mechanism

-  Multi-user involved mechanism

- Conclusions

In this contribution, we discussed the Rel-18 AI/ML impacts to RAN4 and got following proposals

Proposal 1: Based on the progress in RAN1 and RAN2, and the RAN4 scope in the SID, following aspects should be considered:

- Focus on cases and issues with sufficient research progress and consensus in RAN1 and RAN2

- Establish a common testing framework for AI/ML features

- Studying performance requirement and core requirement at the sub use case level

Proposal 2: RAN4 does not need to study requirements/tests for AI/ML model delivery.

Proposal 3: RAN4 does not need to study requirements/tests for AI/ML model update, AI/ML model transfer.

Note: if other WG defines the model update procedure or model transfer procedure, RAN4 may need study to define the requirements for it.

Proposal 4: For RAN4 AI/ML performance requirements and tests, following options should be considered,

Option 1: Task level test

Option 2: Model level test

Note, model input/output may be different from the task input/output in some cases, e.g. consider the pre-	process/post-process, the relationship between intermediate KPIs and system KPIs.

Option 3: Test for generalized performance

Option 4: Test for scenario-based performance

Proposal 5: Regarding the AI/ML life cycle management impacts and performance monitoring, following aspects should be studied.

- KPIs, including

- Eventual KPI (e.g., Throughput, BLER, Positioning accuracy)

- Intermediate KPI (e.g., SGCS, CSI prediction accuracy, BM prediction accuracy, AI assisted positioning measurement)

- Mechanisms to avoid the interference of random effects on the evaluation results, including

-  Multi-sample involved mechanism

-  Multi-user involved mechanism

- References

- RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, RAN#94-e, Dec. 2021

- R4-2308975, On testability issues for two-sided AI/ML model, OPPO, RAN4#107, May 2023.