3GPP TSG-RAN WG4 Meeting #107	R4-2308993

Incheon, Korea, May 22 – May 26, 2023

Agenda item:	10.2.3

Source:            	Google Inc.

Title:                	Discussion on IE intraBandENDC-Support

Document for:	Approval

# Introduction

In RAN4#106, the inconsistency of two configurations Case 3 and Case 4 for intra-band EN-DC band combinations have continued being discussed. Case 3 is the intra-band contiguous DL EN-DC configuration with intra-band non-contiguous UL EN-DC configuration, and Case 4 is the intra-band non-contiguous DL EN-DC configuration, where LTE and NR adjacent DL carriers are contiguous, with intra-band contiguous and non-contiguous UL EN-DC configuration.

- Case 3: All CCs are contiguous in DL but neither carrier is contiguous to each other in UL:

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_(n)41AB DC_(n)41CA DC_(n)41DA | DC_41A_n41A |
| DC_(n)48CA | DC_48A_n48A |
| DC_(n)48DA | DC_48A_n48A |

- Case 4: LTE and NR adjacent carriers are contiguous but carriers in LTE or NR are non-contiguous, it will have two kinds of UL ENDC configurations:

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_48A_(n)48AA | DC_(n)48AA DC_48A_n48A |

According to the approved WF in RAN4#106, there are some wording refinement to the solution Option 2c for Case 3 and the solution Option 2b for Case 4. It is agreed that RAN4 can check if the updated solutions for Case 3 and Case 4 in the next meeting.

# Case 3 solutions/updates ## Update solution for Option 2c Update solution for Option 2c as below. - Option 2c:From Rel-16, UE shall also support non-contiguous operation in the DL (DC_48C_n48A or DC_48D_48A), then the network can configure DL_(n)48CA or DC_(n)48DA with the middle LTE cell DL-only and the UL with a gap (non-contiguous) <Agreement>: RAN4 to check if the updated Option 2c can be agreed in the next meeting. # Case 4 solutions/updates ## Update solution for Option 2b Update solution for Option 2b from the WF in R4-2220589 as below. - Option 2b: From Rel-16, bandcombinations of contiguous and non-contiguous intra-band EN-DC should be limited to two sub-blocks one of which consists of a contiguous EN-DC configuration in table Table 5.3B.0-1 in 38.101-3. For these the UE must support both contiguous and non-contiguous EN-DC in the UL, i.e. - UE indicate “both” capability for DL DC_48A_(n)48AA with UL DC_(n)48AA and UL DC_48A_n48A - The Case 4 is limited to one NR sub-block (band entry) with one or more E-UTRA sub-blocks, the intraBandENDC-Support still indicting the relation between any one of the E-UTRA sub-blocks (band entries) and the single NR sub-block - Check with RAN2 about option 2b and new signalling is not precluded for case 4. <Agreement>: RAN4 to check if the updated Option 2b can be agreed in the next meeting.

In addition, the issue to extend meaning of “both” in the IE intraBandENDC-Support for B48+n48 EN-DC in both Case 3 and Case 4 are classified into two options. Option 1 is to extended the meaning of “both” for the IE intraBandENDC-Support and Option 2 is to keep the existing RAN4 understanding of “both” in the RAN4 LS to RAN2.

Extend the meaning of ‘both’ in the IE intraBandENDC-Support for B48+n48 EN-DC in both case 3 and case 4 - Option 1: Extend the meaning of ‘both’ in the IE intraBandENDC-Support for B48+n48 EN-DC in both case 3 and case 4. - Option 2: Keep existing RAN4 understanding of ‘both’ as communicated in the RAN4 LS to RAN2 in R4-2220837. <Way forward>: Companies are requested to provide further input next meeting

Furthermore, in RAN4#106bis-e, the reply LS on intraBandENDC-Support from RAN2 is sent to RAN4. The new signaling intraBandENDC-Support-UL is proposed to differentiate intra-band DL/UL EN-DC configuration for Case#3 and Case#4. Seven scenarios are listed to differentiate each contiguous/non-contiguous DL EN-DC configuration with contiguous/non-contiguous UL EN-DC configuration.

| Scenario# | intraBandENDC-Support | intraBandENDC-Support-UL | UE supports in DL / UL (if applicable) |
| --- | --- | --- | --- |
| 1 | Absent (Contiguous) | Absent | - Contiguous/Contiguous |
| 2 | Absent (Contiguous) | Non-contiguous | - Contiguous/Non-contiguous NOTE: “Case 3” in R2-2300060 (R4-2220837) |
| 3 | Non-contiguous | Absent | - Non-contiguous/Non-contiguous |
| 4 | Both | Absent | - Contiguous/contiguous - Non-contiguous/Non-contiguous |
| 5 | Both | Non-contiguous | - Contiguous/Non-contiguous - Non-contiguous/Non-contiguous |
| 6 | Absent (Contiguous) | Both | - Contiguous/Contiguous - Contiguous/Non-contiguous NOTE: “Case 4” in R2-2300060 (R4-2220837) |
| 7 | Both | Both | - Contiguous/Contiguous - Non-contiguous/Non-contiguous - Contiguous/Non-contiguous |

In this paper, we would like to share our view in the following.

# Discussion

According to the approved WF in RAN4#106, Option 2c for Case 3 and Option 2b for Case 4 have some wording refinement about the solution should be adopted from Rel-16. The reason to adopt the solution from Rel-16 is that the Case 3 configurations for b41+n41 which are introduced from Rel-15 has been removed in the previous meeting, and both Case 3 and Case4 configurations for b48+n48 are introduced from Rel-16. Besides, another unsolved issue is whether to extend the meaning of “both” to make the solutions for Case 3 and Case 4 feasible. Furthermore, the Reply LS on intraBandENDC-Support from RAN2 in RAN4#106bis-e provide another view point by introducing a new IE intraBandENDC-Support-UL to differentiate contiguous or non-contiguous for both intra-band DL and UL EN-DC configuration.

By considering the above information, we summarize all supporting DL/UL EN-DC configuration in the following table into two methods “To extend the meaning of “both” in intraBandENDC-Support” and “To introduce IE intraBandENDC-Support-UL”. For the method to extend the meaning of “both”, it can be shown in the following table that red mark Case 3 and Case 4 configuration can be supported in Scenario#4 with intraBandENDC-Support=both by extending the meaning to include “contiguous(DL)/ non-contiguous(UL)” configuration. For the method to introduce IE intraBandENDC-Support-UL, it can be shown that the pure Case 3 can be supported in Scenario#2 by indicating intraBandENDC-Support=absent(contiguous) and intraBandENDC-Support-UL=non-contiguous and the pure Case 4 can be supported in Scenario#6 by intraBandENDC-Support=absent(contiguous) and intraBandENDC-Support-UL=both. Besides, Case 3 configuration can also be supported in Scenario#5/Scenario#6/Scenario#7 and Case 4 can be also supported in Scenario#7.

In our view, although both methods are feasible from implementation perspective, we still prefer to extend the meaning of “both” in IE intraBandENDC-Support. The reason is that it would impact less scenarios compared to introducing IE intraBandENDC-Support-UL and the implementation effort to update the in-market devices would also be smaller. However, the method to introduce IE intraBandENDC-Support-UL can avoid forward compatible issue if the band combination with more complicated scenario than Case 3 and Case 4 are introduced in the future. Since there are only several band combinations with Case 3 and Case 4 configurations in Rel-16 and Rel-17, the method to extend the meaning of “both” in IE intraBandENDC-Support would be more simple. Hence, we would like to propose a compromised solution as below:

Proposal 1: In Rel-16 and Rel-17, to extend the meaning of “both” in IE intraBandENDC-Support to include “contiguous(DL)/non-contiguous(UL)” for intra-band DL/UL EN-DC configuration.

Proposal 2: To introduce IE intraBandENDC-Support-UL from Rel-18.

Proposal 3: Send the reply LS to RAN2 in Annex.

| Scenario# | intraBandENDC-Support | intraBandENDC-Support-UL | Reporting EN-DC bandwidth class | Supported DL/UL EN-DC configuration |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | To extend the meaning of “both” in intraBandENDC-Support | To introduce IE intraBandENDC-Support-UL |
| 1 | Absent (Contiguous) | Absent | LTE-DL: 48C/48D | DC_(n)48CA with UL DC_(n)48AA DC_(n)48DA with UL DC_(n)48AA | DC_(n)48CA with UL DC_(n)48AA DC_(n)48DA with UL DC_(n)48AA |
|  |  |  | LTE-UL: 48A |  |  |
|  |  |  | NR-DL: n48A |  |  |
|  |  |  | NR-UL: n48A |  |  |
|  |  |  | LTE-DL: 48A-48A | DC_48A_(n)48AA with UL DC_(n)48AA | DC_48A_(n)48AA with UL DC_(n)48AA |
|  |  |  | LTE-UL: 48A |  |  |
|  |  |  | NR-DL: n48A |  |  |
|  |  |  | NR-UL: n48A |  |  |
| 2 | Absent (Contiguous) | Non-contiguous | LTE-DL: 48C/48D | X | DC_(n)48CA with UL DC_48A_n48A DC_(n)48DA with UL DC_48A_n48A [Case#3] |
|  |  |  | LTE-UL: 48A |  |  |
|  |  |  | NR-DL: n48A |  |  |
|  |  |  | NR-UL: n48A |  |  |
|  |  |  | LTE-DL: 48A-48A | X | DC_48A_(n)48AA with UL DC_48A_n48A |
|  |  |  | LTE-UL: 48A |  |  |
|  |  |  | NR-DL: n48A |  |  |
|  |  |  | NR-UL: n48A |  |  |
| 3 | Non-contiguous | Absent | LTE-DL: 48C/48D | DC_48C_n48A with UL DC_48A_n48A DC_48D_n48A with UL DC_48A_n48A | DC_48C_n48A with UL DC_48A_n48A DC_48D_n48A with UL DC_48A_n48A |
|  |  |  | LTE-UL: 48A |  |  |
|  |  |  | NR-DL: n48A |  |  |
|  |  |  | NR-UL: n48A |  |  |
|  |  |  | LTE-DL: 48A-48A | DC_48A-48A_n48A with UL DC_48A_n48A | DC_48A-48A_n48A with UL DC_48A_n48A |
|  |  |  | LTE-UL: 48A |  |  |
|  |  |  | NR-DL: n48A |  |  |
|  |  |  | NR-UL: n48A |  |  |
| 4 | Both | Absent | LTE-DL: 48C/48D | DC_(n)48CA with UL DC_(n)48AA DC_(n)48DA with UL DC_(n)48AA DC_48C_n48A with UL DC_48A_n48A DC_48D_n48A with UL DC_48A_n48A DC_(n)48CA with UL DC_48A_n48A DC_(n)48DA with UL DC_48A_n48A [Case#3] | DC_(n)48CA with UL DC_(n)48AA DC_(n)48DA with UL DC_(n)48AA DC_48C_n48A with UL DC_48A_n48A DC_48D_n48A with UL DC_48A_n48A |
|  |  |  | LTE-UL: 48A |  |  |
|  |  |  | NR-DL: n48A |  |  |
|  |  |  | NR-UL: n48A |  |  |
|  |  |  | LTE-DL: 48A-48A | DC_48A_(n)48AA with UL DC_(n)48AA DC_48A_(n)48AA with UL DC_48A_n48A [Case#4] DC_48A-48A_n48A with UL DC_48A_n48A | DC_48A_(n)48AA with UL DC_(n)48AA DC_48A-48A_n48A with UL DC_48A_n48A |
|  |  |  | LTE-UL: 48A |  |  |
|  |  |  | NR-DL: n48A |  |  |
|  |  |  | NR-UL: n48A |  |  |
| 5 | Both | Non-contiguous | LTE-DL: 48C/48D | X | DC_(n)48CA with UL DC_48A_n48A DC_(n)48DA with UL DC_48A_n48A [Case#3] DC_48C_n48A with UL DC_48A_n48A DC_48D_n48A with UL DC_48A_n48A |
|  |  |  | LTE-UL: 48A |  |  |
|  |  |  | NR-DL: n48A |  |  |
|  |  |  | NR-UL: n48A |  |  |
|  |  |  | LTE-DL: 48A-48A | X | DC_48A_(n)48AA with UL DC_48A_n48A DC_48A-48A_n48A with UL DC_48A_n48A |
|  |  |  | LTE-UL: 48A |  |  |
|  |  |  | NR-DL: n48A |  |  |
|  |  |  | NR-UL: n48A |  |  |
| 6 | Absent (Contiguous) | Both | LTE-DL: 48C/48D | X | DC_(n)48CA with UL DC_(n)48AA DC_(n)48DA with UL DC_(n)48AA DC_(n)48CA with UL DC_48A_n48A DC_(n)48DA with UL DC_48A_n48A [Case#3] |
|  |  |  | LTE-UL: 48A |  |  |
|  |  |  | NR-DL: n48A |  |  |
|  |  |  | NR-UL: n48A |  |  |
|  |  |  | LTE-DL: 48A-48A | X | DC_48A_(n)48AA with UL DC_(n)48AA DC_48A_(n)48AA with UL DC_48A_n48A [Case#4] |
|  |  |  | LTE-UL: 48A |  |  |
|  |  |  | NR-DL: n48A |  |  |
|  |  |  | NR-UL: n48A |  |  |
| 7 | Both | Both | LTE-DL: 48C/48D | X | DC_(n)48CA with UL DC_(n)48AA DC_(n)48DA with UL DC_(n)48AA DC_(n)48CA with UL DC_48A_n48A DC_(n)48DA with UL DC_48A_n48A [Case#3] DC_48C_n48A with UL DC_48A_n48A DC_48D_n48A with UL DC_48A_n48A |
|  |  |  | LTE-UL: 48A |  |  |
|  |  |  | NR-DL: n48A |  |  |
|  |  |  | NR-UL: n48A |  |  |
|  |  |  | LTE-DL: 48A-48A | X | DC_48A_(n)48AA with UL DC_(n)48AA DC_48A_(n)48AA with UL DC_48A_n48A [Case#4] DC_48A-48A_n48A with UL DC_48A_n48A |
|  |  |  | LTE-UL: 48A |  |  |
|  |  |  | NR-DL: n48A |  |  |
|  |  |  | NR-UL: n48A |  |  |

Table 1. Supporting DL/UL EN-DC configuration for b48+n48 intra-band EN-DC configuration

# Conclusion

The observations and proposals in this contribution are summarized in the following.

Proposal 1: In Rel-16 and Rel-17, to extend the meaning of “both” in IE intraBandENDC-Support to include “contiguous(DL)/non-contiguous(UL)” for intra-band DL/UL EN-DC configuration.

Proposal 2: To introduce IE intraBandENDC-Support-UL from Rel-18.

Proposal 3: Send the reply LS to RAN2 in Annex.

# Reference

- R4-2303635, “WF on intra-band EN-DC support capability”, RAN4#106, AT&T

- R4-2306665, “Reply LS on intraBandENDC-Support”, RAN4#106bis-e, QualComm

- R4-2302067, “Discussion on Intra-Band EN-DC band combinations”, RAN4#106, Google Inc.

# Annex

3GPP TSG-RAN WG4 Meeting #107	R4-2308994

Incheon, Korea, May 22 – May 26, 2023

Title:	[Draft] Reply LS on intraBandENDC-Support

Response to:	R2-2304431(R4-2306665)

Release:	Rel-16

Work Item:	TEI16

Source:	RAN4

To:	RAN2

Cc:

Contact Person:

Name: Clement Huang

E-mail Address: clementhuang@google.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN4 would like to thank RAN2 for the Reply LS on intraBandENDC-Support.

RAN4 has discussed the solutions for Case 3 and Case 4 and make the decisions as below.

- In Rel-16 and Rel-17, to extend the meaning of “both” in IE intraBandENDC-Support by including “contiguous(DL)/non-contiguous(UL)” for intra-band DL/UL EN-DC configuration

- In Rel-18, to introduce IE intraBandENDC-Support-UL.

The supported intra-band DL/UL EN-DC configuration for Case 3 and Case 4 can be referred to the following table.

| Scenario# | intraBandENDC-Support | intraBandENDC-Support-UL | Reporting EN-DC bandwidth class | Supported DL/UL EN-DC configuration |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | To extend the meaning of “both” in intraBandENDC-Support | To introduce IE intraBandENDC-Support-UL |
| 1 | Absent (Contiguous) | Absent | LTE-DL: 48C/48D | DC_(n)48CA with UL DC_(n)48AA DC_(n)48DA with UL DC_(n)48AA | DC_(n)48CA with UL DC_(n)48AA DC_(n)48DA with UL DC_(n)48AA |
|  |  |  | LTE-UL: 48A |  |  |
|  |  |  | NR-DL: n48A |  |  |
|  |  |  | NR-UL: n48A |  |  |
|  |  |  | LTE-DL: 48A-48A | DC_48A_(n)48AA with UL DC_(n)48AA | DC_48A_(n)48AA with UL DC_(n)48AA |
|  |  |  | LTE-UL: 48A |  |  |
|  |  |  | NR-DL: n48A |  |  |
|  |  |  | NR-UL: n48A |  |  |
| 2 | Absent (Contiguous) | Non-contiguous | LTE-DL: 48C/48D | X | DC_(n)48CA with UL DC_48A_n48A DC_(n)48DA with UL DC_48A_n48A [Case#3] |
|  |  |  | LTE-UL: 48A |  |  |
|  |  |  | NR-DL: n48A |  |  |
|  |  |  | NR-UL: n48A |  |  |
|  |  |  | LTE-DL: 48A-48A | X | DC_48A_(n)48AA with UL DC_48A_n48A |
|  |  |  | LTE-UL: 48A |  |  |
|  |  |  | NR-DL: n48A |  |  |
|  |  |  | NR-UL: n48A |  |  |
| 3 | Non-contiguous | Absent | LTE-DL: 48C/48D | DC_48C_n48A with UL DC_48A_n48A DC_48D_n48A with UL DC_48A_n48A | DC_48C_n48A with UL DC_48A_n48A DC_48D_n48A with UL DC_48A_n48A |
|  |  |  | LTE-UL: 48A |  |  |
|  |  |  | NR-DL: n48A |  |  |
|  |  |  | NR-UL: n48A |  |  |
|  |  |  | LTE-DL: 48A-48A | DC_48A-48A_n48A with UL DC_48A_n48A | DC_48A-48A_n48A with UL DC_48A_n48A |
|  |  |  | LTE-UL: 48A |  |  |
|  |  |  | NR-DL: n48A |  |  |
|  |  |  | NR-UL: n48A |  |  |
| 4 | Both | Absent | LTE-DL: 48C/48D | DC_(n)48CA with UL DC_(n)48AA DC_(n)48DA with UL DC_(n)48AA DC_48C_n48A with UL DC_48A_n48A DC_48D_n48A with UL DC_48A_n48A DC_(n)48CA with UL DC_48A_n48A DC_(n)48DA with UL DC_48A_n48A [Case#3] | DC_(n)48CA with UL DC_(n)48AA DC_(n)48DA with UL DC_(n)48AA DC_48C_n48A with UL DC_48A_n48A DC_48D_n48A with UL DC_48A_n48A |
|  |  |  | LTE-UL: 48A |  |  |
|  |  |  | NR-DL: n48A |  |  |
|  |  |  | NR-UL: n48A |  |  |
|  |  |  | LTE-DL: 48A-48A | DC_48A_(n)48AA with UL DC_(n)48AA DC_48A_(n)48AA with UL DC_48A_n48A [Case#4] DC_48A-48A_n48A with UL DC_48A_n48A | DC_48A_(n)48AA with UL DC_(n)48AA DC_48A-48A_n48A with UL DC_48A_n48A |
|  |  |  | LTE-UL: 48A |  |  |
|  |  |  | NR-DL: n48A |  |  |
|  |  |  | NR-UL: n48A |  |  |
| 5 | Both | Non-contiguous | LTE-DL: 48C/48D | X | DC_(n)48CA with UL DC_48A_n48A DC_(n)48DA with UL DC_48A_n48A [Case#3] DC_48C_n48A with UL DC_48A_n48A DC_48D_n48A with UL DC_48A_n48A |
|  |  |  | LTE-UL: 48A |  |  |
|  |  |  | NR-DL: n48A |  |  |
|  |  |  | NR-UL: n48A |  |  |
|  |  |  | LTE-DL: 48A-48A | X | DC_48A_(n)48AA with UL DC_48A_n48A DC_48A-48A_n48A with UL DC_48A_n48A |
|  |  |  | LTE-UL: 48A |  |  |
|  |  |  | NR-DL: n48A |  |  |
|  |  |  | NR-UL: n48A |  |  |
| 6 | Absent (Contiguous) | Both | LTE-DL: 48C/48D | X | DC_(n)48CA with UL DC_(n)48AA DC_(n)48DA with UL DC_(n)48AA DC_(n)48CA with UL DC_48A_n48A DC_(n)48DA with UL DC_48A_n48A [Case#3] |
|  |  |  | LTE-UL: 48A |  |  |
|  |  |  | NR-DL: n48A |  |  |
|  |  |  | NR-UL: n48A |  |  |
|  |  |  | LTE-DL: 48A-48A | X | DC_48A_(n)48AA with UL DC_(n)48AA DC_48A_(n)48AA with UL DC_48A_n48A [Case#4] |
|  |  |  | LTE-UL: 48A |  |  |
|  |  |  | NR-DL: n48A |  |  |
|  |  |  | NR-UL: n48A |  |  |
| 7 | Both | Both | LTE-DL: 48C/48D | X | DC_(n)48CA with UL DC_(n)48AA DC_(n)48DA with UL DC_(n)48AA DC_(n)48CA with UL DC_48A_n48A DC_(n)48DA with UL DC_48A_n48A [Case#3] DC_48C_n48A with UL DC_48A_n48A DC_48D_n48A with UL DC_48A_n48A |
|  |  |  | LTE-UL: 48A |  |  |
|  |  |  | NR-DL: n48A |  |  |
|  |  |  | NR-UL: n48A |  |  |
|  |  |  | LTE-DL: 48A-48A | X | DC_48A_(n)48AA with UL DC_(n)48AA DC_48A_(n)48AA with UL DC_48A_n48A [Case#4] DC_48A-48A_n48A with UL DC_48A_n48A |
|  |  |  | LTE-UL: 48A |  |  |
|  |  |  | NR-DL: n48A |  |  |
|  |  |  | NR-UL: n48A |  |  |

RAN4 respectfully asks RAN2 to take the above decisions into account to update RAN2 specifications.

2. Actions:

To RAN2:

ACTION: RAN4 respectfully asks RAN2 to take the above decisions into account to update RAN2 specifications.

3. Date of Next TSG WG RAN4 Meetings:

TSG-RAN WG4 Meeting #108                  21st August – 25th August 2023    Toulouse, FR

TSG-RAN WG4 Meeting #108bis               9st October – 13th October 2023    Xiamen, CN