3GPP TSG-RAN WG4 Meeting # 107										R4-2309004

Incheon, KR, May 22 – May 26, 2023

Title: 	Discussion on CA_n5-n8

Source: 	Xiaomi

Agenda item:	7.30.2.1

Document for:	Approval

- Introduction

In RAN4#106bis-e meeting, an LS [1] was agreed regarding whether non-simultaneous n5 DL and n8 UL has RAN2 impact. Also RAN4 approved a WF to capture some way forwards and remaining issues. This contribution will provide our view on the remaining issues.

- Discussion

## Signaling of support for different Band combinations options Way forward/Agreement on capturing band combinations to be specified and potential signaling needs: following options are discussed as potential signaling solutions: - Option 1: use different BCS with notes clarifying the UL restrictions based on agreed cases to be specified NR CA configuration; Uplink CA configuration or single uplink carrier; NR Band; Bandwidth combination set. CA_n5A-n8A; n5X; n5; 0. n8. CA_n5A-n8AY; n5; 1. n8. CA_n5A-n8AZ; n5; 2. n8. NOTE X: In BCS0, n8 UL is not supported NOTE Y: In BCS1, the concurrent operation of band n5 DL and band n8 UL is not supported NOTE Z: In BCS2 the band n8 support is restricted to the 904-915MHz frequency range in UL. - Channel bandwidth support per UL configuration case is FFS (some case can be above 10MHz CBW) - Option 2: Optional capability is needed for option 2: CA_n5-n8 2UL non-simultaneous n5Rx/n8Tx - Option 3: Frequency restriction or New frequency band (for n8) is needed for option 3: CA_n5-n8 2UL with n8 restricted to 904-915MHz - Option 4: Other are not precluded - Options are not necessarily exclusive

In the WF, the solution with BCS to indicate different CA configurations is proposed. In the last meeting, RAN4 has agreed on an LS sent to RAN4 and is waiting for RAN2 response. The feasibility of 2UL/1DL CA configuration should be firstly evaluated by RAN2 and if feasible, then the solution or impact on RAN2 spec will be introduced. Currently, RAN4 does not reach consensus that all options should be considered for CA_n5-n8. The WF also mentioned the down selecion is needed in this meeting.

On the other hand, the approach with BCS seems not appropriate to indicate different solutions to supporting CA_n5-n8. In the current specification, BCS, i.e., Bandwidth Combination Set, is used to indicate different channel bandwidth set for each band in a band combination rather than different solutions to support a band combination. If RAN4 agrees on such three options, other signaling for different solutions to supporting CA_n5-n8 can be considered in RAN2.

Observation: Currently the feasibility of 2UL/1DL CA configuration is not confirmed by RAN2 and the approach with BCS seems not appropriate to indicate different solutions to supporting CA_n5-n8. If RAN4 agrees on such three options, other signaling for different solutions to supporting CA_n5-n8 can be considered in RAN2.

- Conclusion

This contribution gives our views on CA_n5-n8. The following observation are provided:

Observation: Currently the feasibility of 2UL/1DL CA configuration is not confirmed by RAN2 and the approach with BCS seems not appropriate to indicate different solutions to supporting CA_n5-n8. If RAN4 agrees on such three options, other signaling for different solutions to supporting CA_n5-n8 can be considered in RAN2.

- Reference

- R4-2306465, LS on non-simultaneous UL and DL from different two bands during UL CA, Nokia, RAN4#106bis-e

- R4-2306467, WF on CA_n5-n8, Skyworks, RAN4#106bis-e