3GPP RAN4 Meeting #107	R4-2309019

# Incheon, Korea, May 22nd – 26th, 2023

Title: 	TP to TR 38.858 feasibility of and impact on FR2 UE RF requirements

Agenda Item:	8.20.2.2.4

Source: 	Qualcomm Incorporated

Document for:	Approval

# 1      Introduction

In this document we propose text to capture some of the agreements that have been made in previous meetings and discussions.

In the last meeting, the skeleton of RAN4 part for TR 38.858 was agreed as follows:

| No. | Section for TR 38.858 |
| --- | --- |
| 1 | 10.1 Background for analysis |
| 2 | 10.2 Feasibility of FR1 Wide Area BS aspects |
| 3 | 10.3 Feasibility of FR1 Medium Range BS aspects |
| 4 | 10.4 Feasibility of FR1 Local Area BS aspects |
| 5 | 10.5 Feasibility of FR2 BS aspects |
| 6 | 10.6. FR1 Feasibility of UE aspects |
| 7 | 10.7 FR2 Feasibility of UE aspects |
| 8 | 10.8 Summary |
| 9 | 11.1 Impact on BS RF requirements |
| 10 | 11.2 Impact on UE RF requirements |
| 11 | 12 Adjacent channel co-existence evaluation results |
| 12 | 13 Regulatory aspects for deploying the duplex enhancements in TDD unpaired spectrum |
| 13 | Annex <D>: Adjacent channel co-existence evaluation |

This proposal updates the TR with some of the WF agreements from the last meeting.

# 2      References

R4-2305969, WF for SBFD feasibility study and requirements impact from UE aspect R4-2305971, TP to TR 38.858 feasibility of and impact on FR2 UE RF requirements

# 3      Text proposal

Begin changed section ******************************************************

10.7 FR2-1 Feasibility of UE aspects

10.7.1    Interference analysis

10.7.1.1  UE-UE co-channel inter-subband CLI modeling

Editor's note: This section captures the CLI modeling.

10.7.1.1.1 Receiver aspects

Existing co-channel UE RX performance requirements

For legacy UEs, the current UE RF architecture can be assumed without any RF architecture modification.

Currently there are no RF requirements for UE co-channel Rx performance.

Sub-band filtering and legacy UEs

For legacy UEs, no sub-band filtering is implemented, and therefore RAN4 has not assumed any subband filtering.

Sub-band filtering for a SBFD-aware UEs – UEs with a new feature

SBFD-aware UEs support half duplex operation while receiving the sub-band configuration from the network. For this kind of UE, the RF architecture and applicable RF requirements need further study for future releases. In the present study, legacy UEs are the main focus.

For new SBFD aware UEs, we need further study on whether sub-filtering can be considered or not. The subband/in-channel selectivity needs further study.

.

Thermal self-noise aspects (both adjacent channel and co-channel)

RAN4 decided on a simple fixed-value noise figure model for the UE receiver. Generally, the receiver noise figure will vary with the input power level, however the single value noise figure model was considered sufficient for the purpose of system studies for SBFD. RAN4 decided on a NF of 10dB.

Subband in-channel selectivity

It is worth noting that the RF degradations can cause inter-subband interference . We analyzed the design of the FR2-1 receiver. There are multiple considerations in the receiver design … for example residual sideband, reciprocal mixing, integrated phase noise, IM3 distortion, and ADC distortions. The effect of all these distortions is lumped into a single parameter we call selectivity. Based on the discussion and analysis from the meeting, contributions suggested possible the sub-band selectivity values from 20 dB to 34 dB. The receiver performance is simply modelled as being 23 dB below the total input power level Pin. Pin the total power into the receiver, whether it is signal or jammer.

The definition of Sub-band/In-channel selectivity is introduced for clarity in the SBFD feasibility study:

- Sub-band/In channel selectivity is the ratio of the interference power in the desired DL to the total input power at the receiver input. The total input power includes both desired signal and the undesired jammers. The selectivity includes the entire receiver all the way through the output of the FFT.

FFT leakage and selectivity

As in FR1, the FFT leakage aspect is not a dominant factor in the UE receiver, and we do not include any factor for this effect.

Jammer frequency and time offset were studied and are not significant factors influencing UE-UE interference.

10.7.1.1.2 Transmitter aspects

Inband emissions (co-channel)

RAN4 has decided to use the IBE requirements from 38.101-2 clause 6.4.2.3.4 (power class 3 UE). It is understood these requirements are minimum performance requirements as opposed to typical requirements. RAN4 has agreed to use typical requirements for the UE parameters, however, did not conclude on the typical values so we are using the formulation from the MPS.

It should also be assumed the LO location is in the center of the channel for the purposes of system studies in RAN4. The LO location is important as it allows placement of the image.

Analysis indicates that the IBE interference is higher and dominates the sub-band co-channel selectivity,

Apart from the selectivity, the degradation can be caused by transmitter leakage from the UL sub-band into the DL sub-band. For co-channel case, the leakage was agreed to be modelled using IBE based model. Additionally, the IQ image contribution for the IBE model for co-channel CLI can be ignored for the DUD configuration.

For UE co-channel Tx model, UE IBE in TS 38.101-2 can be used in the feasibility study as shown in Table 10.7.1.1-1. This model consists of three parts, General, IQ image, Carrier leakage. In the system level simulation, the general and IQ image parts shall be considered, while the carrier leakage part can be ignored in the feasibility study. For DUD configuration, the IQ image from the uplink is fully contained in the UL sub-band and does not land in the DL subband, thus the IQ image can also be ignored in the simulation.

Table 10.7.1.1-1: Requirements for in-band emissions in TS 38.101-2 (For Power class 3)

| Parameter description | Unit | Limit (NOTE 1) |  |  | Applicable Frequencies |
| --- | --- | --- | --- | --- | --- |
| General | dB |  |  |  | Any non-allocated (NOTE 2) |
|  |  |  | Output power for FR2-1 | Output Power for FR2-2 |  |
| IQ Image | dB | -25 | > 10 dBm | > 8.1 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -20 | ≤ 10 dBm | ≤ 8.1 dBm |  |
| Carrier leakage | dBc | -25 | > 0 dBm | > -1.9dBm | Carrier frequency (NOTES 4, 5) |
|  |  | -20 | -13 dBm ≤ Output power ≤ 0 dBm | -14.9 dBm ≤ Output power ≤ -1.9 dBm |  |

10.7.1.2  UE-UE adjacent channel CLI modeling

Editor's note: This section captures the CLI modeling.

10.7.1.2.1 Receiver aspects

Effect of adjacent channel aggressor UE jammer

We analyzed the design of the FR2-1 receiver. There are multiple considerations in the receiver design … for example residual sideband, reciprocal mixing, integrated phase noise, IM3 distortion, and ADC distortions. The receiver performance is simply modelled as being 34 dB below the total input power level Pin. Pin the total power into the receiver, whether it is signal or jammer.

10.7.1.2.2 Transmitter aspects

ACLR (adjacent channel)

ACLR is one aspect modelled as an interference aspect from a nearby aggressor UE transmitting in an adjacent UL subband. UE ACLR is modeled as 24 dB at max power, improving 1 dB/dB with backoff up to a maximum of 10 dB of improvement. Therefore, when the backoff is 10 dB, the ACLR is 34 dB. This model is an approximation of the performance of a typical UE.

10.7.2	Summary

Editor's note: This section captures the conclusion of feasibility.

End changed section***********************************************************