3GPP TSG-RAN4 Meeting #107		R4-2309076

Incheon, South Korea, May 22nd – 26th, 2023

Agenda item:	7.32.2

Source:	Apple Inc.

Title:	On NR-U enhacement

WI/SI:	NR_unlic_enh-Core

Release:	Rel-18

Document for:	Approval

# 1	Introduction

3GPP Rel-16 NR-U WI [1] specified how the NR technology can be used on the unlicensed spectrum thus offering more resources in frequency bands, such as 5GHz and 6GHz. Since not all the countries and regions were addressed during the corresponding Rel-16 and Rel-17 WI, RAN#96 meeting approved a new WI [2] with the main motivation of introducing PC3 and enabling LPI and VLP in countries not covered by previous efforts. In this paper we discuss the streamlining definition of Full and Partial RB allocations.

# 2	Discussion

An issue was raised with the current definition of Full and Partial RB allocations. The description is different for MPR and A-MPR and has alternative text with some network signalling flags. A proposal was made to unify the descriptions to create a simpler specification. The issue was discussed in RAN4#106bis-e and the tentative outcome was captured in WF[3]. The result is provided below for reference:

Streamlining the Full and Partial RB description would create an easier to understand specification and is therefore supported from our side. The discussion considers updating the description for MPR and A-MPR cases. Every refinement of wording needs to guarantee that no requirements are changed.

When checking the wording for PC5 MPR it can be observed that the Full and Partial RB allocations are defined by:

- Full RB requires all sub-carriers to be occupied.

- Full RBs need to be part of configuration A in Table 6.2F.2-2.

- Partial RBs have at least one RB not occupied or are part of configuration B in Table 6.2F.2-2.

To our understanding the differentiation between configuration A and configuration B for MPR is required due to issues with image rejection under wideband operation.  This distinction is not needed for A-MPR as the described issue is already handled by MPR. To not change requirements with MPR it is proposed to keep the definition for MPR as is while streamlining und unifying the definition for A-MPR.

Observation 1: Full and Partial RB allocations for PC5 MPR are defined by:

- Full RB requires all sub-carriers to be occupied.

- Full RBs need to be part of configuration A in Table 6.2F.2-2.

- Partial RBs have at least one RB not occupied or are part of configuration B in Table 6.2F.2-2.

Observation 2: The differentiation between configuration A and configuration B for PC5 MPR is required due to issues with image rejection under wideband operation.

Proposal: To not change requirements with MPR it is proposed to keep the definition for MPR as is while streamlining und unifying the definition for A-MPR.

# Conclusions

The following proposals and observations are made:

Observation 1: Full and Partial RB allocations for PC5 MPR are defined by:

- Full RB requires all sub-carriers to be occupied.

- Full RBs need to be part of configuration A in Table 6.2F.2-2.

- Partial RBs have at least one RB not occupied or are part of configuration B in Table 6.2F.2-2.

Observation 2: The differentiation between configuration A and configuration B for PC5 MPR is required due to issues with image rejection under wideband operation.

Proposal: To not change requirements with MPR it is proposed to keep the definition for MPR as is while streamlining und unifying the definition for A-MPR.

# 4	References

- RP-1912926, "WID on NR-based Access to Unlicensed Spectrum ", Qualcomm Inc.

- RP-221813, "New WID: Introduction of evolved shared spectrum bands", Apple Inc.

- R4-2306557, “WF on NR-U enhancement topics”, Apple, 3GPP TSG-RAN WG4 Meeting #106bis-e