3GPP TSG-RAN WG4 Meeting#107                              R4-2309182

Incheon , KR, May 22 – May 26, 2023

Agenda item:	5.27.1.1

Source: 	ZTE Corporation

Title: 	Further discussion on system parameter for NTN in Ka band

Document for:	Approval

Introduction

In RAN#92e meeting, the work item [RP-221819] on NR NTN (Non-Terrestrial Networks) enhancements was approved as one of Rel-18 RAN1/2/3 package. In this contribution, we want to share further views for system parameters for NTN in Ka-band.

Agreement:

- FSS spectrum should be included in 3GPP band definition for the purpose of enabling ESIM/moving VSAT moveable NTN terminal with priority.  [Fixed VSAT in FSS allocation – Companies to check RAN4 ToR to see if there is a restriction on fixed device in fixed spectrum]

- Define several NTN Ka-bands to address the diversity of spectrum allocation to ESIM by national regulators, RAN4 should consider specifying several “NTN Ka-bands”

Discussion

## 2.1.	Asymmetric channel bandwidths

The UE channel bandwidth can be asymmetric in downlink and uplink. In asymmetric channel bandwidth operation, the narrower carrier shall be confined within the frequency range of the wider channel bandwidth.

In FDD, the confinement is defined as a maximum deviation to the Tx-Rx carrier center frequency separation (defined in table 5.4.4-1) as following:

ΔFTX-RX = | (BWDL – BWUL)/2 |

The operating bands and supported asymmetric channel bandwidth combinations are defined in table 5.3.6-1.

Observation 1: the legacy restriction for Tx-Rx carrier center frequency separation might be not needed if there are no filtering or duplex between Ka-band DL and UL.

## 2.2. Channel raster and sync raster

The channel raster for NTN Ka-band could be be the same as band n257, i.e. 60/120kHz to the simplify the VSAT /ESIM/MES implementation complexity for initial access. In addition, sync raster could use similar approach as band n257. It should be noted that range of NREF should fall into 3GHz to 24.25GHz instead of 24.25-52.6GHz. Therefore global raster should be still 15kHz while with step size as 4 and 8 for 60kHz and 120kHz ΔFRaster.

Proposal 1: to use following NR-ARFCN in the Table 2.3-1/2.3-2 for NTN Ka-band.

| NR operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n512 | [60] | 2070833 – <1> – 2112499 | 1553334 – <4> – 1746666 |
|  | [120] | 2070833 – <2> – 2112499 | 1553334 – <8> – 1746664 |
| n511 | [60] | 2084999 – <1> – 2112499 | 1553334 – <4> – 1746666 |
|  | [120] | 2084999 – <2> – 2112499 | 1553334 – <8> – 1746664 |
| n510 | [60] | 2070833 – <1> – 2084999 | 1553334 – <4> – 1746666 |
|  | [120] | 2070833 – <2> – 2084999 | 1553334 – <8> – 1746664 |

Conclusions

In this contribution, we want to share some further views from system parameter perspective for NTN Ka-band.and proposals are made as following:

Proposal 1: to use following NR-ARFCN in the Table 2.3-1/2.3-2 for NTN Ka-band.

| NR operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n512 | [60] | 2070833 – <1> – 2112499 | 1553334 – <4> – 1746666 |
|  | [120] | 2070833 – <2> – 2112499 | 1553334 – <8> – 1746664 |
| n511 | [60] | 2084999 – <1> – 2112499 | 1553334 – <4> – 1746666 |
|  | [120] | 2084999 – <2> – 2112499 | 1553334 – <8> – 1746664 |
| n510 | [60] | 2070833 – <1> – 2084999 | 1553334 – <4> – 1746666 |
|  | [120] | 2070833 – <2> – 2084999 | 1553334 – <8> – 1746664 |

Proposal 2: to use following GSCN for Ka-band as following

Table 4: Applicable SS raster entries per operating band (FR2-NTN)

| NR operating band | SS Block SCS | SS Block pattern (note 1) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n512 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472– <24> – 19416 |
| n511 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472– <24> – 19416 |
| n510 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472– <24> – 19416 |
| NOTE 1: SS Block pattern is defined in section 4.1 in TS 38.213. |  |  |  |

References

- RP-221819, Core part: NR NTN (Non-Terrestrial Networks) enhancements, Thales, Approved.

- RP-222654, Core part: NR NTN (Non-Terrestrial Networks) enhancements, Thales, Approved.

- TR 38.821

- TR 38.803

- TR 38.828

- R4-2217496, Email discussion summary for [104-bis-e][312] NR_NTN_enh_Part1

- R4-2217467, WF on NR NTN Enhancements Part1, Thales, approved.

- R4-1709181,WF on NR band Numbering, approved.

- R4-2220224 Ad-hoc minutes for NTN above 10GHz band introduction

- R4-2220239 WF for above 10GHz band definition and system parameters,Thales, Approved.

- R4-2302879, Ad-hoc minutes for system parameters of above 10GHz NTN band introduction, Qualcomm, Approved.

- R4-2305925	WF on NTN enhancement - [106bis-e][309] NR_NTN_enh_Part1,Approved.