# 3GPP TSG-RAN WG4 Meeting # 107						R4-2309200

# Incheon, Korea, 22 May - 26 May , 2023

Source:		Ericsson

Title:			simulation parameters for transparent and non-transparent schemes

Agenda item:		8.28.2.3

Document for:		Approval

# Introduction

In this contribution, we present our simulation parameters used on the transparent and non-transparent schemes.

# Discussion

The common simulation parameters used in the simulation are listed in Table 1 below.

Table 1:Parameters for link level and RF simulations

| Parameter | Value |
| --- | --- |
| Filter coefficient | [1 0.28], [0.28 1 0.28],[0.335 1 0.335], RRC, see Figure 1 |
| Modulation scheme | QPSK |
| Waveform | DFT-s-OFDM or CP-OFDM |
| Carrier frequency and duplex mode | 700MHz and 4GHz |
| Subcarrier spacing | 15 kHz and 30kHz |
| System Bandwidth | BW=100MHz@SCS=30kHz, 0.7GHz: BW=20MHz@SCS=15kHz, 4GHz |
| Number of RBs and starting RB | Sweep different combination |
| Counter-IM3 | 60 dB |
| UE, gNB antennas | UE: 1, gNB: 2 (700MHz) and 4 (4GHz) |
| Channel | TDL-D, Medium correlation, 30ns delay spread TDL-C, Medium correlation, 300ns delay spread |

Figure 1:FDSS filter evaluated

The additional simulation parameters used in transparent and non-transparent schemes are listed in Table 2 and Table 3 below.

Table 2: Additional parameters for link level simulations

| Configuration Type | Parameter | Value |
| --- | --- | --- |
| Spectrum Expansion | Inband RBs (non-spectrum expansion PRBs) + MCS | {6 RBs, MCS1 or MCS8} {30 RBs, MCS3 or MCS8} |
|  | Excess/reserved band size | 25% |
|  | Total allocation size | {8 RBs, MCS1} {40 RBs, MCS 8} |
| Without Spectrum Expansion | Allocated RBs + MCS | {8 RBs, MCS0 or MCS6} {40 RBs, MCS2 or MCS6} |
| With and Without Spectrum Expansion | DMRS configuration | 2 DMRS symbols (Type 1) |
|  | PUSCH duration | 14 symbols |
|  | Frequency hopping | Off |
|  | UE speed | 3km/h |
|  | Number of Tx antennas | 1, Optional: 2 |
|  | Number of Rx antennas | 2 or 4 for FR1 2 for FR2 |
|  | Target BLER | 10% |

# Conclusions

In this contribution, we present our simulation parameters used in non-transparent and transparent schemes.

# References

R4-2217745, WF on enhancements to reduce MPR/PAR, Nokia