3GPP TSG-RAN WG4 Meeting # 107						R4-2309210
Incheon, Korea, 22 May - 26 May , 2023

Source:		Ericsson

Title:			On ETSI requirements impact on Rel-17 IoT NTN UE

Agenda item:		9.7.4

Document for:		Approval

# Introduction

In RAN4#106-bis-e, the ETSI requirements of unwanted emission and some receiver requirements are discussed but no conclusion. In this paper we present our view on this.

# Discussion

The harmonized standard of ETSI (MES) [2] covers the band 255 as subBand1. As some of requirements in [2] are not the same with 3GPP TS 36.102, such requirements in [2] needs to be further compared with 3GPP relevant requirements and see if a IoT NTN UE complying 3GPP would also comply with ETSI requirements [2] or not.

One company discussed the unwanted emission requirement in general and identify the ETSI requirement (Table 4b) is more stringent than 3GPP SEM for Cat-M1 and NB-IoT. As 3GPP has A-MPR or guard band method to cope with tighter local regulatory requirement, one way to cope with ETSI requirement is to adopt the 3GPP A-MPR or guard band for unwanted emission within transmission band.  Additional SEM requirement needs to be introduced due to this.

Additional SEM requirement needs to be introduced for OOB emission in Table 4b in ETSI[2]

Apart out of band emission observed above, there is also the emission limitation from the 30MHz to the b255 lower band edge 1626.5MHz and higher band edge at 1660.5 MHz. The OOB emission defined in 3GPP is specified up to the 2 X BW offset from the edge of the channel and beyond this offset, the general spurious requirements apply. In Table 3 of ESTI[2], the requirement at band edge from 1626.2 MHz to 1626.5MHz (0.3MHz offset from band edge) is -40 dBW in 30kHz measurements, which is -10 dBm in 30kHz which is the same with 3GPP SEM level of -10 dBm at frequency offset between 0 to 1MHz, but from 1626.0 to 1626.2 MHz, ETSI has more stringent requirement which is down to – 47 dBW which is -17 dBm in 30kHz.  Because of this, the additional spurious requirement needs to be introduced due to this. From the implementation perspective, as the duplex filter can help to reduce the emission outside the band b255, as such, the filter characteristic on the transition band to stop band needs to be discussed so to see if additional A-MPR or guard band would be needed to comply with Table 3 in ETSI[2].

Additional spurious requirement needs to be introduced for spurious emission in Table 3 in  ETSI[2]

Figure 1:  Table 3 and Table 4a in ETSI [2]

Another requirement is the unwanted emission in the carrier-off state. The definition of the carrier-off is below:

carrier-off state: state in which the MES (Mobile Earth Station) is not transmitting a carrier

comparing the 3GPP definition of the TX OFF power:

Transmit OFF power is defined as the mean power when the transmitter is OFF. The transmitter is considered to be OFF when the UE is not allowed to transmit or during periods when the UE is not transmitting a sub-frame. During DTX and measurements gaps, the UE is not considered to be OFF.

And the TX off power is -50 dBm measured within the configured channel bandwidth for Cat-M1 and the same for NB-IoT, it is not the same requirements as the carrier-off unwanted emissions defined in Table 5 in ETSI [2]. To compare with, the value of -87 dBW at frequency above 1610 MHz (-57 dBm measured in 100kHz) is used in Table 5 below. Scaling it to 200kHz for NB-IoT, the ETSI requirement is -57 dBm + 10log(180/100))= -54.4 dBm, which is 4 dB tighter than 3GPP requirement. For Cat-M1, the ETIS requirement is -57 dBm + 10log(1080/100)= -46.6 dBm, which is higher than -50dBm defined in 3GPP.  As 3GPP defines the RX spurious of -47 dBm within 1MHz measurement bandwidth, to measure the Rx spurious, the UE is set to RRC connect mode but there is no uplink transmitted or downlink data received. So from Rx spurious test, it could regard as an equivalent 3GPP requirement to the unwanted emission in carrier-off state.

Receiver spurious emission requirement in 3GPP could be compared with unwanted emission requirement in carrier-off state in ETSI [2]

The 3GPP requirement of receiver spurious requirement is -47 dBm measured in 1MHz which is -57 dBm in 100kHz measurement bandwidth. This is the same with -87 dBW (-57 dBm) above the 1610 MHz in Table 5 in ETSI[2].  But there is 10 dB more stringent requirement between 1525 to 1559 MHz which ETSI has more stringent requirement. Another aspect is increasing the measurement bandwidth from 100kHz to 1MHz also relax the requirement which smear the narrowband spur which could show up using a lower measurement bandwidth.

ETIS unwanted emission in carrier-off state is more stringent than 3GPP equivalent requirements.

Table 7.9.1-1 [3] : General receiver spurious emission requirements

| Frequency band | Measurement bandwidth | Maximum level | NOTE |
| --- | --- | --- | --- |
| 30MHz f < 1GHz | 100 kHz | -57 dBm |  |
| 1GHz f 12.75 GHz | 1 MHz | -47 dBm |  |
| 12.75 GHz f 5th harmonic of the upper frequency edge of the DL operating band in GHz | 1 MHz | -47 dBm | 1 |
| 12.75 GHz – 26GHz | 1 MHz | -47dBm | 3 |
| NOTE 1: Applies only for Band 22, Band 42, Band 43, Band 48 and Band 49 NOTE 2: Unused PDCCH resources are padded with resource element groups with power level given by PDCCH_RA/RB as defined in Annex C.3.1. NOTE 3: Applies for Band 46 and Band 47 |  |  |  |

Figure 2: Table 5 in ETSI [2]

There also ACS and inband blocking requirement in ETSI[2] which is specified differently with 3GPP. The ACS requirement in ETSI[2] is below:

And blocking requirement in ETSI[2] is below:

The performance requirement for ACS is 0.5 dB degradation for the SNR and for IBB is 1 dB SNR degradation in ETSI[2].

Assuming an IoT NTN UE complies with 3GPP ACS and IBB, for Cat-M1 device, the ACS requirement is 14 dB above the REFSENS and ACS interferer is 45.5 dB higher than REFSENS. For b255,

the wanted signal level is -102.7 dBm + 14 = - 88.7 dBm

The ACS interferer level is : -102.7 dBm + 45.5 = -57.2 dBm

Assume the SNR is -1 dB for LTE-M and IM = 2 dB for the implementation margin, the receiver noise caused by receiver noise figure:

PN_internal= -174 dBm/Hz + 9 + 10log(6*180kHz)= -104.7 dBm

The total noise including the noise increase due to phase noise and other impairment source is:

PN_total = -88.7 – SNR – IM = -88.7 – (-1)- 2 =-89.7 dBm

The increased noise allowance (in linear) is: 10log( lin(PN_total) – lin(PN_internal))= -89.8 dBm

Assuming the phase noise is flat over the RF bandwidth and also assuming such phase noise is Nph dBc/Hz at offset equal to the offset between ACS interferer/IBB blocker which is illustrated in Figure 3, The noise increasing due to the reciprocal mixing is

Pph= Nph + 10log(BW) + I_ACS

With I_ACS= -57.2 dBm and assuming the noise increase all due to the phase noise, Nph = -89.8-3.13= -93 dBc/Hz with offset of BW= 1.4 MHz.

Figure 3: reciprocal mixing of phase noise

To compare with ETSI ACS, as the SNR only is allowed to be worsen by 0.5 dB, this could be expressed with

CNR- 0.5 = 10logPsPN_internal+Pph2=10logPsPN_internal-0.5

So the allowed noise degradation is

Pph2= 10log(PN_internal) + 10log(100.05-1) = -104.7 + (-9.2)= -113.9 dBm

Here Ps is the wanted signal level and Pn is receiver internal noise from noise figure and this can be calculated using the same Nph above

Pph_ETSI_ACS= -93 + 10log (6*180kHz)+ I_ACS_ETSI = -93 + 60.3 + (-102.7) + 12 = -123.4 dBm

Since the allowed noised degradation is greater than noise contribution from cat-M1 UE, so the 3GPP ACS requirement is tougher than ETSI requirement.

The same process can be done for the IBB comparison and the result in listed in the Table below.

Table 1: The comparison between 3GPP and ETSI [2] requirements

|  |  | 3GPP phase noise profile (dBc/Hz) | phase noise impairment from ETSI requirement (dBm) | Phase noise impairment allowance in ETSI requirement (dBm) | Margin ( 3GPP compliance compared to ETSI requirements) |
| --- | --- | --- | --- | --- | --- |
| Cat-M1 | ACS@BW | -93 | -123.4 | -113.9 | 9.5 dB |
|  | IBB (offset @>4.2MHz) | -125 | -104.7 | -110.5 | -5.8 dB |
| NB-IoT | ACS @300kHz | -81 | -121.6 | -125.3 | 3.7 dB |
|  | IBB (offset @5MHz) | -110 | -97.7 | -118 | -20 dB |

In Table 1, the IBB blocker level in 3GPP is adjusted with PAPR of 10 dB to compare with the CW blocker in ETSI, with such consideration, the design margin for ETSI IBB is still negative meaning 3GPP compliance IoT NTN device may have difficulty to meet ETSI IBB requirements.

A 3GPP compliance IoT NTN device in Rel-17 can pass ETSI ACS but fail the ETSI IBB requirement.

In summary, there are many aspects that ETSI[2] requirements impact the 3GPP IoT NTN devices. Though some of requirements can be coped with the traditional approach (A-MPR or guard band), the IBB requirement cannot be met without introducing a tougher requirement in 3GPP specification. As the Rel-17 IoT NTN core is done and adding new RF requirements is not possible to be introduced in TEI, it then up to Rel-17 IoT NTN UE to comply the ETSI in Rel-17. Therefore, there is no need to consider the ETSI in Rel-17 3GPP specification.

ETSI [2] should not be considered in 3GPP IoT NTN release 17 according to 3GPP process.

It is up to Rel-17 IoT NTN to comply the ETSI[2].

As there is ongoing IoT NTN enh Rel-18 work item but there is no RF discussion, it could be possible to introduce such work in Rel-18 IoT NTN enh with WID updates.

Add the ETSI requirement consideration from Rel-18 IoT NTN UE with WID updates.

# Conclusions

In this contribution, we present our view on the ETSI requirement [2] with below observations and proposals:

Observation 1 Additional SEM requirement needs to be introduced for OOB emission in Table 4b in ETSI[2].

Observation 2 Additional spurious requirement needs to be introduced for spurious emission in Table 3 in  ETSI[2]

Observation 3 Receiver spurious emission requirement in 3GPP could be compared with unwanted emission requirement in carrier-off state in ETSI [2]

Observation 4 ETIS unwanted emission in carrier-off state is more stringent than 3GPP equivalent requirements.

Observation 5 A 3GPP compliance IoT NTN device in Rel-17 can pass ETSI ACS but fail the ETSI IBB requirement

Proposal-1: ETSI [2] will not be considered in 3GPP IoT NTN release 17

Proposal-2: It is up to Rel-17 IoT NTN to comply the ETSI[2]

Proposal-3: Add the ETSI requirement consideration in Rel-18 IoT NTN with WID updates.

# References

- [1] R4-2306645,	WF on LTE_NBeMTC_NTN_UERF, MediaTek Inc.

- [2] ETSI EN 301 681, V2.1.2 (2016-11),

- [3] TS 36.101-1