3GPP TSG-RAN WG4 Meeting #107	R4-2309218

Incheon, Korea, 22 – 26 May, 2023

Agenda Item:	8.31.3.3

Source:	Ericsson

Title:	Discussions on Co-channel coexistence for LTE SL and NR SL

Document for:	Discussions

# 1	Introduction

RAN4 had initial discussions to identify the RRM impact of introducing support for co-channel coexistence for LTE SL and NR SL. The progress was quite limited, but some issues were identified for further study in [1]. In this contribution, we provide our further analysis on some of those issues.

# 2	Discussions

In co-channel co-existence scenario supported in Rel-18 for type A device (i.e. device with both NR SL and LTE SL modules where the LTE SL information can be shared with NR SL), solutions are introduced to share the sidelink resource pool between NR SL and LTE SL. Compared to the SL LTE/NR UE from previous releases, the main difference in SL LTE/NR operating in co-channel coexistence scenario is that the SL resources can be shared dynamically, and new resource selection mechanism are introduced to achieve this. In previous releases on the other hand, the separate fixed resources were configured for NR SL and LTE SL and there was no mechanism to share those between NR SL and LTE SL. Different types of sharing mechanism are being discussed in RAN1. RAN1 has reached following agreements at previous meeting for the objective on co-channel coexistence between NR SL and LTE SL where the resource pools are shared [RAN1#112-bis-e (e-Meeting) agreements]:

Agreement In NR SL resource (re)selection procedure, for dynamic resource pool sharing, the list of initial SL RSRP thresholds is separately (pre)configured (i.e., Alt 3 in the agreement of RAN1#112 meeting) for the PHY layer of NR SL module to exclude NR SL candidate resources overlapping with LTE SL reserved resources by other LTE SL UE. - FFS: whether a different initial SL RSRP threshold list may be (pre)configured for selecting single slot resources in NR SL slots with NR PSFCH. Agreement In NR SL resource (re)selection procedure, for determining the LTE SL periodic reserved resources by other LTE SL UE, the formula of Q in Section 8.1.4 in TS 38.214 is used. - FFS: whether the formula of Q in Section 14.1.1.6 in TS 36.213 is used additionally to derive the largest value between the two formulas Note: The reservation period and the location of the LTE SL periodic reserved resources are determined based on the set of LTE SL logical subframe that may belong to LTE SL resource pool as specified in TS 36.213. Agreement In NR SL resource (re)selection procedure, the PHY layer of NR SL module excludes NR SL candidate resources overlapping with LTE SL resources associated with non-monitored subframe (in which UE has not monitored due to its transmission) in LTE SL module - For determining the LTE SL resources associated with non-monitored subframe of LTE SL module, - All the LTE SL resources in the non-monitored subframes are assumed to be repeatedly reserved Q times for each LTE SL resource reservation period (pre)configured in a LTE SL resource pool - The same formula of Q for determining the LTE SL periodic reserved resources by other SL UE is used - Note: The reservation period and the location of the LTE SL resources repeated Q times in the above procedure are determined based on the set of LTE SL logical subframe that may belong to LTE SL resource pool as specified in TS 36.213 - The PHY layer of NR SL module applies the above procedure in Step 5 in Section 8.1.4 of TS 38.214 - FFS: whether the set SA is initialized to the set of all the candidate single-slot resources excluding NR SL candidate resources overlapping with LTE SL resources associated with non-monitored subframes when the amount of candidate single-slot resources is not sufficient as specified in Step 5a in Section 8.1.4 of TS 38.214 - Note: For periodic resource reservation of NR SL transmission, the PHY layer of NR SL module further excludes all NR SL candidate resources in NR SL slots that overlap with LTE SL resource pool where NR SL periodic resources are overlapping with LTE SL resources associated with non-monitored subframes of LTE SL module according to Step 5 in Section 8.1.4 of TS 38.214 - Note: The NR SL module excludes NR SL candidate resources overlapping with NR SL resources (including NR SL resource reservation period(s) (pre)configured in a NR SL resource pool) associated with non-monitored slot as specified in Section 8.1.4 of TS 38.214 (i.e., NR SL slot in which UE has not monitored due to its own transmission), which is the existing Rel-16/17 NR SL behavior - Note: It is assumed that the information relevant to LTE SL resources associated with non-monitored subframe of LTE SL module used in the above procedure is shared from LTE SL module to NR SL module Agreement In NR SL resource (re)selection procedure for dynamic resource pool sharing, the PHY layer of NR SL module excludes NR SL candidate resources in a NR SL slot overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission - For the LTE SL periodic resources selected to be used for LTE SL module’s own LTE SL transmission, - For determining the above LTE SL selected resources, the LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission are repeated according to the LTE SL resource reservation period and LTE SL resource reselection count - The PHY layer of NR SL module applies the above procedure in Step 5 in Section 8.1.4 of TS 38.214 - Note: For periodic resource reservation of NR SL transmission, the PHY layer of NR SL module further excludes all NR SL candidate resources in a NR SL slot where NR SL periodic resources are in the NR SL slot overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission according to Step 5 in Section 8.1.4 of TS 38.214 - Note: When the PHY layer of NR SL module cancels the above procedure according to Step 5a in Section 8.1.4 of TS 38.214, UE selects either LTE SL transmission or NR SL transmission according to Rel-16 NR SL in-device coexistence rule - Alt 1: The above procedure is applied at least when the priority of LTE SL transmission is higher than the priority of NR SL transmission - It is up to UE implementation whether or not to apply the above procedure when the priority of LTE SL transmission is not higher than the priority of NR SL transmission - Note: It is assumed that the information relevant to LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission used in the above procedure is shared from LTE SL module to NR SL module Agreement For NR SL transmissions of 30kHz SCS with dynamic resource pool sharing, the power level of the NR PSCCH/PSSCH transmission in the first of NR SL slots overlapping with an LTE SL subframe is always larger than or equal to the power level(s) of the NR PSCCH/PSSCH transmission in the subsequent NR SL slot overlapping with the LTE SL subframe. - Note: How to ensure the above condition is up to UE implementation - FFS on whether same or different frequency allocation may be used in the second overlapping slot Agreement When the same TB is transmitted on the NR SL slots overlapping with the LTE SL subframe, it is up to UE implementation how to avoid transmitting NR PSCCH/PSSCH only in the subsequent NR SL slot overlapping with an LTE SL subframe according to RAN#99’s agreement for NR PSCCH/PSSCH transmissions of 30kHz SCS with dynamic resource pool sharing - FFS: whether/how to differently handle the case when different TBs are transmitted on the NR SL slots overlapping with the LTE SL subframe and the NR SL transmission in the first overlapping NR SL slot is dropped or reselected. Agreement RAN1 does not pursue further enhancements except for the following to handle NR SL candidate resources overlapping with LTE PSCCH for non-adjacent LTE SL resource pool in NR SL resource (re)selection procedure for dynamic resource pool sharing in Rel-18. - NR SL module uses LTE SL RSRP measurement results (as specified in TS 36.213) shared from LTE SL module to determine excluding NR SL candidate resources overlapping with LTE PSCCH and/or LTE PSSCH resources reserved by other LTE SL UE in NR SL resource (re)selection procedure for dynamic resource pool sharing in Rel-18. Agreement In NR SL resource (re)selection procedure, the PHY layer of NR SL module excludes NR SL candidate resources where the corresponding PSFCH transmission occasions overlap with LTE SL reserved resources by other LTE SL UE in time domain, Down-selection one of followings: - Option 1: - When the SL RSRP value associated with the LTE SL reserved resources is higher than a SL RSRP threshold, where the SL RSRP threshold is derived based on LTE SL priority of other LTE SL UE and NR SL priority for NR SL transmission - The list of the above initial SL RSRP thresholds is separately (pre)configured - FFS: whether this initial SL RSRP threshold list can be (pre)configured per subset of PSFCH resources - For determining the LTE SL periodic reserved resources by other LTE SL UE, - Reuse the same mechanism as in NR SL resource (re)selection procedure excluding NR SL candidate resources overlapping with LTE SL reserved resources by other LTE SL UE - The PHY layer of NR SL module applies the above procedure in Step 6 in Section 8.1.4 of TS 38.214 - Note: For periodic resource reservation of NR SL transmission, the PHY layer of NR SL module further excludes NR SL candidate resources of NR SL periodic resources when the corresponding PSFCH transmission occasions are overlapping with LTE SL reserved resources by other LTE SL UE and the SL RSRP value associated with the LTE SL reserved resources is higher than a SL RSRP threshold according to condition c in Step 6 in Section 8.1.4 of TS 38.214 - Note: It is assumed that the information relevant to LTE SL reserved resources by other LTE SL UE used in the above procedure is shared from LTE SL module to NR SL module - Option 2: - For determining the LTE SL periodic reserved resources by other LTE SL UE, - Reuse the same mechanism as in NR SL resource (re)selection procedure excluding NR SL candidate resources overlapping with LTE SL reserved resources by other LTE SL UE - The PHY layer of NR module applies the above procedure in Step 5 in Section 8.1.4 of TS 38.214 - FFS: whether to apply the above procedure based on the priority of the LTE SL reserved resources and/or the priority of the NR SL transmission - Note: For periodic resource reservation of NR SL transmission, the PHY layer of NR SL module further excludes NR SL candidate resources of NR SL periodic resources when the corresponding PSFCH transmission occasions are overlapping with LTE SL reserved resources by other LTE SL UE in time domain according to Step 5 in Section 8.1.4 of TS 38.214. - Note: It is assumed that the information relevant to LTE SL reserved resources by other LTE SL UE used in the above procedure is shared from LTE SL module to NR SL module Agreement The NR SL module uses the information from the starting LTE SL subframe to the ending LTE SL subframe in the shared information from the LTE SL module. - The starting LTE SL subframe is no later than the time (n-T_start) - n is the time where NR module triggers its NR SL resource (re)selection procedure as defined in clause 8.1.4 of TS 38.214 - Down-selection one of followings for T_start: - Option 1-1: T_start is T_0 as defined in clause 8.1.4 of TS 38.214 - Option 1-2: T_start is 1100ms - Option 1-3: T_start is up to UE implementation - The ending LTE SL subframe is no earlier than the time (n-T_valid2) - Down-selection one of followings for T_valid2: - Option 2-1: T_valid2 is T - Option 2-2: T_valid2 <= T+4ms - Option 2-3: T_valid2 is up to UE implementation

RAN1 has made good progress in many areas. One possible area where RRM impact is needed could be due to the resource selection procedure based on RSRP measurements. For this procedure, the existing measurement requirements shall apply, e.g. for the measurement period and measurement accuracy etc.  Nevertheless, there are still many FFSs in the areas with possible RAN4 impact and therefore RAN4 needs to wait for more RAN1 progress before defining the requirements.

Observation 1: There are many open issues in RAN1 related to co-channel co-existence related to RAN4.

It is also recalled that the SL UE supporting both NR SL and LTE SL can switch between E-UTRA V2X sidelink and NR V2X sidelink transmissions on a dedicated carrier since release 16. For this scenario, RAN4 defined scheduling restrictions requirements for UE switching between E-UTRA V2X sidelink and NR V2X sidelink as follows [2]:

### 12.9.1 Scheduling availability of UE switching between E-UTRA sidelink and NR sidelink This clause contains the restrictions on the scheduling availability for V2X sidelink due to switching between E-UTRA V2X sidelink and NR V2X sidelink transmission on a dedicated carrier. For the NR V2X sidelink, the assumed number of configured symbols in a slot is 14. When switch from E-UTRA V2X sidelink to NR V2X sidelink occurs in NR slot ‘n’, - UE is not expected to transmit or receive on NR V2X sidelink on the slot ‘n’. When switch from NR V2X sidelink to E-UTRA V2X sidelink occurs in NR slot ‘n-1’, - UE is not expected to transmit or receive on NR V2X sidelink on the slot ‘n-1’. When switch from NR V2X sidelink to E-UTRA V2X sidelink occurs in E-UTRA subframe ‘n’, - UE is not expected to transmit or receive on E-UTRA V2X sidelink on the subframe ‘n’. When switch from E-UTRA V2X sidelink to NR V2X sidelink occurs in E-UTRA subframe ‘n-1’, - UE is not expected to transmit or receive E-UTRA on V2X sidelink on the subframe ‘n-1’.

RAN4 needs to discuss whether these requirements can be reused for the co-channel coexistence scenario for device type A in Rel-18.

RAN4 to discuss whether to reuse the scheduling restriction requirements defined in clause 12.9.1 in TS 38.133 for co-channel co-existence between NR SL and LTE SL cenario for a type A device in Rel-18.

# 3	Summary

In this contribution we have discussed the RRM impact of the objective co-channel co-existence for NR SL. Based on the discussion, we have made following proposals:

Observation 1: There are many open issues in RAN1 related to co-channel co-existence related to RAN4.

- RAN4 to discuss whether to reuse the scheduling restriction requirements defined in clause 12.9.1 in TS 38.133 for co-channel co-existence between NR SL and LTE SL scenario for a type A device in Rel-18.

# References

R4-2306364, “WF on NR_SL_enh2_part2”, OPPO

TS 38.133 V18.1.0