3GPP TSG RAN WG4 Meeting #107		R4-2309226

Incheon, Korea, 22nd – 26th May 2023

Agenda Item:	10.1.2

Source:	Ericsson

Title:	Monitoring of paging occasions for CG-SDT with HD-FDD Redcap UEs

Document for:	Discussion, Decision

# 1	Introduction

RAN1 and RAN4 have received an LS from RAN2 in [1] with the following description:

RAN2 has discussed possible clarifications on monitoring of paging occasions for CG-SDT with HD-FDD Redcap UEs based on specification text in RAN2 and relevant sections in RAN1 and RAN4. Current RAN2 specifications do not explicitly specify what happens for UEs in half duplex mode if a paging occasion conflicts with a CG-SDT occasion. It is RAN2’s understanding that although information pertaining to this can be found in e.g., 38.213, clause 17.2 or in 38.133, clause 5.1B.2.6, the UE is only required to monitor paging for SI change indication in any paging occasion at least once per modification period during SDT if the initial downlink BWP on which the SDT procedure is ongoing is associated with a CD-SSB. Similar to connected mode behaviour, since the UE is only required to monitor the paging in any paging occasion at least once per modification period, there should be other paging occasions available (within the modification period) to monitor the paging for SI change even if some of them overlap with the CG-SDT occasion(s). Hence, RAN2 would like to ask RAN1 and RAN4 to take the above understanding into account and discuss possible amendment on misalignment between RAN2 specifications and RAN1 and/or RAN4 specifications.

The action to RAN1 and RAN4 is to to take the above understanding into account and discuss possible amendment on misalignment between RAN2 specifications and RAN1 and/or RAN4 specifications for CG-SDT with HD-FDD RedCap.

In this contribution we provide our view on the LS from RAN2.

# 2	Discussion

# The related RAN4 requirements on maximum interruptions during paging reception are defined as follows [2]:

# “The requirements in clause 4.2B.2.6 shall apply for RedCap UEs.

# For RedCap UE in HD-FDD mode, if a paging occasion overlaps with CG-SDT transmission then the UE shall monitor the paging during the paging occasion. In this case the UE is allowed to drop the CG-SDT transmission.”

# According to the RAN2 LS reply, the UE is required to monitor paging in any paging occasion at least once per modification period. This means, the overlapping between paging reception and CG-SDT transmissions may not happen always within the modification period for HD-FDD since the UE can receive paging in multiple paging occasion within the modification period. However, there is still a risk that the paging reception and CG-SDT occasions always coincide for which the existing requirements can apply, i.e the UE is allowed to drop the CG-SDT transmissions. We therefore propose to clarify the current specification text as follows:

# “For RedCap UE in HD-FDD mode, if all paging occasions within a modification period overlap with CG-SDT transmission then the UE shall monitor the paging during at least one paging occasion. In this case the UE is allowed to drop the CG-SDT transmission.”

# 3	Conclusion

In this contribution, we have discussed the incoming RAN2 LS on monitoring of paging occssions for CG-DST with HD-FDD.  Based on the discsussions, we have proposd to revise the related RAN4 sepcification text as follows:

Proposal: The current specification text related to maximum interruption in paging reception for HD-FDD in clause 5.1B.26 in TS 38.133 is revised as follows:

- “For RedCap UE in HD-FDD mode, if all paging occasions within a modification period overlap with CG-SDT transmission then the UE shall monitor the paging during at least one paging occasion. In this case the UE is allowed to drop the CG-SDT transmission.”

# References

- R1-2304331, “LS on Monitoring of paging occasions for CG-SDT with HD-FDD Redcap UEs”, RAN2, Ericsson

- TS 38.133 V18.1.0