3GPP TSG-RAN WG4 Meeting # 107	R4-2309244

Incheon, KR, May 22 – May 26, 2023

Agenda Item:			8.3.4

Source:					Qualcomm Incorporated

Title:                        Discussion on demodulation test method for FR2 multi-Rx UE

Document for:		Approval

# Introduction

In RAN4#106bis-e, the test methods for FR2 multi-Rx UE were discussed and the WF was approved in [1]. In this meeting, we provide our views on the test method for UE Demodulation.

# Discussion

## Whether to satisfy legacy REFSENS requirement with XdB degradation

### Background on the criteria for legacy FR2 demodulation test

During the discussion in RAN4#106 bis-e, there was no consensus on whether the test directions for multi-Rx demodulation should satisfy legacy REFSENS requirements with XdB degradation. Note that when XdB is equal to the drop between REFSENS requirement and EIS spherical coverage requirements, the side condition becomes the test directions should pass legacy EIS spherical coverage requirement [2].

First of all, we would like to re-iterate the reason why in legacy FR2 demodulation we need the criterion that UE shall pass the REFSENS test as per TC 7.3.2 of TS 38.521-2 [3]. In the legacy FR2 demodulation testing, Noc level configured at the TE transmitter should guarantee no larger than 1dB difference between Reference point SNR and Baseband SNR, the following principle can be found in section 7.2.1.3.1 of TR 38.810 [4].

-	Set wanted noise to give 1dB difference between Reference point SNR and Baseband SNR, using agreed UE requirements, so SNRRP = SNRBB + 1dB

The below figure is from TR 38.810[4] which shows the inherent SNR offset between reference point and baseband.

Figure 1: DL SNR reference point for UE Demodulation [4]

Therefore, as specified in TR 38.810, Noc value is set based on legacy REFSENS with the following equation:

Noc = REFSENSPC3, n260, 50MHz -10log10(SCSREFSENS x PRBREFSENS x 12) - SNRREFSENS + ∆thermal        (1)

Here REFSENSPC3, n260, 50MHz is used with the pre-condition that test direction should pass legacy REFSENS. ∆thermal is the amount of dB that the wanted noise is set above UE thermal noise, giving a rise in total noise of ∆BB. ∆thermal = 6dB, giving a rise in total noise of 1dB.

Due to DL power limitation, without this criterion, the TE could not guarantee SNR at baseband is configured correctly in demodulation testing. For example, if the direction does not satisfy REFSENS, when ZdB SNR is configured at the transmitter side of TE, we cannot correctly configure the baseband SNR as the intended SNR. That says if the test direction satisfies REFSENS, when YdB is configured at the transmitter side of TE, the SNR at baseband should be in the range of Z-1 and ZdB. In addition, the testable SNR specified in RAN4 TS 38.810 [4] and RAN5 TS 38.521-4 [5] are also derived based on the criterion that the test direction satisfies the REFSENS requirement.

Observation 1: In the legacy FR2 demodulation testing, Noc level configured in TE transmitter should guarantee no larger than 1dB difference between Reference point SNR and Baseband SNR.

Observation 2: In legacy FR2 demodulation testing, to achieve 1dB different between reference SNR and baseband SNR, Noc value is set based on the side condition that the test direction shall pass the legacy REFSENS requirement.

Observation 3: The testable SNR for legacy FR2 demodulation testing is also derived based on the criterion that the test direction satisfies the REFSENS requirement.

### What’s different between legacy and multi-Rx demodulation test

Compared with legacy and multi-Rx demodulation testing, the main difference is there is cross-interference between TRP1 and TRP2 on top of the interference between two polarizations. While with good enough isolation (e.g., [12dB] minimum isolation), the impact due to the non-ideal isolation between all the branches within two TRPs and two polarizations can be ignored. Note that we are assuming 12dB isolation is good enough here and will further discuss the required minimum isolation in section 2.2. Then the implementation of multi-Rx demodulation can be illustrated in Figure 2, where a1 and a2 are the polarization- and polarization-  from TRP1 respectively, and a3 and a4 are the polarization- and polarization- from TRP2 respectively. The symbols b1 and b2 are the polarization- and polarization- from panel 1 of UE respectively, and b3 and b4 are the polarization- and polarization- from panel 2 of UE. The dotted line indicates “virtual cable” is achieved between TE and DUT under the minimum isolation requirements. Then the correlation matrix between two TRPs and two polarizations could be added in TE side together with channel model to verify the intended demodulation test cases. The correlation matrix is under discussion in the demodulation session.

Figure 2: Illustration of multi-Rx demodulation test

Therefore, if minimum isolation can be achieved between all the branches, there is no difference between legacy and multi-Rx demodulation test per branch for the procedure configuring the Noc level.

Observation 4: If minimum isolation can be achieved between all the branches, there is no difference between legacy and multi-Rx demodulation test per branch for the procedure configuring the Noc level.

### How to configure the Noc level

In RAN4#106bis-e, the following agreement was captured in WF [1] regarding the criterion in the multi-Rx demodulation testing,

Issue 3-2-3: Whether to satisfy legacy REFSENS requirement with XdB degradation - Proposals - Option 1 (Qualcomm): RAN4 to consider Option 3, i.e., passing legacy REFSENS requirements with XdB degradation per branch as the starting point to select the test directions for UE demodulation testing. - Option 2: Specify other option if any - Agreement: - RAN4 further study how to set wanted noise for the test direction to give [1] dB difference between Reference point SNR and Baseband SNR in demodulation test. The following options can be considered: - Option 1 - Other options not precluded

The principle of 1dB difference between Reference point SNR and Baseband SNR could be confirmed for multi-Rx demodulation test which is used for the current FR2-1 and FR2-2 demodulation testing.

Proposal 1: RAN4 to confirm the test direction should guarantee 1dB difference between Reference point SNR and Baseband SNR in multi-Rx demodulation test.

With the principle of 1dB difference between Reference point SNR and Baseband SNR in multi-Rx demodulation test, the Noc level should be set based on UE noise floor per branch at the reference point which is related to the UE antenna gain. Therefore, when XdB degradation is considered, the Noc level for multi-Rx per branch is calculated by the following equation (2):

Noc = REFSENSPC3, band Y, 50MHz -10log10(SCSREFSENS x PRBREFSENS x 12) - SNRREFSENS + ∆thermal + X       (2)

Note that ∆thermal is determined by the different between Reference point SNR and Baseband SNR and ∆thermal is 6dB if 1dB difference is adopted.

Therefore, for multi-Rx demodulation test, the similar criterion of identifying a test direction where legacy REFSENS is met should be satisfied. Otherwise, the TE with its DL power limitation is not able to configure target baseband SNR correctly during the test. Here XdB is to give more flexibility for test directions search but meanwhile it will lead to lower testable SNR.

Proposal 2: RAN4 to agree the wanted noise, i.e., Noc level is set by the below equation for multi-Rx demodulation test on band Y:

Noc = REFSENSPC3, band Y, 50MHz -10log10(SCSREFSENS x PRBREFSENS x 12) - SNRREFSENS + ∆thermal + X

where:

-	REFSENSPC3, band Y, 50MHz is the REFSENS value in dBm specified for Power Class 3 UE in band Y for 50MHz channel bandwidth in TS 38.101-2 Table 7.3.2.3-1, [dBm/Hz]

-	SCSREFSENS is a subcarrier spacing associated with NRB for 50MHz in TS 38.101-2 Table 5.3.2-1, chosen as 120kHz.

-	PRBREFSENS is NRB associated with subcarrier spacing 120kHz for 50MHz in TS 38.101-2 Table 5.3.2-1 and is 32.

-	12 is the number of subcarriers in a PRB

-	SNRREFSENS = -1 dB is the SNR used for simulation of REFSENS

-	∆thermal is the amount of dB that the wanted noise is set above UE thermal noise, giving a rise in total noise of ∆BB. ∆thermal = 6dB, giving a rise in total noise of 1dB.

-	X is the allowable degradation in sensitivity from legacy REFSENS requirements in dB

### The feasibility of TE to quantify legacy REFSENS + XdB

It was agreed in the previous meeting that the measurement setup for multi-Rx UE RF test can be used for demodulation test. Even in UE RF testing, ‘go or no-go’ was agreed to be used as the metric to define the RF requirements. It is straightforward for the TE to verify whether the UE could pass the REFSENS + XdB (a fixed DL level from TE) in a certain direction.

Proposal 3: There is no testability issue for TE to quantify legacy REFSENS requirement + XdB. There is no testability challenge for TE to identify directions with sensitivity better than REFSENS + X dB.

### How to determine the parameter XdB

In [2], we compare the Pros and Cons for the following three options on how to select test directions:

- Option 1: UE needs to pass legacy REFSENSE requirements per branch (X=0dB)

- Option 2: UE needs to pass legacy EIS spherical coverage requirements per branch (e.g., X=12.6dB for n260)

- Option 3: UE needs to pass legacy REFSENSE requirements with XdB degradation per branch (e.g., X [0dB, 12.6dB] for n260)

The pros and cons for the three options are lists in Table 1:

Table 1: Pros and Cons for the options on how test directions selection

|  | Pros | Cons |
| --- | --- | --- |
| Option 1 | High testable SNR, e.g., 17.7 for n260 | Might be difficult to find test directions for two AoAs with the condition of passing REFSENS and achieving min. isolation between all active branches |
| Option 2 | Test directions can be selected in the spherical coverage map. | Very low testable SNR, e.g., 5.1dB for n260 |
| Option 3 | A compromise between Option 1 and Option 2. It is easier to find the test directions for two AoAs to satisfy all the 3 criteria with with an upper SNR limit that is only slightly degraded from legacy practice. |  |

The following table 2 provides the testable SNR with different X values. Note that the testable SNR is calculated based on the parameters defined in TR 38810, i.e., IFF with 100MHz CBW on n260.

Table 2: Testable SNR with different X values

| X values (dB) | Maximum testable SNR (dB) |
| --- | --- |
| 0 | 17.7 |
| 1 | 16.7 |
| 2 | 15.7 |
| 4 | 13.6 |
| 6 | 11.5 |
| 8 | 9.4 |
| … | … |
| 12.6 | 5.1 |

In figures below, we show legacy antenna gain CDF of two UE implementations that have standards compliant gain drop for PC3 devices. The UE is assumed to be a box with 6 faces. Table 3 captures the fraction of directions on the sphere that meet the sensitivity condition for various X values. We also note that in practice, UEs far exceed the spherical coverage requirement, which has the effect of increasing the number of qualifying directions for any relevant value of X.

| Identical 4x1 dual pol. modules on opposite faces | Identical 4x1 dual pol. modules on adjacent faces |
| --- | --- |

Table 3: Availability of qualifying test directions with different X values

| X (dB) | % of sphere with sensitivity better than REFSENS + X |  |
| --- | --- | --- |
|  | Modules on opposite faces | Modules on adjacent faces |
| 1 | 5 | 5 |
| 2 | 10 | 10 |
| 4 | 19 | 22 |

Observation 5: X = 2 dB implies 1/5th of all legacy spherical coverage directions (or 10% of all directions) will qualify for ‘standard’ UEs used for UE RF requirement simulation (i.e those that just meet the legacy spherical coverage requirements).

In addition, the test directions need to satisfy the minimum isolation requirements of [12dB]. We provide the preliminary simulation to evaluate the number of AoA pair satisfying minimum isolation of [12dB]. The simulation assumptions are following the assumptions agreed in RF session in [7]. Annex C of [8] has details on the module orientation in the UE reference coordinate system. Module locations were grouped into two categories.

- In the first category, one module pattern covered the polar region of the reference coordinate system. Examples: Front+Bottom, Front+Top, Front+Right

- In the second category, both modules cover equatorial regions (away from polar regions). Examples: Top+Left, Top+Right

The isolation is quantified as SIR at (PC3) UE receiver. And for each AoA pair, the test directions are satisfying the legacy REFSENS requirements + X (X=1, 2, 4dB).

Figure 3: Preliminary simulation results for number of AoA pairs for a 15⁰ step size grid, on adjacent faces, and with at least 12dB isolation with X = 1, 2, 4dB)

Figure 4: Preliminary simulation results for number of AoA pairs for a 15⁰ step size grid, on opposite faces, and with at least 12dB isolation with X = 1, 2, 4dB)

From Figure 3, with X=2dB, the number of AoA pairs with at least 12dB isolation is 9. From Figure 4, with X=2dB, the number of AoA pairs with at least 12dB isolation is 6.

Observation 6: With 2dB, the number of AoA paris with at least 12dB isolation is at least 6 even with a very coarse grid, i.e., 15⁰ step size grid.

Note that the SIR is reflecting the cross-interference between two TRPs. As agreed in [1], the minimum isolation of [12dB] should be guaranteed between polarizations. With a small value of X, it is easier to achieve the minimum isolation between polarizations.

Observation 7: With a small value of X, it is easier to achieve the minimum isolation between polarizations.

To retain a balance between ease of identifying directions and retaining testable SNR, it is proposed to adopt X = 2 as baseline.

Proposal 4: As a baseline, RAN4 to use X = 2 for multi-Rx demodulation test directions selection.

For comparison, we also provide the number of AoA pairs with at least 12 dB isolation without assuming that test direction passing the legacy EIS spherical coverage requirements.

Figure 5: Preliminary simulation results for the number of AoA pairs for a 15⁰ step size grid and with at least 12dB isolation within the whole sphere

It can be seen from Figure 3, Figure 4 and Figure 5 that it is not enough to choose the minimum isolation (SIR based) as the only criterion, which will lead to the selected test direction cannot achieve the desired SINR at the baseband. For example, the AoA pair that the antenna gain from AoA1 is -30dB, and the antenna gain from AoA2 is -42dB can achieve 12dB isolation but it will lead to a very high Noc level when configuring intended SNR at baseband.

Observation 8: The number of AoA pairs for a given step size of grid and with at least 12 dB isolation depends on UE implementation. The test directions and AoA separations for demodulation test could be declared by manufacturer.

Observation 9: It is not enough to choose the minimum isolation (strictly SIR based) as the only criterion, which will lead to the selected test direction cannot achieve the desired SINR at the baseband.

With the above observations, we have the following proposals for the criteria of UE test directions

Proposal 5: The test directions and AoA separation for multi-Rx demodulation test can be declared by UE. The UE declared test directions should satisfy the following 3 criteria:

- Minimum isolation shall be at least [12dB]

- Rank number shall be higher or same as intended rank for a given test case

- Each direction of AoA pair shall pass legacy REFSENS requirements with X=2dB degradation.

Proposal 6: If the UE declared test directions and AoA separation cannot satisfy the criteria in Proposal 5. The test directions which can satisfy the criteria should be searched by TE as the fallback approach.

## Minimum isolation requirements

In RAN4#106bis-e, the agreements for minimum isolation requirements were approved in [1].

Issue 3-2-1: Minimum isolation - Proposals - Option 1 (Qualcomm): The minimum isolation requirements of 12dB could be reused for all the active branches for multi-Rx UE demodulation testing. - Option 2: Specify other option if any - Agreement: - The minimum isolation requirements of [12dB] could be reused for all the active branches for multi-Rx UE demodulation testing

The minimum isolation requirements of [12dB] were reused from legacy demodulation test based on the End-to-End System Simulation conducted in RAN5 contribution of [6].

To verify whether minimum isolation of 12dB is applicable for multi-Rx UE demodulation testing, a similar system simulation should be conducted by applying additional isolation on top of low correlation matrix between two TRPs and two polarizations.

Figure 6: Isolation implementation in the simulation

Proposal 7: It is encouraged companies provide the simulation results to verify the minimum isolation requirements of [12dB] for multi-Rx demodulation test.

# Conclusions

In this paper, we discuss the test methods for UE demodulation for multi-Rx UE. The following observations and proposals are made:

Observation 1: In the legacy FR2 demodulation testing, Noc level configured in TE transmitter should guarantee no larger than 1dB difference between Reference point SNR and Baseband SNR.

Observation 2: In legacy FR2 demodulation testing, to achieve 1dB different between reference SNR and baseband SNR, Noc value is set based on the side condition that the test direction shall pass the legacy REFSENS requirement.

Observation 3: The testable SNR for legacy FR2 demodulation testing is also derived based on the criterion that the test direction satisfies the REFSENS requirement.

Observation 4: If minimum isolation can be achieved between all the branches, there is no difference between legacy and multi-Rx demodulation test per branch for the procedure configuring the Noc level.

Proposal 1: RAN4 to confirm the test direction should guarantee 1dB difference between Reference point SNR and Baseband SNR in multi-Rx demodulation test.

Proposal 2: RAN4 to agree the wanted noise, i.e., Noc level is set by the below equation for multi-Rx demodulation test on band Y:

Noc = REFSENSPC3, band Y, 50MHz -10log10(SCSREFSENS x PRBREFSENS x 12) - SNRREFSENS + ∆thermal + X

where:

-	REFSENSPC3, band Y, 50MHz is the REFSENS value in dBm specified for Power Class 3 UE in band Y for 50MHz channel bandwidth in TS 38.101-2 Table 7.3.2.3-1, [dBm/Hz]

-	SCSREFSENS is a subcarrier spacing associated with NRB for 50MHz in TS 38.101-2 Table 5.3.2-1, chosen as 120kHz.

-	PRBREFSENS is NRB associated with subcarrier spacing 120kHz for 50MHz in TS 38.101-2 Table 5.3.2-1 and is 32.

-	12 is the number of subcarriers in a PRB

-	SNRREFSENS = -1 dB is the SNR used for simulation of REFSENS

-	∆thermal is the amount of dB that the wanted noise is set above UE thermal noise, giving a rise in total noise of ∆BB. ∆thermal = 6dB, giving a rise in total noise of 1dB.

-	X is the allowable degradation in sensitivity from legacy REFSENS requirements in dB

Proposal 3: There is no testability issue for TE to quantify legacy REFSENS requirement + XdB. There is no testability challenge for TE to identify directions with sensitivity better than REFSENS + X dB.

Observation 5: X = 2 dB implies 1/5th of all legacy spherical coverage directions (or 10% of all directions) will qualify for ‘standard’ UEs used for UE RF requirement simulation (i.e those that just meet the legacy spherical coverage requirements).

Observation 6: With 2dB, the number of AoA paris with at least 12dB isolation is at least 6 even with a very coarse grid, i.e., 15⁰ step size grid.

Observation 7: With a small value of X, it is easier to achieve the minimum isolation between polarizations.

Proposal 4: As a baseline, RAN4 to use X = 2 for multi-Rx demodulation test directions selection.

Observation 8: The number of AoA pairs for a given step size of grid and with at least 12 dB isolation depends on UE implementation. The test directions and AoA separations for demodulation test could be declared by manufacturer.

Observation 9: It is not enough to choose the minimum isolation (strictly SIR based) as the only criterion, which will lead to the selected test direction cannot achieve the desired SINR at the baseband.

Proposal 5: The test directions and AoA separation for multi-Rx demodulation test can be declared by UE. The UE declared test directions should satisfy the following 3 criteria:

- Minimum isolation shall be at least [12dB]

- Rank number shall be higher or same as intended rank for a given test case

- Each direction of AoA pair shall pass legacy REFSENS requirements with X=2dB degradation.

Proposal 6: If the UE declared test directions and AoA separation cannot satisfy the criteria in Proposal 5. The test directions which can satisfy the criteria should be searched by TE as the fallback approach.

Proposal 7: It is encouraged companies provide the simulation results to verify the minimum isolation requirements of [12dB] for multi-Rx demodulation test.

# References

- R4-2302923, “WF for FR2 OTA test enhancement SI”, Qualcomm, RAN4#106, Athens, GR, Feb-Mar. 2023

- R4-2304679, “Discussion on test method for FR2 multi-Rx UE’, Qualcomm, RAN4#106-Bis-e, Apr. 2023

- TS 38.521-2

- TR 38.810

- TS 38.521-4

- R5-198264, End-to-End System Simulations for Min. Isolation Requirements, Keysight Technologies

- R4-2303708, ‘WF on NR FR2 multi-Rx chain DL reception’, Samsung et al, RAN4#106, Athens, GR, Feb-Mar. 2023

- R4-2304603, On UE RF requirements for 2AoA FR2 DL MIMO, Qualcomm, RAN4#106-Bis-e, Apr. 2023