3GPP TSG-RAN4 Meeting #107	        R4-2309275 Incheon, South Korea 22nd – 26th May 2023

Source: 	Qualcomm Inc.

Title: 	Handling IMD5 MSD for higher power classes when it is not specified for PC3

Agenda Item:	7.17.2

Document for:	Approval

# Introduction

For some band combinations IMD5 related MSD is not specified for PC3. When such band combination is specified for high power UE operation, the IMD5 interference can increase in the order of 15 dB, and MSD can become significant. In this contribution different options how the handle this in the specification are discussed.

# Discussion

38.101-1 has a large number of band combinations where 2 UL IMD related MSD in Table 7.3A.5-1 is subject to note 4: This band is subject to IMD5 also which MSD is not specified. The background for this is that for PC3 the IMD5 MSD is often very small and left unspecified. Additionally, these band combinations often have also lower order IMD issues, and having also IMD5 test point would result in high test burden. There are also band combinations where the IMD5 would fall on the DL band, but note 4 is missing.

When such band combination is specified for HPUE operation, the IMD5 interference can increase approximately 15 dB and as a result the required MSD can become significant. This leaves to the obvious question how to handle the IMD5 test point for HPUE.

Observation 1: For band combinations where PC3 IMD5 is left unspecified, required IMD5 MSD for HPUE operation can be significant.

In the following we use EN-DC combination DC 1A-3A-n77A as example. The PC3 Dual UL MSD is specified as follows

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-3A_n77A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 3 | 1712.5 | 5 | 25 | 1807.5 | 31.5 | IMD2 |
|  | n77 | 3757.5 | 10 | 50 | 3757.5 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 3 | 1775 | 5 | 25 | 1870 | 8.5 | IMD4 |
|  | n77 | 3980 | 10 | 50 | 3980 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 31.0 | IMD2 |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |
|  | n77 | 3915 | 10 | 50 | 3915 | N/A | N/A |

It can be seen that band 3 is subject to IMD2 and IMD4 MSD. However, band 3 would also be impacted by IMD5, but there is no mention of this in the specification. The IMD5 test point could be as follows.

| EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1935 | 5 | 25 | 2125 | N/A | IMD5 |
| 3 | 1770 | 5 | 25 | 1865 | [2.8] | N/A |
| n77 | 3835 | 10 | 50 | 3835 | N/A | N/A |

In the past 2 dB MSD was used as boundary when IMD5 test point is not specified. In this case the MSD is close to 2 dB for PC3. However, the IMD5 interference being approximately 15 dB higher in PC2 operation, the required MSD would be approximately 15 dB, which is not insignificant.

If the IMD5 test point is specified, then band 3 would be subject to three test points which is excessive. On the other hand, not specifying a case where 15 dB MSD is required is not good either.

Observation 2: Adding a test point for IMD5 for HPUE can result in increased test burden. Not specifying a test point for IMD5 for HPUE can leave high MSD cases unspecified.

In our view the lesser evil is additional test point. Therefore we propose that RAN4 evaluates and specifies MSD for IMD for HPUE in cases where it has been left unspecified for PC3 but MSD becomes significant in HPUE operation.

Proposal 1: RAN4 evaluates and specifies MSD for IMD5 for HPUE in cases where IMD5 MSD was left unspecified for PC3 but MSD becomes significant in HPUE operation.

# Conclusions

In this contribution MSD for IMD5 was discussed. There are cases where IMD5 is not specified for PC3, but it could become significant for higher power classes. The following observation and proposals were made.

Observation 1: For band combinations where PC3 IMD5 is left unspecified, required IMD5 MSD for HPUE operation can be significant.

Observation 2: Adding a test point for IMD5 for HPUE can result in increased test burden. Not specifying a test point for IMD5 for HPUE can leave high MSD cases unspecified.

Proposal 1: RAN4 evaluates and specifies MSD for IMD5 for HPUE in cases where IMD5 MSD was left unspecified for PC3 but MSD becomes significant in HPUE operation.

# References