3GPP TSG-RAN WG4 Meeting # 107	R4-2309285

# Incheon, KR, May. 2023

Source	: Qualcomm Incorporated

Title						: Evaluation of MPR reduction schemes

Agenda item		: 8.28.2

Document for	: Approval

# Introduction

In this contribution we summarize our previous analysis work on non-transparent FDSS schemes, non-transparent peak cancellation, and transparent enhancements. The new considerations introduced in this contribution over previous work [8] cover system implementation choices since they have different ramifications on benefits of the schemes.

# Discussion

## On system assumptions

We have previously identified that PcmaxH limits the ability of the UE to increase its maximum output power for some waveforms over its nominal power class expectation [5]. We have therefore proposed establishing system framework to enable power boosting as priority #1 for enhancing UL coverage. The benefits of various schemes, and the conditions where they may provide an enhancement depend strongly on whether RAN4 decide to formally enable power boost. The spec. impact can be discussed separately since those details represent merely the mechanics of enabling.

Observation 1: No decision on viability of any method can be made without deciding whether UE power boost is formally enabled by the standard.

In discussions leading up to the latest RAN4 WF [9] and in RAN1 have identified interest in non-transparent techniques that combine a legacy receiver in conjunction with an enhanced UE. This perspective is reasonable if the analysis is also cleaved along receiver assumption lines, and the conclusions for each can be decoupled. The specific features are:

- Whether legacy DMRS is retained for schemes that use FDSS: the PAPR of DMRS starts to dominate over that of PUSCH, leading to reduction in UL power levels at the UE. Note that RAN4 has a standalone EVM requirement on EVM symbols, this was what motivated development of type 2 DMRS in Rel-16.

- Whether the receiver ignores the extension tones: this scheme gives up some receiver performance in the FDSS case, especially for mild filter shapes. This scheme hurts implementations that use FDSS (milder FDSS is impacted more). On the other hand, schemes like non-transparent peak cancellation are not impacted because they do not place significant power in the extension part.

Observation 2: Evaluation of viability of any method also depends on whether the gNB Rx uses a legacy receiver (legacy DMRS, ignore extension RBs) or an advanced receiver (PAPR optimized DMRS, leverage diversity gain from extension RBs).

## On scope of enhancement

An issue to consider at the outset is what subsets of DFT-s-QPSK waveforms are relevant for coverage enhancements. A competitive network would look to maximize spectral efficiency using link adaption. We studied the implications of this behavior in terms of RB allocation size. The figure on the right captures the optimal RB allocation as a function of bit rate and under the constraints of identical UL power for all cases.

This projection assumes a CDL-C channel, with 64 TxRUs at the gNB. The algorithm optimizes MCS for a given RB alloc to ensure 10% BLER assuming sustained UL (rather than a TDD pattern). i.e. At a target rate of 1Mbps and an UL duty-cycle of 20%., we would have to refer to the 5 Mbps level on the y-axis.

Observation 3: For a target rate of 1Mbps in a typical TDD system (DDDSU), the optimal RB allocation is between 16 and 32, assume < 50.

Observation 4: For a target rate of 0.1Mbps in a typical FDD system, the optimal RB allocation is 2 or 4, assume < 10.

Proposal 1: RAN4 to discuss if coverage enhancement is relevant for wide allocation low MCS cell-edge cases. Wide allocation in this context is >50 for 100M channels and >10 for 20M channels.

## Comparing various schemes

### The legacy DMRS problem for FDSS+BWE

Previous analysis assumed that there would be a suitable DMRS that is defined for this feature. Unfortunately, legacy DMRS with FDSS does not work as well as PUSCH in the sense that PUSCH PAPR improves much faster than that of DMRS. DMRS signals have their own EVM requirements, so the UL power capability cannot be judged on PUSCH EVM/IBE. Our preliminary results suggest that UE Tx power estimates are lower for legacy DMRS compared to DFT-s-QPSK by about 0.4 dB for inner waveforms and about 0.7 dB for outer waveforms. Further study is required to properly capture the impact of using legacy DMRS. In this analysis, a very conservative 0.1 dB is assumed, but this may have to be revised upwards in the future.

Proposal 2: RAN4 to study the impact of using legacy DMRS with FDSS.

### The legacy receiver problem for FDSS+BWE

It is important to consider here that FDSS+BWE is uniquely sensitive to receiver assumptions. The reason can be traced back to the fact that this scheme puts significant power in the extension RBs. The FDSS+BWE scheme therefore suffers from SNR degradation at the receiver if the diversity tones in the extension RBs are ignored. To compound the problem, FDSS+legacy DMRS can start to dominate in terms of PAPR beyond a certain degree of frequency selectivity in the filter. To preserve signal quality of legacy DMRS, PUSCH power availability reduces for FDSS+BWE.

Observation 5: FDSS+BWE is penalized by legacy receivers that do not consider diversity across replicated tones by 0.2 dB or more for the FDSS schemes studied.

In contrast, to FDSS schemes non-FDSS non-transparent schemes (for example, one that uses peak cancellation) do not place any information in the extension part, nor do they commit a significant fraction of the total UL power to the extension part. These schemes do not require specialized receivers.

### Organization of results

Our previous analysis [8] highlights the impossibility to confirm any one non-transparent scheme as an enhancement that applies to all relevant DFT-s-QPSK waveforms. The search for an enhancement needs a more nuanced evaluation rather than a blunt goal to ‘confirm’ one scheme or another.

The first axis of fragmentation in analysis is imposed by the MPR table because of impact on UL power. Based on MPR for DFT-s-QPSK, there are two broad groups to consider: inner waveforms and outer waveforms. Edge waveforms can be ignored for now, because of low population count.

The second  dimension to consider is the collection of system assumptions (see section 2.1). To reduce variant diversity, we have grouped the features together based on whether the enhancement should have low spec. and implementation impact (‘legacy’), or if a more potentially disruptive treatment is enabled (‘advanced’). For each category of enhancement, what is considered ‘advanced’ or ‘legacy’ are listed below.

- Legacy system assumptions: UE boost is precluded, gNB uses legacy DMRS, gNB ignores extension RBs

- Advanced system assumptions for schemes other than FDSS+BWE: UE boost enabled.

- Advance system assumption for FDSS+BWE: UE boost enabled, gNB+UE use new UL DMRS with low PAPR, gNB uses diversity combining across repeated tones in the extension part.

Proposal 3: To systematically identify enhancement opportunities, RAN4 to pursue evaluation by decoupling inner/outer and differentiating by system assumptions (DMRS, extension RBs, power boost).

We evaluated the 10 cases identified in RAN1 as representative for the feature, included to the right in baseline form (prior to any enhancement). The data is presented in ascending order of throughput for added perspective. The max. power capability of each case was determined as the worst case over the possible locations.

The schemes considered are:

- Baseline: Rel-17 or older

- FDSS w/ BWE:

  - Type 3 filter [7]: 3-tap time domain imp. response (0.28 1 0.28)

  - Type 4 filter [7]: tr RRC [0.5 1/6]

- Non-transparent peak cancellation

- Transparent techniques

Boost simulation data was presented in annexes of a previous contribution [8] and are not reproduced here.

### Inner DFT-s-QPSK

#### ‘Advanced’ system assumptions

In figure 2.3.4.1-1, we present UL power potential and link level benefits for various schemes assuming the gNB maximizes its avenues for improving its UL. The results for each scheme are stitched together for easier grouping, but the actual curves are not intended to convey meaning.

Figure 2.3.4.1-1: Link level benefit of various schemes, inner waveforms

Observation 6: For inner DFT-s-QPSK waveforms 8/10 representative cases are better enhanced by schemes other than FDSS+BWE, i.e. by schemes that do not need advanced gNB receivers or upgraded DMRS.

#### ‘Legacy’ system assumptions

In figure 2.3.4.2-1, we present UL power potential and link level benefits for various schemes assuming the gNB remains largely unchanged from Rel-17 practice (it only harvests opportunistic benefits). The results for each scheme are stitched together for easier grouping, but the actual curves are not intended to convey meaning.

Figure 2.3.4.2-1: Link level benefit of various schemes, inner waveforms

Observation 7: For inner DFT-s-QPSK waveforms with legacy assumptions (legacy gNB receivers and boost precluded UEs), FDSS+BWE schemes enjoy marginal superiority in just 1/10 representative cases, but are inferior to even legacy techniques for 9/10 representative cases.

### Outer DFT-s-QPSK

#### ‘Advanced’ system assumptions

In figure 2.3.5.1-1, we present UL power potential and link level benefits for various schemes assuming the gNB maximizes its avenues for improving its UL. The results for each scheme are stitched together for easier grouping, but the actual curves are not intended to convey meaning.

Figure 2.3.5.1-1: Link level benefit of various schemes, outer waveforms

The trends above show the benefit of enabling power boosting from the UE. From the link level benefit curves it is evident that 6/10 representative case can be enhanced by FDSS+BWE, provided both: UE power boosting is enabled and an advanced receiver is used at the gNB. Unfortunately, it is also evident that for FDSS schemes, it appears that the UE must dynamically choose the optimal filter based on allocation size and MCS. Not only is this a significant burden for the UE, but the link adaptation would also have to keep up with this behavior.

Observation 8: For boost-enabled UEs working in concert with advanced receivers, BWE+FDSS has theoretical benefit, but only if the UE dynamically optimizes its pulse shaping filter mask based on RB size and MCS.

#### ‘Legacy’ system assumptions

In figure 2.3.5.2-1, we present UL power potential and link level benefits for various assuming the gNB remains largely unchanged from Rel-17 practice (it only harvests opportunistic benefits). The results for each scheme are stitched together for easier grouping, but the actual curves are not intended to convey meaning.

Figure 2.3.5.2-1: Link level benefit of various schemes, outer waveforms

From the link level benefit curves for legacy receivers and boost-precluded UEs, it is evident that the enhancement is best served by transparent schemes or non-FDSS non-transparent schemes. FDSS+BWE schemes do not excel for any of the representative test cases. Transparent schemes offer the most general benefit for these boundary conditions.

Observation 9: For legacy receivers and boost-precluded UEs, UL enhancement is best served by transparent schemes. In some select cases non-transparent schemes like peak cancellation and FDSS+BWE may provide mild benefit over transparent schemes.

### Summary of results

The results can be summarized in tabular form (below), noting that it is counter-productive to enable a non-transparent scheme where a transparent scheme will suffice:

Observation 10:

| Optimal enhancement scheme | Inner waveforms | Outer waveforms |
| --- | --- | --- |
| Need advanced system features: - Diversity combining of repetition tones at gNB Rx. - Low-PAPR DMRS design - UE power boost enabled | Transparent | A uniformly optimal scheme is not evident In some cases, BWE+FDSS provides best benefit, but it requires UE to dynamically change filter profile with RB allocation and MCS. In other cases, transparent schemes are the optimal choice. |
| Legacy system assumptions ok: - gNB Rx only focuses on non-extension part - Legacy DMRS design - UE not enabled to boost | Transparent | Transparent* (*) In very select cases, non-transparent peak cancellation and FDSS+BWE provide more benefit than transparent. |

From the table above it is evident transparent schemes suffice for most scenarios: FDSS + BWE does not seem to be the optimal choice for inner waveforms, regardless of whether advanced system assumptions or legacy system assumptions are used. For outer, legacy system implementations mean that FDSS+BWE cannot shine.

In the special case of outer waveforms in concert with advanced features, there are select cases where FDSS+BWE might be useful as an enhancement. Before this observation can be used as a basis to confirm this scheme as an enhancement, further consideration is necessary.

In [5] we worked out that for allocations < 64 RB in a 100 MHz channel, more than 89% were inner. This fraction increases as the RB length allocation cut off reduces. See figures below:

The number of outer DFT-s-QPSK waveform is an order of magnitude smaller than inner DFT-s-QPSK, which suggests the complexity/benefit trade-off must be closely scrutinized for outer waveforms. Some concerns for FDSS+BWE are listed below:

- Since the benefit of BWE+FDSS is not consistent across all RB allocations and MCS cases, enabling outer waveforms would require significant fragmentation of the MPR table.

- Another concern with BWE schemes in general is the vastly more complicated link adaptation strategy. Some skeleton algorithm is necessary to adopt as a working assumption to justify feasibility of BWE schemes. This aspect is a necessary ingredient to establish confidence in the feasibility of BWE schemes. Some concerns to address:

  - The 3GPP boundary condition to retain spectral efficiency is unlikely to be the operating model because of how restrictive it is.

  - Another concern is that the network cannot estimate the benefit of an FDSS+BWE scheme unless the FDSS filter profile itself is standardized. Without a guaranteed benefit, it is not clear how FDSS+BWE can be incorporated into link adaptation.

- Since the benefit is contingent on adoption of advanced receivers, demod requirements for the gNB must be instituted for this feature as well as new low-PAPR DMRS must be defined. This aspect is outside the purview of RAN4, but the observation must be packaged with any recommendation proclaiming feasibility of FDSS+BWE.

Proposal 4: To confirm FDSS+BWE as an enhancement feature, several concerns must first be addressed in the study phase:

- confirmation of scope limitation to outer waveforms

- skeleton real-life link adaptation strategy (to establish confidence in spectral eff. benefit)

- demod requirements for the gNB supporting this feature

- how to determine the subset of the outer waveform region in waveform space where this scheme may be used (MPR table fragmentation)

# Conclusions

Observation 1: No decision on viability of any method can be made without deciding whether UE power boost is formally enabled by the standard.

Observation 2: Evaluation of viability of any method also depends on whether the gNB Rx uses a legacy receiver (legacy DMRS, ignore extension RBs) or an advanced receiver (PAPR optimized DMRS, leverage diversity gain from extension RBs).

Observation 3: For a target rate of 1Mbps in a typical TDD system (DDDSU), the optimal RB allocation is between 16 and 32, assume < 50.

Observation 4: For a target rate of 0.1Mbps in a typical FDD system, the optimal RB allocation is 2 or 4, assume < 10.

Proposal 1: RAN4 to discuss if coverage enhancement is relevant for wide allocation low MCS cell-edge cases. Wide allocation in this context is >50 for 100M channels and >10 for 20M channels.

Proposal 2: RAN4 to study the impact of using legacy DMRS with FDSS.

Observation 5: FDSS+BWE is penalized by legacy receivers that do not consider diversity across replicated tones by 0.2 dB or more for the FDSS schemes studied.

Proposal 3: To systematically identify enhancement opportunities, RAN4 to pursue evaluation by decoupling inner/outer and differentiating by system assumptions (DMRS, extension RBs, power boost).

Observation 6: For inner DFT-s-QPSK waveforms 8/10 representative cases are better enhanced by schemes other than FDSS+BWE, i.e. by schemes that do not need advanced gNB receivers or upgraded DMRS.

Observation 7: For inner DFT-s-QPSK waveforms with legacy assumptions (legacy gNB receivers and boost precluded UEs), FDSS+BWE schemes enjoy marginal superiority in just 1/10 representative cases, but are inferior to even legacy techniques for 9/10 representative cases.

Observation 8: For boost-enabled UEs working in concert with advanced receivers, BWE+FDSS has theoretical benefit, but only if the UE dynamically optimizes its pulse shaping filter mask based on RB size and MCS.

Observation 9: For legacy receivers and boost-precluded UEs, UL enhancement is best served by transparent schemes. In some select cases non-transparent schemes like peak cancellation and FDSS+BWE may provide mild benefit over transparent schemes.

Observation 10:

| Optimal enhancement scheme | Inner waveforms | Outer waveforms |
| --- | --- | --- |
| Need advanced system features: - Diversity combining of repetition tones at gNB Rx. - Low-PAPR DMRS design - UE power boost enabled | Transparent | A uniformly optimal scheme is not evident In some cases, BWE+FDSS provides best benefit, but it requires UE to dynamically change filter profile with RB allocation and MCS. In other cases, transparent schemes are the optimal choice. |
| Legacy system assumptions ok: - gNB Rx only focuses on non-extension part - Legacy DMRS design - UE not enabled to boost | Transparent | Transparent* (*) In very select cases, non-transparent peak cancellation and FDSS+BWE provide more benefit than transparent. |

Proposal 4: To confirm FDSS+BWE as an enhancement feature, several concerns must first be addressed in the study phase:

- confirmation of scope limitation to outer waveforms

- skeleton real-life link adaptation strategy (to establish confidence in spectral eff. benefit)

- demod requirements for the gNB supporting this feature

- how to determine the subset of the outer waveform region in waveform space where this scheme may be used (MPR table fragmentation)

# Reference

[1] 	R4-2217745, ‘WF on enhancements to reduce MPR/PAR’, Nokia, RAN4#104-Bis-e, Oct. 2022

[2]	R4-2218044, ‘Spec. impact of UL power enhancement from transparent techniques’, Qualcomm, RAN4#105, Toulouse, FR, Nov. 2022

[3]	R4-2218373, ‘Discussion on some transparent techniques for UL power enhancement’, Qualcomm, RAN4#105, Toulouse, FR, Nov. 2022

[4]	R4-2220841, ‘WF on the MPR reduction for coverage enhancement’, Nokia et al, RAN4#105, Toulouse, FR, Nov. 2022

[5]	R4-2300710, ‘On UL power enhancement from transparent techniques’, Qualcomm, RAN4#106, Athens, GR, Feb.-Mar. 2023

[6]	R4-2302648, ‘On UL power enhancement from BWE techniques’, Qualcomm, RAN4#106, Athens, GR, Feb.-Mar. 2023

[7]	R4-2304004, ‘LS to RAN4 for results of the LLS performance evaluation of MPR/PAR reduction solutions’, RAN1, RAN4#106-Bis-e, Apr. 2023

[8]	R4-2304604, ‘On link-level benefits of transparent and non-transparent PAPR reduction techniques’, Qualcomm, RAN4#106-Bis-e, Apr. 2023

[9]	R4-2306627, ‘WF on remaining open issues for MPR/PAR reduction for coverage enhancement’, Nokia, RAN4#106-Bis-e, Apr. 2023