3GPP TSG-RAN4 Meeting #107	R4-2309291

# Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on remaining open issues for the core part in NTN

Agenda item:	5.2.8.4

Document for:	Discussion

Introduction

The WI description for RAN4 work related to NR over NTN was presented in [1]. For the past meetings, RAN4 has made progress in developing core requirements for NTN new features, as well as creating the performance testing configuration and specification to test such requirements. This WI was considered complete by RAN plenary in RAN #99, with the maintenance work moving forward to finalize the pending details on this work item. This document discusses pending open issues on the Way Forward  from RAN4 #106 [2]

Discussion

There are two open issues for the core part left for discussion in [2]:

## Issue 1-1: Additional delay for acquisition of epoch time during HO/CHO. - FFS: - Add a clarification that HO/CHO interruption time would be longer when UE does not have valid and applicable ephemeris information of target cell - FFS: Whether to define how much longer. ## Issue 1-6: Parallel measurement capability. - FFS: - It shall be clarified that which feature between 25-2 and 25-5 defines parallel L3 measurements on serving cell and intra-frequency cells. - In case that UE doesn’t support parallel L3 measurements on serving cell and intra-frequency cells, for time-critical neighbor cell measurement requirements, we suggest to skip serving cell measurements since cell change is inevitable in very short time and the update doesn’t impact existing Ttrigger definition.

The open issue 1-1 is related to two situations:

- the problem in the conditional handover that may ensue as the event that triggers the HO procedure might happen much later than the RRC message configuring the conditional handover. Depending on the validity timer of the satellite assistance information provided, the information may no longer be valid for the UE. This situation can also happen at traditional HO, but there is higher probability of seeing this at CHO.

- During blind HO, the UE receives the assistance information from the target cell, which contains the ephemeris information associated to an epoch time of the target cell. However, as the UE does not know the target cell, it cannot fully apply the ephemeris information for the lack of knowledge on the current SFN on this cell.

For conditional HO and blind HO, when the UE cannot determine the validity of the ephemeris of the target cell or it has expired, the handover delay requirement must be postponed to include the time required for the UE to acquire and apply a new valid epoch time information.

For the open issue 1-6, it is unclear whether the UE will benefit from skipping the measurements of serving cell. If the UE is supposed to measure neighbor cells in the same satellite as the serving satellite the UE is capable to collect the serving cell measurements when measuring from neighbor cells on the same satellite. Likewise, it needs clarification on how this will add UEs to obtain time-critical measurements.

For the open issue 1-6, when the UE doesn’t support parallel L3 measurements, and the UE is configured with t-service the UE may skip serving cell measurements, if the UE is not configured to measure any other cells associated to the serving satellite.

Another relevant aspect in the specifications is related to the parameter “Ttrigger”. The parameter is currently defined as: “Ttrigger = max(Tdetect,NR_Intra, Kcarrier* Tdetect,NR_Inter) when serving cell is below the search threshold, and Ttrigger = max(Tdetect,NR_Intra, Nlayer* [60s]) when serving cell is above the search threshold”.

However, if a large number of satellites or SMTCs are configured for measurement for a particular UE, the factor “Kcarrier* Tdetect,NR_Inter” will not be sufficient for the UE to measure across all potential target cells.

The definition of parameter Ttrigger shall be revised for accounting for measurements across multiple satellites as well, including the scaling factor given by Kmulti_SMTC.

Conclusion

This paper has addressed two open issues in the WI for NR_NTN_Solutions and presented the following proposals:

Proposal 1: For conditional HO and blind HO, when the UE cannot determine the validity of the ephemeris of the target cell or it has expired, the handover delay requirement must be postponed to include the time required for the UE to acquire and apply a new valid epoch time information.

- Proposal 2: For the open issue 1-6, when the UE doesn’t support parallel L3 measurements, and the UE is configured with t-service the UE may skip serving cell measurements, if the UE is not configured to measure any other cells associated to the serving satellite.

- Proposal 3: The definition of parameter Ttrigger shall be revised for accounting for measurements across multiple satellites as well, including the scaling factor given by Kmulti_SMTC.

- References

- RP-213691, Revised WID: Solutions for NR to support non-terrestrial networks (NTN), Thales

- R4-2303147, WF on NR NTN RRM requirements, Qualcomm, October 2022