3GPP TSG-RAN4 Meeting #107	R4-2309425

Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

Source: 	Qualcomm Incorporated

Title: 	Discussion on the requirement of subsequent CPC

Agenda item:	8.25.3

Document for:	Discussion

# Introduction

In this paper, we would like to share our view for remaining issues from the last RAN4 #106bis-e meeting[1].

# Discussion

Issue 1-6: whether to consider UE is not evaluating the execution condition of other candidate PSCells while executing CPC - Candidate solutions: - Option 1: Yes. Event_DU for the subsequent change can be set to zero, if the execution condition for another PSCell is fulfilled while the UE is completing the PSCell change. - Option 2: No. According to TS37.340 and RAN2 agreements, UE is not evaluating the execution condition of other candidate PSCells while executing CPC, and the evaluation is continued after finishing the PSCell addition or change.

Observation : UE is not expect to evaluating the execution condition while executing CPC. Moreover, it is not guarantee UE can meet execution condition for other candidate PSCells during executing CPC.

Proposal : UE is not evaluating the execution condition of other candidate PSCells while executing CPC, and the evaluation is continued after finishing the PSCell addition or change.

# Conclusion

Observation : UE is not expect to evaluating the execution condition while executing CPC. Moreover, it is not guarantee UE can meet execution condition for other candidate PSCells during executing CPC.

Proposal: UE is not evaluating the execution condition of other candidate PSCells while executing CPC, and the evaluation is continued after finishing the PSCell addition or change.

# References

[1] R4-2306356, WF on NR Mobility Enhancements RRM requirements (part 2), RAN4 #106bis-e