3GPP TSG-RAN WG4 Meeting # 107							              R4-2309441

Incheon, Korea, May 22 – May 26, 2023

Agenda item:	8.14.3

Source: 	CMCC

Title: 	Discussion on ATG BS RF requirements

Document for:	Discussion

# 1. Introduction

After RAN4 #106-bis meeting, a good process had been reached for ATG BS RF requirements. This paper will focus on ACLR and ACS requirements.

# 2. Discussion

## 2.1. ACLR and ACS requirements

ATG co-existence part reached an agreement about the timeline in the last meeting.

Agreements: It’s suggested to approve following timeline and strictly implement this timeline. date; RAN4 meeting; Target for high priority scenario. 2023-04; RAN4#106bis; - Deadline for official calibration phase, note 1 - deadline for completeness of all assumptions for 1st priority. 2023-05; RAN4#107; - deadline for collection of simulation results for 1st priority - conclude ACLR/ACS for ATG BS and UE based on 1st priority scenarios - deadline for completeness of all assumptions. 2023-08; RAN4#108; - other co-existence related requirements discussion based on co-existence results - conclude isolation distance for 2nd priority non-synchronized scenario. 2023-10; RAN4#108 bis; - conclude co-existence related RF requirements. 2023-12; RAN4#109; - final results check and summarizing - CR drafting. Note 1: companies that doesn’t show calibration results until this meeting could also provide final simulation in future meeting but have to company with calibration results to confirm their simulation results are aligned with other companies. After RAN4 #106bis meeting, it’s also OK to update the data that would be captured into final TR. Note 2: “TR drafting” is ongoing in RAN4 #106bis meeting and could continue through the whole WI. TPs should only be made for issues that are already decided..

According to this timeline, the ACLR and ACS requirements of ATG BS need to be concluded in this meeting. Based on the last meeting simulation result from other companies, we suggest reusing legacy FR1 45dB ACLR and 46dB ACS for ATG BS. And other RF requirements, such as in-band blocking and receiver intermodulation requirements in TS 38.104 also could be reused

Proposal 1: To reuse legacy FR1 ACLR (i.e. 45dBc) requirement.

Proposal 2: To reuse legacy FR1 ACS (i.e. 46dBc) requirement.

Proposal 3: In-band blocking requirements in TS 38.104 could be reused for ATG BS.

Proposal 4: Receiver intermodulation requirements in TS 38.104 could be reused for ATG BS.

# 3. Conclusions

Proposal 1: To reuse legacy FR1 ACLR (i.e. 45dBc) requirement.

Proposal 2: To reuse legacy FR1 ACS (i.e. 46dBc) requirement.

Proposal 3: In-band blocking requirements in TS 38.104 could be reused for ATG BS.

Proposal 4: Receiver intermodulation requirements in TS 38.104 could be reused for ATG BS.

# 4. References

[1] R4-2305894, WF on ATG BS RF requirements, ZTE