3GPP TSG-RAN WG4 Meeting # 107							              R4-2309442

Incheon, Korea, May 22 – May 26, 2023

Agenda item:	8.35.3

Source: 	CMCC

Title: 	Discussion on network energy saving RF requirements

Document for:	Discussion

# 1. Introduction

After discussion in the last meeting, a WF [1] for network energy saving have been approved. This paper focuse on some issues which are shown in the WF.

# 2. Discussion

## 2.1. Baseline

After the last meeting discussion, we discussed the feasibility conclusion and baseline for NES. As discussed in the last meeting, the performance of some inter-band CA combinations is similar to the intra-band contiguous CA. In our opinion, for the wider application of network energy saving in the future, the evaluation should not only focus on these cases. The study for inter-band CA which has two frequencies that are far apart, e.g., CA_n41-n79, should also be listed as a baseline. So we suggest the study for NES could be divided into two parts. One is for some inter-band CA band combinations that have close frequencies, and the other one is for inter-band CA which has two frequencies that are far apart.

Proposal 1: The study for NES could be divided into two parts:

- For inter-band CA band combinations that have close frequencies.

- For inter-band CA which has two frequencies that are far apart.

At the last meeting, some RF requirements for NES feasibility with SSB-less were discussed. Some companies imposed strict requirements on NES. In the operators’ opinion, we are very hopeful that NES will be realized in the near future, and we are also interested in whether the BS that has been deployed in the commercial network could support this NES feature, i.e. by updating with only software or we have to re-deploy the BSs due to hardware update. So we suggest companies provide how the gNB should be updated to achieve energy saving function.

Proposal 2: Companies are welcome to provide how the gNB should be updated, i.e. software update or hardware update, to achieve energy saving function.

## 2.2. TAE / RTD

To achieve time synchronization, the TAE between two carriers should be less than 1/2 CP.

Observation 1: The TAE between the two carriers should be less than 1/2CP.

The TAE requirement for intra-band CA is able to be within 260ns. The question of whether the inter-band CA could reuse the same TAE requirements as 260ns for intra-band contiguous CA had been discussed during the last meeting. In our opinion, the factor that determines the TAE requirement is the clock synchronization performance of BBU, and our testing results show that the TAE of the BS in the network could achieve 300ns – 700ns.

Observation 2: TAE requirements are based on the synchronization performance of BBU, and the TAE of the BS in the network could be optimized to 300ns – 700ns.

Table 1. The length of CP

| Numerology () | SCS (kHz) | ECP | NCP for Long Symbol | NCP for Other Symbol |
| --- | --- | --- | --- | --- |
| 0 | 15kHz | 16.7 s | 5.2 s | 4.69 s |
| 1 | 30kHz | 8.3 s | 2.86 s | 2.34 s |
| 2 | 60kHz | 4.17 s | 1.69 s | 1.17 s |

Table 1 shows the length of CP. It is easy to find that the range of 300ns-700ns could be less than CP/2 in most cases. This result shows that the BSs in the network have the ability to achieve TAE requirements under existing conditions. So we think there is no need to define new TAE requirements for BS.

Proposal 3: The BS in the network could ensure time synchronization, and there is no need to change the inter-band CA TAE requirements.

## 2.3. Propagation delay

In the last meeting whether the propagation delay could be ignored was discussed. In our opinion, the light speed for different frequencies is the same, and the propagation delay is depended on the implementations and configurations of RRU/AAU and BBU. We assume the distance between co-located RRU/AAU is 3m-5m, and if the channel propagation characteristics are similar, the propagation delay could be ignored. For example, assume the distance between two co-located RRU is 6m, there are two path for different RRU to a same UE, and the maximum distance difference between the two path is 6 meters. Consider of the light speed, the propagation delay is 20ns which could be ignored in most cases

Observation 3: In most cases, the propagation delay could be ignored.

## 2.4. RF requirements for Cell DTX

In the last meeting, we also analyzed the impact of DTX on RF requirements, e.g., transmitter OFF power and transmitter transient period. Last meeting, RAN2 has reached one of the agreements.

Agreement: The Cell DTX/DRX configuration contains at least: periodicity, start slot/offset, on duration.

Since the details of DTX/DRX are still pending, we suggest waiting for RAN1/2 to conclude further discussion.

Proposal 4: Waiting for more conclusions from RAN1/2 before discussing this question.

# 3. Conclusions

In this contribution, some issue for RAN4 are discussed with the following proposals.

Proposal 1: The study for NES could be divided into two parts:

- For inter-band CA band combinations that have close frequencies.

- For inter-band CA which has two frequencies that are far apart.

Proposal 2: Companies are welcome to provide how the gNB should be updated, i.e. software update or hardware update, to achieve energy saving function.

Observation 1: The TAE between the two carriers should be less than 1/2CP.

Observation 2: TAE requirements are based on the synchronization performance of BBU, and the TAE of the BS in the network could be optimized to 300ns – 700ns.

Proposal 3: The BS in the network could ensure time synchronization, and there is no need to change the inter-band CA TAE requirements.

Observation 3: In most cases, the propagation delay can be ignored.

Proposal 4: Waiting for more conclusions from RAN1/2 before discussing this question.

# 4. References

[1] R4-2306639, WF on RF feasibility study of Network Energy Savings for NR, Huawei