| 3GPP TR 38.741 V0.0.0 (2023-05) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Non-Terrestrial Networks (NTN) L-/S-band for NR; (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

5	Regulations

NOTE:	The purpose of this section is to collect information from existing regulatory documents concerning regulatory rules applicable to this NTN L-/S-band.

## 5.1	FCC

### 5.1.1	47 CFR § 25.202

NOTE: This section will contain the corresponding excerpts concerning frequencies, frequency tolerance, and emission limits.

According to paragraph (a)(4)(i), the following frequencies are available for use by the 1.6/2.4 GHz Mobile-Satellite Service:

-	1610-1626.5 MHz: User-to-Satellite Link

-	1613.8-1626.5 MHz: Satellite-to-User Link (secondary)

-	2483.5-2500 MHz: Satellite-to-User Link

According to paragraph (f) on emission limitations, except for SDARS terrestrial repeaters and as provided for in paragraph (i), the mean power of emissions shall be attenuated below the mean output power of the transmitter in accordance with the schedule set forth in paragraphs (f)(1) through (f)(4) of this section:

(1)	In any 4 kHz band, the center frequency of which is removed from the assigned frequency by more than 50 percent up to and including 100 percent of the authorized bandwidth: 25 dB;

(2) In any 4 kHz band, the center frequency of which is removed from the assigned frequency by more than 100 percent up to and including 250 percent of the authorized bandwidth: 35 dB;

(3) In any 4 kHz band, the center frequency of which is removed from the assigned frequency by more than 250 percent of the authorized bandwidth: An amount equal to 43 dB plus 10 times the logarithm (to the base 10) of the transmitter power in watts;

(4) In any event, when an emission outside of the authorized bandwidth causes harmful interference, the Commission may, at its discretion, require greater attenuation than specified in paragraphs (f) (1), (2) and (3) of this section.

### 5.1.2	47 CFR § 25.216

NOTE: This section will contain the corresponding excerpts concerning limits on emissions from mobile earth stations for protection of aeronautical radionavigation-satellite service.

(c) The e.i.r.p. density of emissions from mobile earth stations placed in service after July 21, 2002 with assigned uplink frequencies between 1610 MHz and 1660.5 MHz shall not exceed −70 dBW/MHz, averaged over any 2 millisecond active transmission interval, in the band 1559-1605 MHz. The e.i.r.p. of discrete emissions of less than 700 Hz bandwidth from such stations shall not exceed −80 dBW, averaged over any 2 millisecond active transmission interval, in the 1559-1605 MHz band.

## 5.2	ETSI

### 5.2.1	ETSI EN 301 441

NOTE: This section will contain the corresponding excerpts from ETSI EN 301 441.

Table 3: Maximum unwanted emissions outside the band 1610 MHz to 1626.5 MHz and the band 1626.5 MHz to 1628.5MHz.

| Frequency (MHz) | Carrier-on |  |  |
| --- | --- | --- | --- |
|  | EIRP (dBW) | Measurement bandwidth | Measurement method |
| 0.1 to 30 | -66 | 10 kHz | Peak-hold |
| 30 to 1 000 | -66 | 100 kHz | Peak-hold |
| 1000 to 1559 | -60 | 1 MHz | Average |
| 1559 to 1580.42 | -70 | 1 MHz | Average |
| 1580.42 to 1605 | -70 | 1 MHz | Average |
| 1605 to 1610 | -70 to -10 | 1 MHz | Average |
| 1610 to 1626.5 | N/A | N/A | N/A |
| 1626.5 to 1628.5 | N/A | N/A | N/A |
| 1628.5 to 1631.5 | -60 | 30 kHz | Average |
| 1631.5 to 1636.5 | -60 | 100 kHz | Average |
| 1636.5 to 1646.5 | -60 | 300 kHz | Average |
| 1646.5 to 1666.5 | -60 | 1 MHz | Average |
| 1666.5 to 2200 | -60 | 3 MHz | Average |
| 2200 to 12750 | -60 | 3 MHz | Peak-hold |

Table 4: Maximum unwanted emissions within the band 1610MHz to 1 626.5MHz and the band 1626.5 MHz to 1628.5MHz of MES operating such that the nominated bandwidth is entirely or partially contained in the frequency band 1618.25 MHz to 1626.5MHz.

| Frequency offset (kHz) | Carrier-on |  |  |
| --- | --- | --- | --- |
|  | EIRP (dBW) | Measurement bandwidth | Measurement method |
| 0 to 160 | -35 | 30 kHz | Average |
| 160 to 225 | -35 to -38.5 | 30 kHz | Average |
| 225 to 650 | -38.5 to -45 | 30 kHz | Average |
| 650 to 1365 | -45 | 30 kHz | Average |
| 1365 to 1800 | -53 to -56 | 30 kHz | Average |
| 1800 to 16500 | -56 | 30 kHz | Average |

Table 5: Maximum unwanted emissions within the band 1610MHz to 1626.5 MHz and the band 1626.5MHz to 1628.5MHz of MES operating such that the nominated bandwidth is entirely contained in the frequency band 1610MHz to 1618.25MHz.

| Frequency offset (kHz) | Carrier-on |  |  |
| --- | --- | --- | --- |
|  | EIRP (dBW) | Measurement bandwidth | Measurement method |
| 0 to 160 | -32 | 30 kHz | Average |
| 160 to 2300 | -32 to -56 | 30 kHz | Average |
| 2300 to 18500 | -56 | 30 kHz | Average |

Table 6: Maximum unwanted emissions of MES carriers within the operational band of CDMA carriers.

| Frequency offset (kHz) | Carrier-on |  |  |
| --- | --- | --- | --- |
|  | EIRP (dBW) | Measurement bandwidth | Measurement method |
| 0 to 70 | -6 to -20 | 30 kHz | Average |
| 70 to 600 | -20 to -28 | 30 kHz | Average |
| 600 to 2000 | -28 to -45 | 30 kHz | Average |
| 2000 to 5000 | -45 to -69 | 30 kHz | Average |
| 5000 to 16500 | -69 | 30 kHz | Average |

# 6	NR

## 6.1	Band plan and system parameters

Table 6.1-1: NTN L-/S-band n254

| NTN satellite operating band | Uplink (UL) operating band Satellite Access Node receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band Satellite Access Node transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n256 | 1980MHz – 2010 MHz | 2170 MHz – 2200 MHz | FDD |
| n255 | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz | FDD |
| n254 | 1610 – 1626.5 MHz | 2483.5 – 2500 MHz | FDD |
| NOTE: NTN satellite bands are numbered in descending order from n256. |  |  |  |

Table 6.1-2: NTN L-/S-band n254 channel bandwidths

| NTN satellite band | SCS kHz | UE Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 |
|  | 15 | 5 | 10 | 15 | 20 |
| n256 | 30 |  | 10 | 15 | 20 |
|  | 60 |  | 10 | 15 | 20 |
|  | 15 | 5 | 10 | 15 | 20 |
| n255 | 30 |  | 10 | 15 | 20 |
|  | 60 |  | 10 | 15 | 20 |
| n254 | 15 | 5 | 10 | [15] |  |
|  | 30 |  | 10 | [15] |  |
|  | 60 |  | 10 | [15] |  |

Table 6.1-3: NTN L-/S-band n254 applicable NR-ARFCN

| NTN satellite operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 100 | 396000 – <20> – 402000 | 434000 – <20> – 440000 |
| n255 | 100 | 325300 – <20> – 332100 | 305000 – <20> – 311800 |
| n254 | 100 | 322000 – <20> – 325300 | 496700 – <20> – 500000 |
| NOTE : The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. |  |  |  |

Table 6.1-4: NTN L-/S-band n254 applicable SS raster entries

| NTN satellite operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 15 kHz | Case A | 5429 – <1> – 5494 |
| n255 | 15 kHz | Case A | 3818 – <1> – 3892 |
|  | 30 kHz | Case B | 3824 – <1> – 3886 |
| n254 | 15kHz | Case A | 6215 – <1> – 6244 |
| NOTE : SS Block pattern is defined in clause 4.1 in 3GPP TS 38.213 [7]. |  |  |  |

## 6.2	UE requirements

### 6.2.1	UE transmitter characteristics

#### 6.2.1.1	Maximum output power

Table 6.2.1.1-1: NTN L-/S-band n254 Power Class

| NR satellite band | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| n256 | 23 | ±2 |
| n255 | 23 | ±2 |
| n254 | 23 | ±2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated |  |  |

#### 6.2.1.2	MPR/A-MPR

##### 6.2.1.2.1	Maximum power reduction

Maximum power reduction for the NTN L-/S-band is the same as for other NTN bands, such as L-band n255 and S-band n256.

Table 6.2.1.2.1-1: Maximum power reduction (MPR) for power class 3.

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.51 | ≤ 1.21 | ≤ 0.21 |
|  |  | ≤ 0.52 | ≤ 0.52 | 02 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | ≤ 0.52 | 02 | 02 |
|  | QPSK | ≤ 1 |  | 0 |
|  | 16 QAM | ≤ 2 |  | ≤ 1 |
|  | 64 QAM | ≤ 2.5 |  |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3 |  | ≤ 1.5 |
|  | 16 QAM | ≤ 3 |  | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |
| NOTE 1: Applicable for UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and if the IE powerBoostPi2BPSK is set to 1 and 40 % or less slots in radio frame are used for UL transmission for bands n40, n41, n77, n78 and n79. The reference power of 0 dB MPR is 26 dBm. NOTE 2: Applicable for UE operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79 with Pi/2 BPSK modulation and if the IE powerBoostPi2BPSK is set to 0 and if more than 40 % of slots in radio frame are used for UL transmission for bands n40, n41, n77, n78 and n79. |  |  |  |  |

#### 6.2.1.3	Emission requirements and NS values

#### 6.2.1.4	UE co-existence requirements

Table 6.2.1.4-1: Requirements for spurious emissions for UE co-existence

| NR NTN satellite Band | Spurious emission for UE co-existence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n254 | NR Band n1, n2, n3, n5, n7, n8, n12, n13, n14, n18, n20, n24, n25, n26, n28, n29, n30, n34, n38, n39, n40, n41, n48, n50, n51, n53, n54, n65, n66, n67, n70, n71, n74, n75, n76, n77, n78, n85, n90, n91, n92, n93, n94, n100, n101, n105, n255, n256 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 31, 54, 72, 73, 87, 88, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79, n104 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| n255 | NR Band n1, n2, n3, n5, n7, n8, n12, n13, n14, n18, n20, n24, n25, n26, n28, n29, n30, n34, n38, n39, n40, n41, n48, n50, n51, n53, n65, n66, n67, n70, n71, n74, n75, n76, n85, n90, n91, n92, n93, n94, n100, n101, n254 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| n256 | NR Band n1, n3, n5, n7, n8, n12, n13, n14, n18, n20, n24, n26, n28, n29, n30, n38, n39, n40, n41, n48, n50, n51, n53, n65, n66, n67, n71, n74, n75, n76, n78, n79, n85, n90, n91, n92, n93, n94, n101, n100, n101, n254 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 33, 35, 54 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | NR Band n2, n25, n70 | FDL_low | - | FDL_high | NA | NA | 3 |

### 6.2.2	UE receiver characteristics

#### 6.2.2.1	Reference sensitivity

#### 6.2.2.2	In-band blocking

#### 6.2.2.3	Out-of-band blocking

## 6.3	SAN requirements

There are no SAN RF specific requirements associated with this band.