3GPP TSG-RAN WG4 Meeting #107 	R4-2309567

Incheon, Korea meeting, May 22nd – 26th, 2023

Agenda Item: 8.11.1

Source: MediaTek inc.

Title: Discussion on L3 measurements and multiple options for BWP wor

Document for: Discussion and decision

# Introduction

RAN plenary discussed the topic BWP operation without restrictions in Meeting RAN#99 and defined a new WI with objectives captured below [1]:

## Objective ### Objective Core part WI ### The work item includes following objectives: - For Option A - Study and specify if any clarifications of the existing requirements are needed, e.g., applicability of requirements, conditions of gap configuration etc. (RAN4) - For Option B-1-1 - Specify support of BM/RLM/BFD based on SSB outside the active BWP without interruptions (RAN4, RAN2, RAN1) - For Option C - Specify support of BM/RLM/BFD based on NCD-SSB within active BWP for non-RedCap UEs (RAN4, RAN2, RAN1) - For Option B-1-2 - Specify support of BM/RLM/BFD based on SSB outside the active BWP with interruptions with the following conditions (RAN4, RAN2, RAN1): - The UE shall be allowed to use B-1-2 only if there is no CSI-RS, no NCD SSB and no CD SSB configured for RLM/BM/BFD in the active BWP of the corresponding carrier(s) to be measured; and - UE shall support option (C) NCD-SSB (subject to IoDT availability). - The interruption related requirements will be decided and specified in RAN4. - The expected RAN2 impacts are the RRC configuration signalling for the above options, and the capability signalling aspects.

In this contribution paper, we provide analysis and discussion on L3 measurements and the discussion for the support of multiple options.

# Discussion on RRM requirements impact/workload in RAN4

## Discussion on L3 measurements.

RAN4 has discussed the impact for L3 measurements for this WI and the following agreement and way forward are captured in the way forward:

<Agreement >: - It is a common RAN4 understanding that L3 measurements requirements impact for options B-1-1 and B-1-2 are not explicitly included in the WI objectives. - RAN4 work will focus on enabling L1 measurements for options B-1-1 and B-1-2 in Q2’2023. - In RAN4#107 RAN4 can continue discussion on whether any further clarifications are needed on intra-frequency measurements for UEs supporting options B-1-1 and B-1-2. ## Issue 4-4: Impact to intra-frequency measurement requirements for supporting option C During GTW on 17th April, following agreements were reached. <Agreement >: - It is a common RAN4 understanding that support of L3 measurements based on NCD-SSB for Option C can be beneficial, but it is not explicitly included in the WI objectives. Whether to support L3 measurements based on NCD-SSB is up to RAN decision. - RAN4 work will focus on enabling L1 measurements based on NCD-SSB in Q2’2023. ## Issue 1-2: Whether to send LS to RAN plenary and RAN1/2 on agreements on L3 related requirements for the options <Way forward >: - Proposals - Option 1: Yes - Option 2: No

It is RAN4 common understanding that UE L3 measurements behaviour is not explicitly included in the current WID and it is also common RAN4 understanding that L1 and L3 measurements are coupled. Hence, the work will be incomplete, if RAN4 consider L1 measurement only. Thus, RAN4 should request from RAN plenary to add the following to the current WID:

•	UE L3 measurement behaviour, if related, can be discussed.

•	The corresponding requirements can be specified, if necessary.

- RAN4 shall request from RAN plenary to add the following to the current WID: (i) UE L3 measurement behaviour, if related, can be discussed and (ii) The corresponding requirements can be specified, if necessary.

## Discussion on the supporting for multiple options.

RAN4 has discussed the supporting of multiple options and the following agreement and way forward are captured in the way forward:

## Issue 3-6: Whether to define requirements for supporting multiple options <Way forward >: - Proposals - Option 1: - Requirements/UE behaviour for UE supporting both option B-1-1 and C - Option 2: - Requirements for UE supporting both option B-1-1 and A.

The following are analysis for some scenarios where the UE could be able to perform measurements on multiple options, yet it is not clear what is the UE behaviour.

- Scenario 1: If the UE is operating in an active BWP with no NCD-SSB, and no CD-SSB but WITH CSI-RS, then the following are potential discussion in RAN4 RRM:

  - Would the UE perform the measurements based on B-1-1 or Option A (CSI-RS)?

  - Is it left for UE implementation to decide which option to utilise for the measurements?

  - Would the NW provide explicit configuration for multiple options for different resources?

- Scenario 2: If-else the UE is operating in an active BWP with no NCD-SSB, no CD-SSB and NO CSI-RS, then the following are potential discussion in RAN4 RRM:

  - Would the UE perform the measurements based on B-1-1 or B-1-2?

  - Is it left for UE implementation to decide which option to utilise for the measurements?

  - Would the NW provide explicit configuration for multiple options for different resources?

- RAN4 shall discuss the following scenario: If the UE is operating in an active BWP with no NCD-SSB, and no CD-SSB but WITH CSI-RS, then would the UE perform the measurements based on Option B-1-1 or Option A (CSI-RS).

- RAN4 shall discuss the following scenario: If-else the UE is operating in an active BWP with no NCD-SSB, no CD-SSB and NO CSI-RS, then would the UE perform the measurements based on Option B-1-1 or Option B-1-2.

# Summary

In this contribution, we have the following proposal:

Proposal 1: RAN4 shall request from RAN plenary to add the following to the current WID: (i) UE L3 measurement behaviour, if related, can be discussed and (ii) The corresponding requirements can be specified, if necessary.

Proposal 2: RAN4 shall discuss the following scenario: If the UE is operating in an active BWP with no NCD-SSB, and no CD-SSB but WITH CSI-RS, then would the UE perform the measurements based on Option B-1-1 or Option A (CSI-RS).

Proposal 3: RAN4 shall discuss the following scenario: If-else the UE is operating in an active BWP with no NCD-SSB, no CD-SSB and NO CSI-RS, then would the UE perform the measurements based on Option B-1-1 or Option B-1-2.

# References

- RP-230805, “New WID: Complete the specification support for BandWidth Part operation without restriction in NR”, Vodafone, vivo, Rotterdam, Netherlands, March 2023.

- R4-2306397, “WF on completion of specification support for BWP operation without restriction”, vivo, e-meeting, April 2023.