3GPP TSG-RAN WG4 Meeting #107	R4-2309602

Incheon, Korea, May 22 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on RRM Core Requirements for LPHAP Use Case 6 in NR Positioning

Agenda item:	8.23.3.3

Document for:	Discussion

Introduction

The Rel-18 WID [1] for NR positioning includes the enhancements for LPHAP.

- Specify enhancements for enabling LPHAP use-case 6 as defined in TS 22.104 including: - Extending eDRX cycle beyond 10.24s in RRC_INACTIVE state towards meeting the battery life requirement for LPHAP [RAN2, RAN3, RAN4] - Positioning-specific enhancement for eDRX cycle beyond 10.24s to be defined as part of Rel-18 WI on expanded and improved NR positioning. - NOTE: Work on this objective should be coordinated with that in Rel-18 WI on eRedCap. Towards this, the feature of extending eDRX cycle beyond 10.24s should be defined as part of Rel-18 WI on eRedCap. - NOTE: Inputs from RAN1 as necessary may be facilitated via LSs - For UL and DL+UL positioning for UEs in RRC_INACTIVE state, specify SRS configuration enhancements based on SRS positioning validity area to avoid frequent RRC connection for SRS (re)configuration [RAN2, RAN1, RAN3]. - SRS for positioning configurations in multiple cells [RAN2, RAN1]. - Note: Details including issues such as interference, timing advance, spatial relation information, pathloss reference and common SRS parameters across multiple cells can be further discussed during normative work. - Pre-configuration of one or multiple SRS for positioning configurations [RAN2, RAN3]. - SRS for positioning activation/request procedure(s) [RAN2, RAN1]. - Specify solutions for DL PRS measurements for a UE in RRC_IDLE state and reporting of the measurements in RRC_CONNECTED state [RAN2]. - Specify solutions for alignment between eDRX and PRS configurations [RAN2]. - Specify corresponding new core requirements, as well as identifying and specifying the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4].

In RAN4#106bis-e [2], the issues on eDRX cycle beyond 10.24s in INACTIVE were discussed.

Issue 1-1-1: PRS measurement requirements - Agreement (from GTW) - Define requirements for normal (non-RedCap) and RedCap type of devices - Define measurement requirements when eDRX is larger than 10.24s. FFS whether to define requirements for eDRX cycle less or equal to 10.24s - Wayforward - FFS whether to define requirements for eDRX <= 10.24s - Option 1: Define requirements for eDRX <= 10.24s - Option 2: Not to define requirements for eDRX <= 10.24s - Option 3: Define requirements for eDRX <= 10.24s for RedCap UE, and FFS for non-RedCap UE - Option 4: other Issue 1-1-2: Baseline for new PRS measurement requirements - Wayforward - FFS how to define requirements for eDRX <= 10.24s, if requirements are to be defined - Option 1: the requirements for RRC_INACTIVE state in R17 can be reused as baseline - Option 2: other - FFS how to define requirements for eDRX cycle larger than 10.24s. - Option 1: the requirements for RRC_INACTIVE state in R17 are reused as baseline - Option 2: the requirements for eDRX cycle > 10.24 sec in Rel-18 eRedCap WI are considered - Option 3: other Issue 1-1-3: Adaptations for new PRS measurement requirements - Agreement - The following R17 enhancements for latency reduction are supported for PRS measurement with eDRX in RRC_INACTIVE - Reduced number of PRS samples - parallelPRS-MeasRRC-Inactive-r17 capability - Lower Rx beam sweeping in FR2 - Wayforward - FFS whether ONLY reduced number of samples is considered with a higher SINR side condition - FFS whether and how to take the alignment between eDRX and PRS configuration into account based on RAN2 progress - FFS whether to define a scaling factor related to eDRX in the period requirement calculation - FFS whether and how to update Tavailable in the requirements when eDRX cycle is much larger than positioning interval - Other adaptations are not precluded Issue 1-1-4: RRM measurement requirements - Wayforward - FFS whether and how to define RRM measurement requirements for positioning specific needs with > 10.24s eDRX cycle in RRC_INACTIVE - RRM measurement requirements from eRedCap WI are to be considered as baseline - Positioning specific needs are to be considered - Note the eDRX cycle can be much larger than the positioning interval

Also, RAN4#106bis-e discussed SRS positioning validity area.

Issue 1-2-1: General impacts of SRS positioning validity area - Wayforward - Subject to progress in RAN1 and RAN2, RAN4 to investigate and specify the RRM impacts, if any, due to the introduction of SRS positioning validity area. - The applicability of UE Rx-Tx measurement requirements will be studied, and revisited/updated if necessary, based on RAN1 and RAN2 agreements on SRS positioning validity area, details FFS. - Other RRM impacts are not precluded. Issue 1-2-2: Detailed impacts of SRS positioning validity area - Wayforward - The issue is merged into issue 1-2-1.

In RAN4#106bis-e, PRS measurement in IDLE was discussed.

Issue 1-3-1: New PRS measurement requirements - Agreement (from GTW) - Define requirements for normal (non-RedCap) and RedCap type of devices - Requirements for PRS measurement in INACTIVE, both with and without eDRX, are re-used for PRS measurement in IDLE. Issue 1-3-2: Adaptations for new PRS measurement requirements - Agreement - RAN4 will specify NR positioning requirements in RRC_IDLE for DL-only positioning methods and measurements. - Note: UE Rx-Tx is not considered as DL-only measurement, and no requirement would be defined for UE Rx-Tx in RRC_IDLE. - For UEs capable of performing RRM measurement and PRS measurement in parallel to each other in RRC_IDLE mode Kcarrier_PRS = 1. - For UEs that are not capable of performing RRM measurement and PRS measurement in parallel to each other in RC_IDLE mode then Kcarrier_PRS is calculated as: - If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, Kcarrier_PRS=Kcarrier+1, where Kcarrier is defined in clause 4.2.2.4 of 38.133. - If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, Kcarrier_PRS=Nlayers+1, where Nlayers is defined in clause 4.2.2.7 of 38.133. - Wayforward - FFS whether the reporting delay for PRS measurements performed in RRC_IDLE needs to account for extra delay to transition to RRC_CONNECTED state. - FFS whether to reuse values from Rel. 17 for NRx,TEG,i and NRxBeam,i in positioning measurement delay requirements for RRC_IDLE state. - FFS whether reduced sample number are considered in requirements for PRS measurement in RRC_IDLE - FFS whether Rel. 17 accuracy requirements in RRC_INACTIVE state for Nsample = 4 also apply to positioning measurements in RRC_IDLE. - FFS impact of paging cycle on positioning measurement delay requirement - Other adaptations are not precluded

RRM core requirements are discussed in this contribution and few proposals on further proceedings are made.

Discussion

Impacts to RRM core requirements for some of the above enhancements are discussed in this section.

eDRX cycle beyond 10.24s in INACTIVE

PRS measurement requirements

In RAN4#106bis-e, it is agreed to define the PRS measurement requirements for non-RedCap and RedCap type of devices when eDRX is larger than 10.24 sec. While we can focus on the case where eDRX > 10.24 sec as described in the WID, it is also needed to define the requirements for eDRX ≤ 10.24 sec. Therefore, Option 1 is preferred to define the PRS measurement requirements.

We support Option 1 to define requirements for eDRX ≤ 10.24 sec.

Baseline for new PRS measurement requirements

If it is agreed to define requirements for eDRX ≤ 10.24 sec, the requirements for RRC_INACTIVE state in Rel-17 can be used as a baseline for new PRS measurement requirements.

The requirements for RRC_INACTIVE state in Rel-17 can be reused as a baseline for PRS measurement requirements for eDRX ≤ 10.24 sec.

Also, when for eDRX > 10.24 sec, the requirements for RRC_INACTIVE state in Rel-17 can be used as a starting point to define the new PRS measurement requirements. If the requirements of Rel-18 eRedCap WI is considered as a baseline, it should be discussed whether the typical scenario of interest such as a positioning interval of 15 – 30 sec can be satisfied for the UE with the extended eDRX cycle. Therefore, it is reasonable to reuse the PRS measurement requirements for RRC_INACTIVE in Rel-17 as an initial baseline.

We prefer Option 1 to reuse the requirements for RRC_INACTIVE state in Rel-17 as a baseline when eDRX > 10.24 sec.

Adaptations for new PRS measurement requirements

For the latency reduction in PRS measurements with eDRX in RRC_INACTIVE, it is agreed to reduce the number of PRS samples. In the case it is questionable about whether ‘only’ reduced number of samples is considered with a higher SINR side condition. We can generally support reducing the number of PRS samples for latency reduction in PRS measurements.

We support the first FFS on reduced sample number if ‘only’ is removed.

We agree the second FFS aspect on the alignment between eDRX and PRS configuration into account based on RAN2 progress.

RAN4 should reuse the existing framework of requirements rather than introducing a new scaling factor.

Next, it is needed to discuss how to update Tavailable for UE in RRC_INACTIVE state. For instance, to derive the measurement periodicity, the definition is given by Tavailable_PRS,i=LCMTPRS,i,TDRX. For a purpose of positioning, RAN4 should agree to update Tavailable in the requirements when eDRX cycle is much larger than positioning interval.

RAN4 should agree to update Tavailable in the requirements when eDRX cycle is much larger than positioning interval.

SRS positioning validity area

Impacts of SRS positioning validity area

The measurement and measurement reporting requirements can be investigated considering the change of SRS positioning validity area for LPHAP in RRC_INACTIVE. For area-based SRS configuration, when a validity area consists of multiple cells, the UE can continue sending SRS without the update of SRS configurations if it remains within the coverage of any of the cells in the validity area. Therefore, SRS configuration could not change when moving to a different cell within a validity area. Otherwise, if the UE leaves the validity area, a new SRS configuration may be required for UE. Therefore, any measurement period and measurement reporting requirements need to be specified.

As discussed in RAN4#106bis-e, the RRM impact of SRS positioning validity area depends on the progress in RAN1 and RAN2. RAN1#112bis-e discussed the spatial relation of an SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state when the spatial relation information is absent and provided as shown in Annex. RAN4 would study the RRM impact including the applicability of UE Rx-Tx measurement requirements once the design in RAN1 and RAN2 is stable.

RAN4 wait for the progress in RAN2 and RAN1.

PRS measurement in IDLE

Adaptations for new PRS measurement requirements

When PRS measurement is performed in RRC_IDLE state, the measurement reporting is done in RRC_CONNECTED state. Therefore, a delay between measurement and reporting can be expected, and the amount of reporting delay can be studied.

The reporting delay for PRS measurements in RRC_IDLE should consider extra delay for transition to RRC_CONNECTED state.

If the number of TEGs and Rx beams can be remained the same, the values from Rel-17 for NRx,TEG,i and NRxBeam,i in positioning measurement delay requirements can be reused for the requirements of RRC_IDLE state. In case the number of Rx beams in RRC_IDLE is less than RRC_CONNECTED, the values from Rel-17 need to be revisited. Therefore, it can be investigated whether the number of Rx beams for PRS measurement should be reduced in RRC_IDLE.

RAN4 should reuse values from Rel-17 for NRx,TEG,i and NRxBeam,i in INACTIVE as a starting point and study whether the values have to be adjusted if the number of Rx beams for PRS measurement is reduced in RRC_IDLE.

The PRS measurement in RRC_INACTIVE is performed by receiving messages from LMF via LPP. Here the number of samples Nsample is defined as follows.

Nsample= 1 if the UE supports supportedDL-PRS-ProcessingSamples-RRC-Inactive, and the LMF requests the UE to perform positioning measurements with reduced number of samples, and meets the following conditions:

-PRS bandwidth is within the initial BWP and

-Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

Nsample= 2 if the UE supports supportedDL-PRS-ProcessingSamples-RRC-Inactive, and the LMF requests the UE to perform positioning measurements with reduced number of samples, and does not meet the following conditions:

-PRS bandwidth is within the initial BWP and

-Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

Nsample= 4 otherwise.

Regarding the number of samples, Nsample= 4 can be used as a baseline. Also, if the conditions above or another condition can be defined and evaluated by UE in RRC_IDLE state, the reduced number of samples for UE in RRC_IDLE state can be considered. Therefore, the applicability of the reduced number of samples should be further studied. For instance, Nsample= 2 can be used for reduced latency. Moreover, the accuracy requirements related to the reduced number of samples should be discussed as a part of the performance requirement.

Reduced sample number of 2 can be considered in the requirements for PRS measurement in RRC_IDLE.

The accuracy requirements on the reduced sample number should be discussed as a part of performance requirement.

The impact of paging cycle on positioning measurement delay requirement can be studied for UE in RRC_IDLE state. In IDLE state, UE is configured with an eDRX cycle and PTWs. RAN4 should discuss the impact of paging cycle on positioning measurement delay requirement for UE in IDLE state.

The alignment between eDRX and PRS measurement needs to be accounted, and the impact depends on RAN2.

Conclusion

The scope of RRM impacts for Rel-18 Expanded and Improved NR positioning is investigated in this contribution.

The following proposals are made.

- We support Option 1 to define requirements for eDRX ≤ 10.24 sec.

The requirements for RRC_INACTIVE state in Rel-17 can be reused as a baseline for PRS measurement requirements for eDRX ≤ 10.24 sec.

We prefer Option 1 to reuse the requirements for RRC_INACTIVE state in Rel-17 as a baseline when eDRX > 10.24 sec.

We support the first FFS on reduced sample number if ‘only’ is removed.

We agree the second FFS aspect on the alignment between eDRX and PRS configuration into account based on RAN2 progress.

RAN4 should reuse the existing framework of requirements rather than introducing a new scaling factor.

RAN4 should agree to update Tavailable in the requirements when eDRX cycle is much larger than positioning interval.

RAN4 wait for the progress in RAN2 and RAN1.

The reporting delay for PRS measurements in RRC_IDLE should consider extra delay for transition to RRC_CONNECTED state.

RAN4 should reuse values from Rel-17 for NRx,TEG,i and NRxBeam,i in INACTIVE as a starting point and study whether the values have to be adjusted if the number of Rx beams for PRS measurement is reduced in RRC_IDLE.

Reduced sample number of 2 can be considered in the requirements for PRS measurement in RRC_IDLE.

The accuracy requirements on the reduced sample number should be discussed as a part of performance requirement.

The alignment between eDRX and PRS measurement needs to be accounted, and the impact depends on RAN2.

- References

- RP-230328, “Revised WID on Expanded and Improved NR Positioning,” Intel Corporation, RAN#99.

- R4-2306353, “WF on RRM requirements for LPHAP,” Huawei, HiSilicon, RAN4#106bis-e.

- Annex

The agreements related to LPHAP positioning in RAN1#112bis-e are captured below.

Agreement For SRS for positioning configuration in multiple cells for a UE in RRC_INACTIVE state, sequenceID in SRS for positioning configuration is commonly configured across cells within the validity area. Working assumption For the power control of an SRS for positioning configuration in multiple cells for a UE in RRC_INACTIVE state, support the following options: - Option 1: Pathloss RS is absent in the configuration. - Option 2: Pathloss RS is provided in the configuration. Working assumption For the spatial relation of an SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state, support that spatial relation information can be absent or present in the configuration. Agreement For the spatial relation of an SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state: - When the spatial relation information is absent in the configuration, the UE may use a fixed spatial domain transmission filter for transmissions of the SRS configured by the higher layer parameter SRS-PosResource across multiple SRS resources or it may use a different spatial domain transmission filter across multiple SRS resources; - When the spatial relation information is provided in the configuration, it is applicable across the cells within the SRS positioning validity area. Further study the configuration details. - FFS: spatial relation information validity criteria, and whether/how to determine UE fallback behavior if validity criteria for spatial relation of the configured RS is not met. Agreement For the power control of an SRS for positioning configuration in multiple cells for a UE in RRC_INACTIVE state, when pathloss RS is absent in the configuration, the UE determines the pathloss RS using a RS resource obtained from the SS/PBCH block of the camping cell that the UE uses to obtain MIB as the pathloss RS. Agreement For the power control of an SRS for positioning configuration in multiple cells for a UE in RRC_INACTIVE state, when pathloss RS is provided in the configuration, support one or multiple of the following alternatives on how to configure pathloss RS (down-selection to be made in RAN1#113 meeting): - Alt. 2-1: Reuse the configuration of pathloss RS in Rel-17; - Reuse the validity criteria of OPLC for SRS transmissions by RRC_INACTIVE UEs in Rel-17, i.e. when the pathloss RS cannot be accurately measured. When the validity criteria of OPLC fail, pathloss is calculated based on the RS resources obtained from SS/PBCH block of the new camping cell that the UE uses to obtain MIB. - Alt. 2-4: Configure a list of candidate pathloss RSs or candidate cells per SRS for positioning resource set. The UE transmits SRS resources in a SRS resource set using a pathloss RS determined from the candidate pathloss RSs based on its DL measurements on the candidate pathloss RSs; - FFS: whether/how to define validity criteria of OLPC and UE fallback behavior if validity criteria fail. - Alt. 2-5: Reuse the configuration of pathloss RS in Rel-17. The UE only transmits SRS resources in a SRS resource set associated with pathloss RS that can be accurately measured. - FFS: whether/how to define validity criteria of OLPC and UE fallback behavior if validity criteria fail. - Alt. 2-6: Reuse the configuration of pathloss RS in Rel-17; - A cell-agnostic DL RS can be the pathloss RS in the validity area and the cell-agnostic DL RS itself. - Note: UE power consumption should be considered