3GPP TSG-RAN WG4 Meeting #107	R4-2309674

Incheon, Korea, May 22 - 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on Case 1 requirements

Agenda item:	8.10.2.2

Document for:	Discussion

Introduction

This paper presents Nokia’s view on RRM aspects related to further enhancements of measurement gaps for Rel-18 [1]. In particular, it presents our view on open issues for Case 1 requirements as discussed at RAN4 #106bis-e [2], i.e. the combination of pre-configured MG and concurrent MG and lists corresponding proposals.

Discussion

Agreements in [2]

## Sub-topic 3-1: Gap combinations - Issue 3-1-1: [Case 1] Measurement gaps combinations principles < Agreement >: - The previous agreements from meeting #105 and #106 on the maximum number of configured MGs still apply: - Previous agreement 1: ‘Do not increase the max number of configured gaps for Case 1 (Pre-configured MG and multiple concurrent MGs) comparing to Rel-17 concurrent MG design’. - Previous agreement 2: ‘Gap combination configuration of case 1 can use gap combination of concurrent gaps defined in TS38.133 table 9.1.8-1 as formatting baseline with the clarification that each configured gap can be Pre-MG or Type-2 MG’. - New agreement: - For UE capable of [Concurrent Pre-MG], one FR can be configured with up to 2 Pre-MGs, regardless they are per-UE or per-FR configured and the configuration in the other FR. - UE capabilities for [Concurrent Pre-MG] are FFS. - FFS the wording of the notes to be added in the specification. - Issue 3-1-2: [Case 1] Detail measurement gaps combinations for UE supporting per-FR gap < Agreement >: - Discuss the notes for the detail measurement gaps combinations for UE supporting per-FR gap during the CR drafting phase.

## Sub-topic 3-2: Pre-MGs activation/deactivation procedure - Issue 3-2-2: [Case 1] Whether to extend the delay for fully overlapped simultaneous activation/deactivation for Pre-MG+Pre-MG < Agreement >: - The fully overlapped simultaneous multiple Pre-MGs activation/deactivation delay equals the BWPs/SCells/RRC reconfiguration delay plus existing processing time (5ms) plus the additional post-processing time T1, where T1 value is FFS. - An illustration example is captured below: < Way forward >: - FFS the value of T1 with the following options: - Option 1: 0 ms - Option 2: < = 5 ms - Other values are not precluded - Sub-topic 3-5: Others - Issue 3-5-1: [Case 1] Priority rules related issues < Agreement>: - RAN4 to stick to NW configured priority for Case 1.

Open issues in [2]

At last RAN4 #106bis-e, discussion on Case 1 requirements was continued. Following open issues are contained in [2].

Definitions for simultaneous and non-simultaneous Pre-MGs activation/deactivation for Pre-MG + Pre-MG

- Issue 3-2-1: [Case 1] Define definitions for simultaneous and non-simultaneous Pre-MGs activation/deactivation for Pre-MG + Pre-MG < Way forward >: FFS the following scenarios: The simultaneous case can be discussed in two scenarios, which are whether they are fully overlapped or partially overlapped, regardless of whether the Pre-MG (de)activation are triggered by the same or different events. Clarification on the fully and partially overlapping scenarios are provided in the figures below. Moderator suggests that companies provide feedback on whether the requirements should be defined for all scenarios captured in the figures.

- Fully overlap: - Both Pre-MGs are triggered by the same event, as shown below: - Two Pre-MGs are triggered by 2 events of the same type at the same time, as shown below: - Partially overlap: - Two Pre-MGs are triggered by 2 events of the same type at different time, as shown below: - Two Pre-MGs are triggered by different type of events at the same time, as shown below: - Two Pre-MGs are triggered by different type of events at different time, as shown below: - Two Pre-MGs are triggered by different type of events at different time but the delay finish at the same time, as shown below:

Regarding issue 3-2-1, we see some of the scenarios as relevant, for which requirements should be specified in TS 38.133. Table 1 below summarizes our view on relevant scenarios.

| Scenario | Specification support | Comments |
| --- | --- | --- |
| 1 a) | no | as only one Pre-MG per CC is configured |
| 1 b) | yes (but same DCI) | Simultaneous multiple BWP switching triggered by same DCI |
| 2 a) | yes | Non-simultaneous MAC-CE SCell activation |
| 2 b) | no | Simultaneous BWP switching / SCell activation |
| 2 c) | no | Non-simultaneous BWP switching / SCell activation |
| 2 d) | no | Non-simultaneous BWP switching / SCell activation |
| multiple SCell activation | yes | Simultaneous multiple SCell activation |
| Non-simultaneous BWP switching | yes | BWP switching happens with partial overlap |

Table 1: Scenarios for Pre-MG activation/deactivation

Thus, we propose to consider only following scenarios for Pre-MG activation/deactivation as depicted in the proposal below.

RAN4 to consider following scenarios for Pre-MG activation/deactivation:

| Scenario | Specification support | Comments |
| --- | --- | --- |
| 1 | yes | Simultaneous multiple MAC-CE SCell activation |
| 2 | yes | Simultaneous multiple BWP switching (triggered by same DCI) |
| 3 | yes | Non-simultaneous MAC-CE SCell activation |
| 4 | yes | Non-simultaneous BWP switching |

Additional delay for fully overlapped simultaneous activation/deactivation for Pre-MG+Pre-MG

Regarding issue 3-2-2 (see above), the additional delay T1 due to activation/deactivation of two fully overlapped simultaneous Pre-MGs should be rather low and hence less or equal to 1 ms.

The additional delay T1 due to activation/deactivation of two fully overlapped simultaneous Pre-MGs should be less or equal to 1 ms.

Additional delay for partially overlapped simultaneous activation/deactivation for Pre-MG+Pre-MG

- Issue 3-2-3: [Case 1] Whether to extend the delay for partially overlapped simultaneous activation/deactivation for Pre-MG+Pre-MG < Way forward >: FFS the options: - Option 1: - RAN4 shall extend the delay for partially overlapped simultaneous Pre-MGs activation/deactivation, when multiple Pre-MGs activation/deactivation processes are overlapped in time, until the end of the last Pre-MG activation/deactivation duration + T2. Where the value of T2 is FFS. - Option 3: - For partially overlapping simultaneous activation/deactivation of two Pre-MGs due to different events, RAN4 shall discuss necessity of defining requirement. The following two options can be considered: - only clarify in spec that extra delay can be expected in high-level. - the completion of activation/deactivation of the first Pre-MG is extended to the end of completion of activation/deactivation of the second Pre-MG. - Option 4: - Since the UE capability of supporting simultaneous pre-MG multiple activation/deactivation procedure has been identified, for the UE capable of such capability, the UE is capable to perform multiple activation/deactivation procedure in parallel, so we prefer to reuse the R17 activation/deactivation switching delay.

Regarding issue 3-2-3, we think option 1 is suitable. The additional delay T2 due to activation/deactivation of two partially overlapped simultaneous Pre-MGs should be rather low, can be same as T1 and hence less or equal to 1 ms.

RAN4 shall extend the delay for partially overlapped simultaneous Pre-MGs activation/deactivation, when multiple Pre-MGs activation/deactivation processes are overlapped in time, until the end of the last Pre-MG activation/deactivation duration + T2, where the additional delay T2 should be less or equal to 1 ms.

Collision handling – changes for Pre-MG

RAN4 discussed the collision handling for Case 1. Among others, following issue was left open.

- Issue 3-3-1: [Case 1] Required changes for Pre-MG on collision < Way forward >: - Option 1: RAN4 shall stick to agreed baseline that collision and priority rule on Pre-MG are considered only when Pre-MG is activated (deactivated Pre-MG is not considered in collisions). - Option 2: De-activated pre-MG is considered in collisions handling.

Regarding issue 3-3-1, collision of configured measurement gap occasions is usually being considered in contrast to collision of measurement objects associated with activated Pre-MG and those associated with concurrent MG in Case 1 scenario. However, in case of deactivated Pre-MG, the associated measurement objects are still active and are measured gapless. Gapless measurements can still be performed in the same time window corresponding to the MG occasion for activated Pre-MG. Thus, we support option 2.

Deactivated Pre-MG is considered in collisions handling.

In case of collision of deactivated Pre-MG with concurrent MG, the UE has two options

- select concurrent MG occasion, and at the same time drop gap-less measurements for deactivated Pre-MG in the known measurement location, or

- select gapless measurements for deactivated Pre-MG in the known measurement location and drop the concurrent MG occasion.

In option a), UE has to search for other suitable measurement locations to perform intra-/inter-frequency cell monitoring for measurement objects associated to Pre-MG, which will require additional effort or may be even challenging in case of short SMTC periods and long MGL of concurrent MG. On the other hand, measurement objects associated with concurrent MG are always being served.

In option b), UE has to drop MG occasion. If MGRP of concurrent gap and Pre-MG are not a multiple of each other, then the next concurrent MG occasion will be selected to serve the associated measurement objects.

The UE behavior, whether to adopt option a) or b) depends on the priority of measurement object associated with Pre-MG and Concurrent MG. For instance, if intra-frequency SSB measurements are associated with Pre-MG and positioning measurements with Concurrent MG, measurement objects associated with Pre-MG may have higher priority than those associated with Concurrent MG. Otherwise, if Pre-MG is associated with CSI-RS measurements or positioning measurements and Concurrent MG with intra-/inter-frequency measurements, then it is vice versa.

For deactivated Pre-MG in Case 1, network should thus also apply collision handling. Hence, the UE will select the   (de-)activated Pre-MG or Concurrent MG with the higher priority.

However, the deactivated Pre-MG collision with concurrent MG can be distinguished from the activated Pre-MG collision with concurrent MG, as in first case the gapless measurement can still be performed in other suitable measurement locations, which is not possible in the second case. Thus, priority handling for first and second case can be different, and this can be achieved by using another priority level for the deactivated Pre-MG.

Assuming the above case, i.e. intra-frequency SSB measurements are associated with Pre-MG, and positioning measurements with Concurrent MG, in case of collision, activated Pre-MG has higher priority than concurrent MG and deactivated Pre-MG could have either also higher or lower priority than concurrent MG, depending on SMTC configurations and MG patterns. Thus, the network can determine if the UE prioritizes measurements with deactivated Pre-MG or measurements with concurrent MG.

Collision handling in case of deactivated Pre-MG is done based on a second priority level associated with Pre-MG.

Collision handling – overlapping of pre-configured MG activation procedure with one of concurrent gap occasion

RAN4 discussed the scenario of a concurrent gap occasion colliding with Pre-MG activation at RAN4 #106 and RAN4 #106bis-e. We supported the proposal to extend the Pre-MG activation delay to 5ms after end of MGL of concurrent gap, namely option 1a in issue 3-3-2 below.

- Issue 3-3-2: [Case 1] When the pre-configured MG activation procedure is overlapped with one of concurrent gap occasion during the dynamic collision (i.e. Pre-MG has higher priority than the MG) < Way forward >: - FFS the following options: - Option 1: - A collision between a change in the status of a pre-configured MG and a gap instance happens when the change occurs ≤ 4 ms before the start or ≤ 4 ms after the end of a gap instance of an activated concurrent MG. (UE shall extend the activation procedure) - Option 1a: - If a change in the status of a pre-configured MG collides with a gap instance, the change in status is delayed by (MGL of the gap instance plus 5 ms) to avoid the collision. - Option 2: - For some gap patter configuration, for example when the current gap pattern is gap pattern 4 (MGL = 6ms and MGRP = 20ms), after the activation of Pre-MG, that Pre-MG occasion will collide with the next gap occasion of the concurrent gap and gaps will be dropped based on priority rule. Clarifications may need since for a particular window W there is an extra collision due to the extension of the activation procedure and the collision may not happen again for other time window W. - Option 3: - Alternatively, when the pre-configured MG activation procedure is overlapped with one of concurrent gap occasion, the Pre-MG status shall not be changed immediately and its status shall be changed prior to the next gap occasion. - Option 4: - When Pre-MG activation/deactivation period collides Type-2 gap, - during Pre-MG activation/deactivation period, - the UE is not required to receive or transmit in the corresponding NR serving cells in the Pre-MG occasions. - gap dropping rule won’t be applied. - how to use such gap occasion is up to UE implementation. - the gap dropping rule will be re-applied after the 1st effective MG occasion after Pre-MG activation. - Option 5: - If the priority of pre-MG is higher than the other MG, applying priority rule, i.e. pre-MG activation/deactivation procedure prioritizes the other MG occasion, so the other MG occasion is dropped; - then the MO(s) associated with this MG accordingly be canceled at this MG occasion.

- Option 6: - If two MG occasions collide, one of the two MGs is pre-MG and the pre-MG has higher priority, then UE is allowed to keep or drop any of the two occasions if - the (de)activation procedure of pre-MG ends earlier than the start of pre-MG occasion, and - the (de)activation procedure of pre-MG overlaps with time period T, where T starts from 4ms before the other MG occasion and ends at 4ms after the other MG occasion - Option 7: - Agree on option 2 for issue 3-3-1 and close this issue.

We continue to support option 1a in this scenario, irrespective whether the activated Pre-MG has higher or lower priority then the concurrent gap. For subsequent gap occasions, if they collide, the priority rule applies when comparing the priority of the activated Pre-MG against the priority of the concurrent MG.

If a change in the status of a pre-configured MG to activated Pre-MG collides with a concurrent gap instance, the change in status of the Pre-MG is delayed by (MGL of the concurrent gap instance plus 5 ms) to avoid the collision. For subsequent gap occasions, if they collide, the priority rule applies when comparing the priority of the activated Pre-MG against the priority of the concurrent MG.

Collision handling – overlapping of pre-configured MG deactivation procedure with one of concurrent gap occasion

- RAN4 discussed the scenario of a concurrent gap occasion colliding with Pre-MG deactivation at RAN4 #106bis-e in issue 3-3-3.

- Issue 3-3-3: [Case 1] When the pre-configured MG deactivation procedure is overlapped with one of concurrent gap occasion during the dynamic collision (i.e. Pre-MG has higher priority than the MG) < Agreement/ Way forward >: - FFS. Wait for outcome of issue 3-3-1.

In our view, the UE behavior for Pre-MG deactivation can be same as for Pre-MG activation and irrespective whether the deactivated Pre-MG has higher or lower priority than concurrent MG. For subsequent gap occasions, if they collide, the priority rule applies when comparing the priority of the deactivated Pre-MG against the priority of the concurrent MG.

If a change in the status of a pre-configured MG to deactivated Pre-MG collides with a concurrent gap instance, the change in status of the Pre-MG is delayed by (MGL of the concurrent gap instance plus 5 ms) to avoid the collision. For subsequent occasions of the deactivated Pre-MG and the concurrent gap, if they collide, the priority rule applies when comparing the priority of the deactivated Pre-MG against the priority of the concurrent MG.

Above proposals 6 and 7 cover issues 3-3-4 and 3-3-5.

UE capability for dynamic collisions

Regarding issue 3-3-6, RAN4 discussed whether to define a new UE capability for dynamic collisions.

- Issue 3-3-6: [Case 1] Whether to define a new UE capability for dynamic collisions? < Agreement/ Way forward >: - FFS. Wait for outcome of issue 3-3-1.

As mentioned in section 2.2.4, the handling of dynamic collisions is supported by adopting proposal 4 and proposal 5. Thus, the network configures two priority levels for Pre-MG, one for activated Pre-MG and one for deactivated Pre-MG, which are valid, once the activation or deactivation, respectively, has been completed. In our view, this does not require introduction of a new UE capability for dynamic collisions, as the gap priority rule for collision of activated Pre-MG and concurrent MG and for collision of deactivated Pre-MG and concurrent MG is applicable to all UEs supporting Case 1 requirements.

No new UE capability for dynamic collisions is needed for UEs supporting Case 1 requirements.

Regarding issue 3-3-7, whether to define dynamic collisions dropping rule, RAN4 could not achieve consensus.

- Issue 3-3-7: [Case 1] dynamic collisions dropping rule < Agreement/ Way forward >: - FFS. Wait for outcome of issue 3-3-1.

We propose to extend the Rel-17 gap dropping rule, comparing priorities between concurrent gaps, for Case 1 requirements to include configured priorities for activated and deactivated Pre-MG. Thus, the priority rule also applies to collisions of activated Pre-MG with concurrent MG and of deactivated Pre-MG with concurrent MG. No further change is needed.

The Rel-17 gap dropping rule, comparing priorities between concurrent gaps, is extended for Case 1 requirements to include configured priorities for activated and deactivated Pre-MG. Thus, the priority rule also applies to collisions of activated Pre-MG with concurrent MG and of deactivated Pre-MG with concurrent MG.

Measurement delay requirements

Regarding issue 3-4-1 on measurement delay requirements due to change in Pre-MG status, RAN4 discussed following options.

- Issue 3-4-1: [Case 1] Measurement delay requirements due to change in Pre-MG status < Way forward >: - Postpone the discussion until it is clear whether to support dynamic collision or not. - FFS the options: - Option 1: - When gap combinations including pre-configured MGs (Case 1) are provided to the UE, measurement requirements do not apply if the following parameters change during the measurement period due to changes in the status of any pre-configured MGs: - Kp for intra-frequency and inter-frequency measurements without gaps - Kgap for intra-frequency and inter-frequency measurements with gaps - Kgap_EUTRA for inter-RAT measurements - Kp_CSI-RS for CSI-RS L3 measurements - Kp,PRS,i for NR positioning measurements - CSSFintra for intra-frequency measurements - CSSFinter for intra-frequency measurements - CSSFinterRAT for intra-RAT measurements - P scaling factor for L1-RSRP and L1-SINR measurements - Option 2: - R17 measurement delay requirements for concurrent MG can be reused for case 1 without any status change.

- Option 3: - RAN4 to investigate whether measurement requirements in case of changes of Pre-MG status are based on more relaxed requirements between both Pre-MG status.

In our view, option 1 and option 2 treat different scenarios. Option 1 treats the transition scenario due to status change of Pre-MG and option 2 the overlapping between Pre-MG and concurrent MG. For the first scenario, RAN4 may either not specify requirements for the transition scenario or may specify requirements based on applying more relaxed requirements between both Pre-MG status. For the second scenario, option 2 does not apply, as RAN4 will add the Pre-MG activation delay as discussed under issues 3-3-2 and 3-3-3.

RAN4 to investigate whether measurement delay requirements for the transition scenario (i.e. due to change in Pre-MG status) are based on the most relaxed requirements between both Pre-MG status.

Regarding issue 3-4-1 on measurement delay requirements due to Pre-MG activation, RAN4 discussed following options.

- Issue 3-4-2: [Case 1] Measurement delay requirements < Way forward >: - Postpone the discussion until it is clear whether to support dynamic collision or not. - FFS the options: - Option 1: - No need to consider the delayed Pre-MG activation in measurement requirements - Option 2: - Measurement delay requirements should also consider the case of delayed Pre-MG activation due to overlap between Pre-MG and concurrent MG. - Option 2a: - RAN4 to add the Pre-MG activation delay of 5 ms to the measurement delay in case of overlapping of Pre-MG with concurrent MG.

We support option 2a in accordance with proposal 6 related to issue 3-3-2. In addition to Pre-MG activation, we support to include Pre-MG deactivation in the measurement delay requirements in according with proposal 7 related to issue 3-3-3.

Measurement delay requirements should also consider the case of delayed Pre-MG activation / deactivation due to overlap between Pre-MG and concurrent MG.

Conclusion

This paper has presented Nokia’s views on Case 1 requirements as discussed at RAN4 #106bis-e [2], i.e. the combination of pre-configured MG and concurrent MG.

Resulting from this discussion, we make following proposals:

- RAN4 to consider following scenarios for Pre-MG activation/deactivation:

| Scenario | Specification support | Comments |
| --- | --- | --- |
| 1 | yes | Simultaneous multiple MAC-CE SCell activation |
| 2 | yes | Simultaneous multiple BWP switching (triggered by same DCI) |
| 3 | yes | Non-simultaneous MAC-CE SCell activation |
| 4 | yes | Non-simultaneous BWP switching |

The additional delay T1 due to activation/deactivation of two fully overlapped simultaneous Pre-MGs should be less or equal to 1 ms.

RAN4 shall extend the delay for partially overlapped simultaneous Pre-MGs activation/deactivation, when multiple Pre-MGs activation/deactivation processes are overlapped in time, until the end of the last Pre-MG activation/deactivation duration + T2, where the additional delay T2 should be less or equal to 1 ms.

Deactivated Pre-MG is considered in collisions handling.

Collision handling in case of deactivated Pre-MG is done based on a second priority level associated with Pre-MG.

If a change in the status of a pre-configured MG to activated Pre-MG collides with a concurrent gap instance, the change in status of the Pre-MG is delayed by (MGL of the concurrent gap instance plus 5 ms) to avoid the collision. For subsequent gap occasions, if they collide, the priority rule applies when comparing the priority of the activated Pre-MG against the priority of the concurrent MG.

If a change in the status of a pre-configured MG to deactivated Pre-MG collides with a concurrent gap instance, the change in status of the Pre-MG is delayed by (MGL of the concurrent gap instance plus 5 ms) to avoid the collision. For subsequent occasions of the deactivated Pre-MG and the concurrent gap, if they collide, the priority rule applies when comparing the priority of the deactivated Pre-MG against the priority of the concurrent MG.

No new UE capability for dynamic collisions is needed for UEs supporting Case 1 requirements.

The Rel-17 gap dropping rule, comparing priorities between concurrent gaps, is extended for Case 1 requirements to include configured priorities for activated and deactivated Pre-MG. Thus, the priority rule also applies to collisions of activated Pre-MG with concurrent MG and of deactivated Pre-MG with concurrent MG.

RAN4 to investigate whether measurement delay requirements for the transition scenario (i.e. due to change in Pre-MG status) are based on the most relaxed requirements between both Pre-MG status.

Measurement delay requirements should also consider the case of delayed Pre-MG activation / deactivation due to overlap between Pre-MG and concurrent MG.

- References

- RP-230755, Revised WID: Further enhancements on NR and MR-DC measurement gaps and measurements without gaps, Intel, MediaTek

- R4-2306330, WF on Rel-18 NR MG enhancements, MediaTek