3GPP TSG-RAN WG4 Meeting #107	R4-2309704

Incheon, KR, May 22 – May 26, 2023

Agenda item:	8.13.4.4

Source: 	Qualcomm Incorporated

Title: 		RRM aspects for tunnel deployment scenario for FR2 HST

Document for:	Discussion

# Introduction

Deployment assumptions for Scenario#1

Agreement:

- Consider both uni-directional and bi-directional deployments in the tunnel for Sceanrio#1 (single-panel reception UE and DPS transition scheme).

Way forward:

- FFS additional assumptions on tunnel deployment:

  - Option 1: In uni-directional scenarios assume the at least one open-space RRH is deployed close to railway, e.g., with tunnel deployment parameters, and orientations of RRH panels are the same in open-space and in the tunnel.

  - Option 2: No need to taking exit/entrance of the tunnel into account of tunnel scenario and channel.

Solution to the mobility issue in the tunnel

Way forward:

- FFS possible solution to the mobility issue inside the tunnel when CPE is travelling in the direction opposite to the serving beam:

  - Option 2: Solutions that allow network to trigger early handover/beam switch

  - Option 2a: Enabling CHO with special settings next to the RRH

  - Option 3: UE-initiated beam selection/activation based on beam measurement

  - Option 5: No need to introduce new mechanism for mobility issue when the train is travelling opposite to the serving beam orientation

  - Option 6: Consider bi-directional deployment with simultaneous multi-panel reception in the tunnel scenario to alleviate the mobility issues.

RAN4 has been discussing the reference tunnel deployment scenario for FR2 HST for past few meetings and the WF [2] from RAN4#106bis lists the latest agreements and open issues.

In this contribution, we provide our views on some of the open issues under this topic.

# Deployment assumption

During the last meeting it was proposed to assume that in uni-directional scenarios, at least one open-space RRH is deployed close to tunnel, e.g., with tunnel deployment parameters, and orientations of RRH panels are the same in open-space and in the tunnel. We think it’s up-to network deployment and there’s no need to make such assumption and discuss any related requirements. This was discussed in demod session as well and it was agreed to consider CPEs fully inside the tunnel to define RRM requirements within the tunnel. We think similar assumptions can be made in RRM session as well no need to consider deployment of an RRH at entry/exit of the tunnel in defining RRM requirements within the tunnel.

Observation 1: Demod session agree to only consider CPEs fully inside the tunnel to define demod requirements for the tunnel scenario.

Proposal 1: Following demod agreement, no need to taking exit/entrance of the tunnel into account for specifying the channel and RRM requirements within tunnel scenario.

# Mobility issue

RAN4 discussed the mobility issue when the train is travelling opposite to the serving beam orientation in a unidirectional tunnel deployment. As the UE approaches the RRH, it is observed that the RSRP values may drop significantly and quickly. This may lead to handover/beam switch failures. RAN4 acknowledged this issue and identified potential solutions listed below [1]:

Agreement [From RAN4#106][1]:

- Mobility issue at HO/beam switch when CPE is travelling in the direction opposite to the serving beam is observed due to the sharp drop of the signal strength at the edge of the beam next to the RRH.

Way forward [From RAN4#106bis][2]:

- FFS possible solution to the mobility issue inside the tunnel when CPE is travelling in the direction opposite to the serving beam:

  - Option 2: Solutions that allow network to trigger early handover/beam switch

  - Option 2a: Enabling CHO with special settings next to the RRH

  - Option 3: UE-initiated beam selection/activation based on beam measurement

  - Option 5: No need to introduce new mechanism for mobility issue when the train is travelling opposite to the serving beam orientation

  - Option 6: Consider bi-directional deployment with simultaneous multi-panel reception in the tunnel scenario to alleviate the mobility issues.

We think option 2 and/or option 3 can provide good solutions to this problem. It makes sense to study solutions that allow network to trigger early Handover based on parameters such as train speed, inter-RRH distance and time from the last handover/TCI state switch. For example, considering a train speed of 350km/h and inter-RRH distance of 700m, it’ll take approximately 7.2 seconds for the train to travel from one RRH to the other. So, network can trigger a handover/TCI state switch about 7.2 seconds after the previous handover/TCI state switch assuming that the beam coverage offset is the same for all RRHs.

Proposal 2: RAN4 to discuss solutions that allow network to trigger early handover/beam switch, if needed, when the train mounted UE/CPE travels in the direction opposite to the RRM beam.

# Conclusion

Observation 1: Demod session agree to only consider CPEs fully inside the tunnel to define demod requirements for the tunnel scenario.

Proposal 1: Following demod agreement, no need to taking exit/entrance of the tunnel into account for specifying the channel and RRM requirements within tunnel scenario.

Proposal 2: RAN4 to discuss solutions that allow network to trigger early handover/beam switch, if needed, when the train mounted UE/CPE travels in the direction opposite to the RRM beam.

# References

[1] R4-2303174, “WF on NR FR2 HST Tunnel deployment and UL timing adjustment”, Nokia, RAN4#106

[2] R4-2306398, “WF on NR FR2 HST UL Timing Adjustment Solutions and Tunnel Deployment”, Nokia, RAN4#106bis-e